3GPP TSG-RAN WG4 Meeting #113	R4-2418439

# Orlando, US, 18th – 22nd November, 2024

Source: 	KDDI, NTT Docomo

Title: 	(NR_newRAT-Core) NR A-MPR reevaluation scenarios on n1 for Japan

Agenda Item:	4.2.1

Document for:	Discussion

- 1. Introduction

In Japan, a discussion is ongoing about MOP relaxation. In Rel-15, based on operators’ requirements [1], NS_05 for n1 has been supported to protect domestic PHS (Personal Handy-phone System), and it has been shut down. This means that the current NS_05/NS_05U for n1 can be relaxed, but also there is still a necessary for protecting DECT(Digital Enhanced Cordless Telecommunications, the other domestic digital cordless-phone service) instead of PHS. The new domestic protection requirement has been applied on Sep. 30th, 2024.

Relevant to the possible relaxation, the contribution[2] was submitted in last meeting and this paper is intended to propose necessary evaluation to specify new NS_05/NS_05U for n1 PC3 and PC2.

Current additional spurious emission requirements on NS_05/NS_05U are as follows:

38.101-1) Table 6.5.3.3.4-1: Additional requirements for "NS_05" and “NS_05U”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 1884.5 f 1915.7 | -41 | 300 kHz |  |

36.101) Table 6.6.3.3.1-1: Additional requirements (PHS)

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz | 20MHz |  |  |
| 1884.5 f 1915.7 | -41 | -41 | -41 | -41 | 300 KHz | 1 |

On the other hand, the PHS protection requirement will be modified to protect DECT as follows:

Observation 1: The PHS protection requirement will be modified to protect DECT as follows:

For n1/B1)

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

For other bands)

- 1884.5-1915.7 MHz: -30 dBm/MHz

- 2. Discussion

Reevaluation of n1 NS_05 and NS_05U

Up to 20MHz CBW of n1 has been supported since Rel-15 with current NS_05 and NS_05U (all the sections/Tables are in TS 38.101-1 sec 6.2.3.4) for protecting PHS.

So far, the max output power is limited according to TS 38.101-1 Table 6.2.3.4-1, 6.2.3.4-2, 6.2.3.4-3 and 6.2.3.4-10. On the other hand, the MOP relaxation discussion  will be allowed soon in Japan. Then we’d like to prepare for the updated as:

While domestic regulation does not mention anything about indication of NS itself, UTRA ACLR is specified for UE transmitting at 2GHz. Therefore, from standard perspective, we need NS_05U to ensure UE can meet UTRA ACLR adding to NS_05.

Proposal 1: Invite reevaluation/simulation for modified NS_05 and NS_05U for n1 PC3 and PC2 for RAN4#113 Nov. A-MPR is reevaluated for the following limits and for channel positions in figure 1 and 2.
For n1/B1)

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

Band n1 – in 1920-1940 MHz case

Can smaller A-MPR values be expected than those of current NS_05/NS_05U for Case#1-#8?

Figure.1: 1920-1940 MHz case

Band n1 – in 1940-1960 MHz case

In current spec, A-MPR is defined for only 20 MHz CBW (Fc = 1950 MHz), no A-MPR for less than 20MHz CBW. With regulatory update, no A-MPR is needed even for 20MHz CBW for Digital cordless-phone service protection as it can be met by SEM and general spurious emission.

Note: NS_05U requires the same A-MPR with NS_100 to meet UTRA ACLR

Figure.2: 1940-1960 MHz case

How to introduce new relaxed A-MPR

Using modifiedMPR-behavior and keeping the numbering of NS, so that we can avoid introducing new NS and any impacts on legacy UEs.

Proposal 2: To introduce new relaxed A-MPR, use modifiedMPR-behavior and reuse NS_05/NS_05U instead of introducing new NS.

The modifiedMPR-behavior is flexible. If PC2 is not ready in Feb. RAN4 can do it later and assign modifiedMPR-behavior bit only for PC3 modification in Feb.

Proposal 3: If RAN4 can’t conclude n1 PC2 NS_05 and NS_05U related CRs in Feb., RAN4 conclude only n1 PC3 NS_05 and NS_05U related CRs in Feb.

As usual, we prefer to support modified NS_05/NS_05U from the earliest possible release, but also it is up to marketing or implementation plans of UE/chipset vendors. We’d like to collect the views on preferred release. To leave some flexibility for the release, if situation permits, we’d like to conduct this activity under up to Rel-17  maintenance AI. (it is up to RAN4 Chair’s decision.)

Proposal 4: Discuss whether mandating new n1 NS_05 and NS_05U can be applied from Rel-18 or Rel-19 under up to Rel-17 maintenance AI.

Relaxation of additional spurious emission and spurious emission for UE coexistence

Meanwhile, we need to consider relaxation of the n1 common emission requirements specified in 38.101-1 Table 6.5.3.3.4-1: Additional requirements for "NS_05" and “NS_05U”.

38.101-1) Table 6.5.3.3.4-1: Additional requirements for "NS_05" and “NS_05U”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 1884.5 f 1915.7 | -41 | 300 kHz |  |

Proposal 5: Update n1 additional spurious emission in 38.101-1 Table 6.5.3.3.4-1.

Additionally, related spurious emission for UE coexistence and for NR CA in 38.101-1 (102 hits searching for “1915.7” in clause 6) also needed to be updated. Then, related spurious emission for UE coexistence and for EN-DC in 38.101-3 (87 hits searching for “1915.7” in clause 6) also needed to be updated.

Proposal 6: Update spurious emission for UE coexistence and for NR CA in 38.101-1 and for EN-DC 38.101-3.

Adding to spurious emission for UE coexistence and for NR CA in 38.101-1 and for EN-DC 38.101-3, we also need to consider relaxation of the LTE B1 common emission requirements specified in 36.101 Table 6.6.3.3.1: Minimum requirement (network signalled value “NS_05”).

36.101) Table 6.6.3.3.1-1: Additional requirements (PHS)

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz | 20MHz |  |  |
| 1884.5 f 1915.7 | -41 | -41 | -41 | -41 | 300 KHz | 1 |

Proposal 7: Update B1 common emission requirements in 36.101-1 Table 6.6.3.3.1.

On the other hand, Not need to reevaluate any B1 A-MPR for NS_05/NS_05U different from n1 reevaluation. In our understanding, the protection requirement will be relaxed. Therefore, updating only 36.101 Table 6.6.3.3.1 cause no harm.

UE Capability description on modifiedMPR-Behaviour

For the IE modifiedMPR-Behaviour, it should be CY rather than optional. From reading purely RAN4 spec, we do not understand how it could be optional. If companies can confirm this understanding, RAN2 can fix it from Rel-16 in which this IE was introduced.

Observation 2: If companies can confirm this understanding, RAN2 can fix it from Rel-16 in which this IE was introduced. Something like the following:

| modifiedMPR-Behaviour Indicates whether UE supports modified MPR behaviour defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 38.101-5 [34]. It is mandatory for certain bands as defined in TS XXXXX. Otherwise, it is optional. | Band | NoCY | N/A | N/A |
| --- | --- | --- | --- | --- |

- 3. Conclusion

Observation 1: The PHS protection requirement will be modified to protect DECT as follows:

For n1/B1)

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

For other bands)

- 1884.5-1915.7 MHz: -30 dBm/MHz

Proposal 1: Invite reevaluation/simulation for modified NS_05 and NS_05U for n1 PC3 and PC2 for RAN4#113 Nov. A-MPR is reevaluated for the following limits and for channel positions in figure 1 and 2.
For n1/B1)

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

Proposal 2: To introduce new relaxed A-MPR, use modifiedMPR-behavior and reuse NS_05/NS_05U instead of introducing new NS.

Proposal 3: If RAN4 can’t conclude n1 PC2 NS_05 and NS_05U related CRs in Feb., RAN4 conclude only n1 PC3 NS_05 and NS_05U related CRs in Feb.

Proposal 4: Discuss whether mandating new n1 NS_05 and NS_05U can be applied from Rel-18 or Rel-19 under up to Rel-17 maintenance AI.

Proposal 5: Update n1 common emission requirements in 38.101-1 Table 6.5.3.3.4-1.

Proposal 6: Update spurious emission for UE coexistence and for NR CA in 38.101-1 and for EN-DC 38.101-3.

Proposal 7: Update B1 common emission requirements in 36.101-1 Table 6.6.3.3.1.

Observation 2: If companies can confirm this understanding, RAN2 can fix it from Rel-16 in which this IE was introduced.

- 4. Reference

[1] R4-1803744 NR A-MPR evaluation scenarios for n1 and n8 for Japan, RAN4#86bis

[2] R4-2415439 NR A-MPR evaluation scenarios for n1 for Japan, RAN4#112bis