# 3GPP TSG-RAN WG4 Meeting #113                              R4-2418441

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.19.3.2

Source: 	CMCC

Title: 			 Discussion on other RRM requirements for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1]. In RAN#105 meeting, the revised WID are approved [2], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was initial discussion on NR MIMO Phase 5, and a WF was agreed [3]. This contribution provides discussion on RRM requirements on UE reporting enhancement for CJT calibration and enhancements for asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

## 2.1 UE reporting enhancement for CJT calibration

Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define accuracy requirements. - Option 2: FFS. Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

RAN1 agreements on UE reporting enhancement for CJT deployments which may have RAN4 impacts are duplicated as following.

Agreements in RAN1#116 For the Rel-19 aperiodic standalone CJT calibration reporting, the following use cases are assumed: - For per-TRP delay offset reporting: - Use case 1.1: TRP selection - Use case 1.2: delay offset compensation for at least one TRP to ensure the CJT-composite delay spread doesn’t exceed a pre-defined dynamic range/threshold - For per-TRP frequency offset (FO) reporting: - Use case 2.1: TRP selection - Use case 2.2: per-TRP FO compensation at NW side For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {(Dn,offset, dn), n=0, 1, …, N – 1} where - Dn,offset is a B-bit indicator representing the delay offset associated with the n-th CSI-RS resource/resource set - For the reference CSI-RS resource/resource set nref, the value of Dnref,offset is assumed 0 and not reported - dn is a 1-bit indicator associated with the n-th CSI-RS resource/resource set, indicating whether the measured delay offset, plus delay spread, is inside or outside a pre-defined range/interval For the Rel-19 aperiodic standalone CJT calibration reporting, in addition to the already agreed use cases, the following use cases are assumed: - For per-TRP DL/UL Rx-Tx phase misalignment reporting: - Use case 3.1: TRP selection - Use case 3.2: per-TRP DL/UL Rx-Tx phase compensation at NW side for reciprocity (e.g. using both CSI-RS and SRS for measurement) For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {FOn , n=0, 1, …, N – 1, n≠nref}, where FOn denotes the measured frequency offset associated with the n-th CSI-RS resource/resource set relative to the reference CSI-RS resource/resource set nref - For the reference CSI-RS resource/resource set nref, the value of FOnref is assumed 0 and not reported Agreements in RAN1#116bis For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {n,, n=0, 1, …, N – 1, n≠nref, =0,1,…,-1}, where n, denotes the measured phase offset between the n-th CSI-RS resource/resource set and the reference CSI-RS resource/resource set nref for the -th frequency unit - The value n, indicates a uniformly quantized phase between –A and A, or 0 and A - FFS: supported quantization alphabet(s) (including A and resolution) for n, For the Rel-19 aperiodic standalone CJT calibration reporting, for a given CJT calibration report of one [or more] CJT calibration report type, the nref is selected by the UE and reported as a part of the CJT calibration report - Note: CJT calibration report type refers to the Doffset/d report, FO report, and, TDD PO report For the Rel-19 aperiodic standalone CJT calibration reporting of {FOn , n=0, 1, …, NTRP – 1, n≠nref}, the value of FOn indicates a uniformly quantized frequency offset between 0 and AFO For the Rel-19 aperiodic standalone CJT calibration reporting, regarding phase offset reporting, the value n,, indicates a uniformly quantized phase between 0 and 2 For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for delay offset reporting Dn,offset, i.e. (AD, MD), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AD ={0.5CP, 0.75CP, CP, 1.5CP, 2CP,1/4∆f, 1/12∆f, 1/24∆f } where CP and ∆f denote the length of the cyclic prefix according to the current specifications (for normal CP) within a slot and the SCS, respectively - FFS: Further down-selection of the above candidate values for AD, including the use of a same unit for all supported values - MD ={32, 64} - FFS: If TDD TX/RX timing misalignment report is supported, whether different set of candidate MD values is needed In addition, the inside/outside range for the 1-bit indicator dn is equal to [0, CP]. FFS: Further implicit/explicit restriction(s) on candidate value(s) depending on the CSI-RS configuration For the Rel-19 aperiodic standalone CJT calibration reporting, an ‘invalid’ quantization state/hypothesis is supported for frequency offset and phase offset CJT calibration reporting - Note: already supported as ‘out-of-range’ for the (Dn,offset, dn) reporting - FFS (RAN1#117): The need for a condition/event for ‘invalid’ to be specified as a UE procedure e.g. RSRP-based For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for frequency offset reporting FOn, i.e. (AFO, MFO), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AFO = {0.01ppm, 0.1ppm, 0.2ppm, ∆f, ∆f/2, ∆f/4, ∆f/8, 1/(4∆t), 1/(8∆t), 1/(16∆t), 1/(32∆t), 1/(512∆t)} where ∆f and ∆t denote the SCS and duration of one OFDM symbol, respectively - FFS: Further down-selection of the above candidate values for AFO, including the use of a same unit for all supported values - MFO = {16,32} For the Rel-19 aperiodic standalone CJT calibration reporting, the resolution parameters for n,, i.e. M, are NW-configured via higher-layer (RRC) signalling from the candidate values {16, 32} For the Rel-19 aperiodic standalone CJT calibration reporting, in addition to reporting one type of CJT calibration report in one report, at least support reporting {(Dn,offset, dn), n=0, 1, …, NTRP – 1, n≠nref1} and {FOn , n=0, 1, …, NTRP – 1, n≠nref2} in one report - nref1 and nref2 are independently selected and indicated by the UE, - One-part UCI is used Agreements in RAN1#117 For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP} For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm} Agreements in RAN1#118 For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256.

According to RAN1 agreements, it can be seen that CJTC delay offset reporting, CJTC frequency offset (FO) reporting, CJTC phase offset reporting are aperiodic reporting. For the impact on RRM performance requirements, more discussion is needed. In Rel-18 MIMO, time domain channel property (TDCP) is introduced, which is defined as wideband normalized correlation between two CSI-RS transmission occasions, corresponding to CSI-RS resourece(s) from NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-info, that are separated by Dn symbols or slots, depending on the configuration, where Dn is the n-th delay configured value among DelayD configured delay values {D1, …, DY} and Y is number of configured delay values. From RRM perspective, TDCP measurement report mapping requirements including amplitude measurement report mapping and phase measurement report mapping are introduced. For Rel-19 aperiodic standalone CJT calibration reporting, similar as Rel-18 TDCP, at least report mapping need to be specified. RAN1 has some initial agreements on report range and resolution. For delay offset reporting, the dynamic range at least support the following values: {0.5CP, CP}. For frequency offset reporting, the dynamic range at least support the following values: {0.1ppm, 0.2ppm}. For phase offset reporting, it indicates a uniformly quantized phase between 0 and 2. For reporting range of these reporting type, whether other values are supported will be discussed later. Regarding resolution parameters, {16, 32, 256} are supported for frequency offset reporting and phase offset reporting, and {32, 64} are supported for delay offset reporting. Based on above RAN1 agreements, RAN4 can discuss how to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting. Except report mapping, accuracy requirements also need to be considered to guarantee measurement performance, but whether and how to specify the requirements are pending on RAN1 further agreements.

Proposal 1: for UE reporting enhancement for CJT calibration, it is proposed to define accuacy requirements.

Proposal 2: for UE reporting enhancement for CJT calibration, it is proposed to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting.

## 2.2 Enhancements for asymmetric DL sTRP/UL mTRP scenarios

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting. Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI? Way forward: - Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states. - Option 2: RAN4 to specify UL TCI state switching requirements. - Option 3: FFS. Issue 4-3: Whether to define RRM core requirements of pathloss offset update? Agreement: - Previous RAN4 agreement captured below still apply and close the issue. Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update? Agreement: Whether to define RRM core requirements of pathloss offset update? - Not define RRM requirement for MAC CE decoding delay. - The agreement can be revisited depending on further RAN1 progress..

Firstly, we would like to discuss the target scenario. According to the WID, asymmetric DL sTRP/UL mTRP deployment scenarios assume intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), and assume the Rel-17/18 unified TCI framework and fully reuse the legacy QCL/UL spatial relation rules, target FR1 and FR2. Since intra-DU is considered, it means that inter-cell is considered. And not changing existing cell definition or defining a new cell (e.g. UL-only cell) means that Pico site can transmit SSB/CSI-RS.

According to the discussion in last meeting, some companies are not sure whether ICBM is considered. We would like to provide more details. In RAN1 #117 meeting, RAN1 agreed to support ICBM in asymmetric DL sTRP/UL mTRP scenarios as duplicated as following.

RAN1#117: Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

From deployment point of view, ICBM is in use since normal TRP instead of UL-only TRP are deloyed in our  HetNet deployment. As shown in Fig.1, indoor pico site and outdoor macro site are deployed at same frequency with different cell ID (Inter-cell). Both indoor pico site and outdoor macro site transmit SSBs with different cell ID. In this case, when asymmetric DL and UL transmission scheme is used, SSBs from indoor pico site can be used as pathloss RS when UE transmits PUCCH and PUSCH to indoor pico site. The pathloss is more accurate compared to calculate pathloss from RSs from outdoor macro site and add a pathloss offset to obtain the pathloss for indoor pico site.

Fig.1 Inter-cell deployment with asymmetric DL and UL transmission

Observation 1: according to RAN1 agreements, ICBM is supported in asymmetric DL sTRP/UL mTRP scenarios.

For inter-cell deployment with asymmetric DL and UL transmission, SSB is transmitted from the indoor pico site (i.e. TRP with additional PCI), which can be used as pathloss RS when UE transmits PUCCH and PUSCH to indoor pico site. So pathloss offset is not needed which is used to calculate pathloss from RSs from outdoor macro site and add a pathloss offset to obtain the pathloss for UL-only indoor pico site.

Proposal 3: pathloss offset has no impact on ICBM based asymmetric DL and UL transmission, i.e. legacy requirements can be reused.

As discussed above, for inter-cell deployment with asymmetric DL and UL transmission, SSB is transmitted from the indoor pico site, which means that the SSB from indoor pico site can be configured as source RS for the transmition of PUCCH and PUSCH to the indoor pico site. In other words, legacy UL TCI state switching delay requirements, e.g. ICBM related requirements, can be reused

Proposal 4: it is proposed that legacy UL TCI state switching delay requirements can be reused for ICBM based asymmetric DL and UL transmission.

According to the revised WID in RAN#105 meeting, the objective on two TAs enhancements for sDCI is confirmed. Two TA mechanism for multi-DCI multi-TRP scenario, enabling UE to transmit with two different timings toward two different TRPs, are introduced in Rel-18, and related timing requirements are specified in RAN4. UL timing is derived based on the DL timing, two individual DL timing are needed, and the TCI states associated to a coresetPoolIndex corresponding to one TAG.

TS 38.133, 7.1 UE transmit timing For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point. . For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal. For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1. For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. Where NTA and NTA offset are commanded by the network independently for each TAG [TS 38.331]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in Table 7.1.2-2. In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18.

According to the revised WID, two TAs mechanism is extended to single DCI through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

RAN1 Agreements Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0: - This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset. - When one joint/UL TCI state is indicated in Rel-17 unified TCI, - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power. - FFS: Whether the bit field can be used to indicate other information - FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI

According to RAN1 agreements, one downlink reference timing is supported. For the ICBM based asymmetric DL sTRP/UL mTRP, the outdoor macro site (serving cell) and the the indoor pico site (TRP with additional PCI) transmit SSB, the downlink reference timing can be based on the DL timing of serving cell.

According to legacy TS 38.133, for multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission may be the DL timing of the reference cell associated with the first TAG or the DL timing of the reference cell associated with the second TAG. However, for Rel-19, since there is only one DL reference timing, the spec need to be updated. Also, according to legacy TS 38.133, the reference point for PRACH transmission is related with the value of “PRACH association indicator”. However, “PRACH association indicator” is not used for Rel-19 asymmetric DL sTRP/UL mTRP. According to RAN1 agreements, new DCI field is introduced., which means legacy timing requirements need to be updated to support two TA.

Proposal 5: for the supporting of 2TA, UE transmit timing requirements need to be updated considering that one downlink reference timing and new DCI filed are agreed in RAN1.

- Conclusion

This contribution provides discussion on RRM requirements on UE reporting enhancement for CJT calibration and enhancements for asymmetric DL sTRP/UL mTRP scenarios. The proposals are:

Proposal 1: for UE reporting enhancement for CJT calibration, it is proposed to define accuacy requirements.

Proposal 2: for UE reporting enhancement for CJT calibration, it is proposed to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting.

Observation 1: according to RAN1 agreements, ICBM is supported in asymmetric DL sTRP/UL mTRP scenarios.

Proposal 3: pathloss offset has no impact on ICBM based asymmetric DL and UL transmission, i.e. legacy requirements can be reused.

Proposal 4: it is proposed that legacy UL TCI state switching delay requirements can be reused for ICBM based asymmetric DL and UL transmission.

Proposal 5: for the supporting of 2TA, UE transmit timing requirements need to be updated considering that one downlink reference timing and new DCI filed are agreed in RAN1.

- Reference

- RP-234007, New WID: NR MIMO Phase 5

- RP-242394, WID revision: NR MIMO Phase 5

- 2416921, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung