# 3GPP TSG-RAN WG4 Meeting #113                                 R4-2418442

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.24.1

Source: 	CMCC

Title: 			 Discussion on inter-CU subsequent LTM

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #105 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meetin, there was initial discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on inter-CU subsequent LTM.

- Discussion

- Issue 3-1: other issues - Candidate solutions: - Proposal 1: it is proposed to define dedicated requirements, e.g.timing requirements, for UE based TA adjustment. (CMCC) - Proposal 2: For subsequent LTM, cell switch delay requirements for R18 LTM in clause 6.3 could be reused. (CMCC, ZTE) - Proposal 3: RAN4 to discuss whether to define requirements for subsequent LTM cell switching delay in R19. The requirements would be the same for both intra-CU and inter-CU. (HW) - Agreements: - It is RAN4 common understanding that R18 LTM cell switch requirements also apply for R18 subsequent LTM.

In August meeting, it was agreed that exsiting R18 LTM related RRM requirements also apply to same procedures in inter-CU scenario, FFS subsequent LTM.

Firstly, subsequent LTM is already supported for Rel-18 intra-CU LTM. In last meeting, it was also confirmed that R18 LTM cell switch requirements also apply for R18 subsequent LTM.

From RAN4 requirements point of view, Rel-18 LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.  DLTM = Tcmd  + TLTM-interrupt. Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement. TLTM-interrupt is interruption time, and TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms. In detail, TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. TLTM-IU is the interruption uncertainty during LTM cell switch which is related with whether RACH-based LTM cell switch or RACH-less LTM cell switch is in use.

The motivation to introduce subsequent LTM is to avoid RRC configuration between cell switches. According to above analysis, it can be seen that only the component of TLTM-RRC-processing is related with RRC, which is the the time for ASN.1 decoding and validity/compliance check for the RRC configuration. And TLTM-RRC-processing = 0 if the UE supports [earlyDecodingAndValidityCheck] capability, and some conditions are met. Otherwise,  TLTM-RRC-processing =10ms.

According to TS38.300, subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion. Rel-19 LTM target for inter-CU case. The difference between Rel-18 subsequent LTM and Rel-19 subsequent LTM is that the candidate LTM cells no need to be limited to intra-CU, which has no impact on the UE behaviour. The Rel-18 cell switch delay requirements can be reused.

Proposal 1: it is proposed that the Rel-18 cell switch delay requirements can be reused for Rel-19 inter-CU subsequent LTM.

- Conclusion

This contribution provides discussion on inter-CU subsequent LTM. The proposals are:

Proposal 1: it is proposed that the Rel-18 cell switch delay requirements can be reused for Rel-19 inter-CU subsequent LTM.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4