# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418455

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.17.2.1

Source: 	CMCC

Title: 			 Discussion on testability and interoperability issues for CSI compression

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on testability and interoperability issues for  CSI compression.

- Discussion

Issue 4-2: Selection criteria for decoder(s) - Proposals - Option 1: Select a single decoder, the decoder with best SGCS - Option 2: Select 3 decoders: with highest, lowest and closest to mean relative performance gain in SGCS over eTypeII decoder. - Option 3: Select 3 decoders randomly - Option 4: others

The discussion is for checking the feasibility of testing option 3 for CSI compression. Since this is for feasibility check, no need to spend much time on slection criteria. For simplicity, ransom selection can be considered. As for the number of selected decoders, we do not have strong view.

Proposal 1: for option 3 feasibility strudy, the decoders can be selected randomly.

Issue 4-4: Option 4a(Dataset based) for 2-sided model - Proposals - Option 1: Next steps: - Discuss/decide how to generate the training dataset and other assumptions - need to align the ML model, what should the dataset contain (channel/eigenvector realization, etc), how to generate the dataset (aggregate dataset based on inputs from multiple companies, etc), - Check on performance alignment see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models (encoder and/or decoder)/datasets (training/testing/inference) (sharing framework is currently under discussion) - Select one or more datasets for further analysis - Selection criteria is TBD - Companies bring results for “own encoder and decoder” with selected dataset - performance alignment to be checked ( - Conclude Option 4a feasibility - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). - Option 2: Others

Option 3 is to standarize test decoder. Instead of specification of decoder, option 4 is to standarize sufficient information so that TE vendor can develop a test decoder. In detail, there are 3 sub-options:

- Option 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

- Option 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

- Option 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

The motivation of this test feasibility study for two-sided mode is to reduce the impact due to TE implemeting the test decoder so that TE could provide alligned results and the tests are repeatable.

For option 4a, if only dataset is specified, it is questionable above motivation can be achived, since decoder implement by TE vendor and encoder implemented by UE vendor may be based on different backbone.

For option 4b, encoder is documented in the specification. Compared with option 3 with documenting doceder, there is not too much difference.

Option 4c, in our view, is a sub-set of option 3, which means option 3 discussion can be reused for option4.

What’s more, it is also proposed to document reference encoder and/or training dataset for Option 3 for the purpose of performance verification or TE test decoder verification. From this point of view, we do not see the ncessity to consider option 4 if option 3 is confirmed feasible.

Proposal 2: it is proposed to discuss if option 3 is feasible, whether option 4 is still needed.

- Conclusion

This contribution provides discussion on testability and interoperability issues for CSI compression. The proposals are:

Proposal 1: for option 3 feasibility strudy, the decoders can be selected randomly.

Proposal 2: it is proposed to discuss if option 3 is feasible, whether option 4 is still needed.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface