# 3GPP TSG-RAN WG4 Meeting #113                            R4-2418462

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	5.23.1

Source: 	CMCC

Title: 			 (NR_Mob_enh2-Core) Discussion on open issues for L1/L2 based inter-cell mobility

Document for:	Discussion

- Introduction

In last meeting, there is discussion on L1/L2 based inter-cell mobility, and a WF was approved [1]. This contribution provides further discussion on remaining open issues.

- Discussion

Issue 1-4-1-1: T/F tracking when TRS as QCL source in cell switch delay Ad hoc agreement <Agreement>: - Confirm current cell switch delay requirements are applicable to the case that TRS is configured as a QCL source in the indicated TCI state in cell switch command. - FFS whether Tfirst-ssb in cell switch delay requirements need to be updated, e.g. based on TRS.

According to the RAN1 feature group, it can be seen that SSB, TRS, or both as QCL source RS for cell switch and early TCI state are supported. To guarantee the performance, it is necessary to define requirements for the case with TRS as QCL source RS for cell switch and early TCI state. In last meeting, it was agreed that current cell switch delay requirements are applicable to the case that TRS is configured as a QCL source in the indicated TCI state in cell switch command.

From RAN4 requirements point of view, one impact to consider TRS as QCL source RS is to update Tfirst-RS for cell switch delay requirements. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. Currently, Tfirst-RS is the time to the first SSB transmission on the target cell [after Tcmd], which need to be updated to cover the time to the first TRS transmission.

In last meeting, companies not supporting to update Tfirst-RS to cover TRS argue that eventually the RS is QCLed to SSB. Howerver, this is for fine time tracking, after deriving rough timing based on SSB, why TRS is not used for fine timing if TRS comes earlier. A companied CR is provided

Proposal 1: it is proposed to update Tfirst-ssb to cover TRS in cell switch delay requirements.

- Conclusion

This contribution provides discussion on the open issues for L1/L2 based inter-cell mobility. The proposals are:

Proposal 1: it is proposed to update Tfirst-ssb to cover TRS in cell switch delay requirements.

- Reference

- R4-2406502, WF on NR mobility enhancements (part 1), MediaTek Inc.

- R4-2413960, WF on Further NR mobility enhancements, MediaTek Inc.

- R4-2410302, WF on NR mobility enhancements (part 1), MediaTek Inc.

[3] R4-2321621, WF on NR mobility enhancements (part 1), MediaTek Inc.

[4] R4-2403385, WF on NR mobility enhancements (part 1), MediaTek Inc.