# 3GPP TSG-RAN WG4 Meeting #113	R4-2418467

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.2.2

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

For solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, CSSF value is related to the number of serving cells, with large number of serving cells, L3 measurement delay would be long, thus the number of serving cells related enhancements in CSSF equation can be considered as a starting point. It’s supported that UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CC to be measured or CCs not to be measured, we prefer to be based on network indication. Network can indicate that the other CCs need not to be measured and reported, and network assume the same measurement results on the measured CC and unmeasured CCs in the same band.

Besides, the solution of changing CSSF allocation (statically or dynamically) among PCC/PSCC/SCC can be considered, UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

Proposal 1: It’s supported that UE can measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 2: For the CCs to be measurement, it’s preferred to be based on network indication. Network can indicate that the other CCs need not to be measured and reported, and network assume the same measurement results on the measured CC and unmeasured CCs in the same band.

Proposal 3: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

Status in the WF [1] and summary [2]:

Issue 2-6: feature capability of L3 measurement delay reduction by optimizing CSSF Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Option 1 (CATT): The feature capability depends on the solution: - For option 1 and option 2 in issue 2-5, a new UE capability is needed to indicate the UE support of new solution. - For option 3 in issue 2-5, the existing per-FR gap capability can be reused. - Option 2 (Apple, Nokia, CTC, Samsung): RAN4 to introduce a new individual capability for CSSF reduction in R19. - Option 3 (Apple, CTC, HW, Ericsson, Samsung): delay the capability discussion to the end of the core part. - Recommended WF - Moderator note: - Check if following option is agreeable: - Option 4(moderator): RAN4 to introduce a new individual capability for CSSF reduction in R19. - delay the detailed capability design to the end of the core part.

It’s proposed to introduce a new individual capability for CSSF reduction in R19, and the capability discussion can be delayed to the end of the core part.

Proposal 4: It’s proposed to introduce a new individual capability for CSSF reduction in R19, and the capability discussion can be delayed to the end of the core part.

Status in the WF [1] and summary [2]:

Issue 2-7: Other WID scope discussion Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2415656. - Proposal 1(CMCC, ZTE, CTC): after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable. - Recommended WF - [Moderator note]: - Check if proposal 1 is agreeable?

The bands in FR1 are widely used in network deployment, there also exists the demand of L3 measurement delay reduction in FR1, from operator's perspective, the solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

Besides, in RAN #103 meeting, it was regarded as common understanding that RAN4 will consider to apply CSSF enhancements in FR1 once the work for FR2-1 is done, in our views, the CSSF enhancements in FR1 is also in the WID scope.

Proposal 5: The solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC, with the following proposals:

Proposal 1: It’s supported that UE can measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 2: For the CCs to be measurement, it’s preferred to be based on network indication. Network can indicate that the other CCs need not to be measured and reported, and network assume the same measurement results on the measured CC and unmeasured CCs in the same band.

Proposal 3: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

Proposal 4: It’s proposed to introduce a new individual capability for CSSF reduction in R19, and the capability discussion can be delayed to the end of the core part.

Proposal 5: The solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

- References

- R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.

- R4-2415656, Topic summary for [112bis][209] NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.