# 3GPP TSG-RAN WG4 Meeting #113	R4-2418469

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.3.1

Source:	China Telecom

Title:	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1] and summary [2]:

#### Issue 1-1-3: Measurement methods - Recommended WF - Further discuss the issue - Background Agreement (from RAN1#118) For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband. - Proposals - Proposal 1 (vivo, ZTE, HW, MTK): - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions. - Proposal 2 (LGE): - The UE to support L1 based UE-to-UE CLI measurement has to capable to measure the CLI measurement with sub-band unit rather than DL-active BWP unit. - Proposal 3 (Nokia): - RAN4 to define the CLI measurement requirements in SBFD as the following: - SRS-RSRP measurement requirements for Method#2. - CLI-RSSI measurement requirements for Method#1 and Method#3. - Proposal 4 (Samsung, CTC, QC): - RAN4 shall specify L1 CLI measurement requirements at least for below methods - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Recommended WF - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

In RAN1 #118 meeting, it was agreed that for L1 UE-to-UE CLI measurement and reporting, CLI measurement is performed within the active DL BWP, Method#1 and Method#2 are supported, and FFS for Method#3. In our views, RAN4 can discuss the requirements for UE-to-UE CLI measurements based on Method#1 and Method#2. Besides, we support that differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

Proposal 1: RAN4 to discuss the requirements for UE-to-UE CLI measurements based on Method#1 and Method#2, and differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

Status in the WF [1] and summary [2]:

#### Issue 1-1-7: Measurement period - Recommended WF - Further discuss the issue - Proposals for L1-SRS-RSRP - Proposal 1 (vivo): - Requirements based on 1 sample measurement assumption should be used. - Proposal 1a (QC, Ericson): - RAN4 to define the measurement delay for aperiodic reporting of L1-SRS-RSRP at least based on a single sample. - Proposal 2 (CATT): - Re-use the framework of R16 CLI measurement period except for the sample number. - Proposal 3 (Nokia): - Follow L1-RSRP measurement approach, and both 1 and 3 samples are considered based on network configuration - Proposal 3a (Samsung): - RAN4 can evaluate L1 SRS-RSRP measurement performance considering both single shot and multiple shots measurements if different side condition identified - Proposals for L1-CLI-RSSI - Proposal 1 (vivo, Nokia, HW, QC): - Requirements based on 1 sample measurement assumption should be used - Proposal 2 (E///): - Rel-16 CLI-RSSI requirement framework can be used as a baseline. Further updates can be made when RAN1 finalized the measurement RS configuration and method. - Recommended WF - For L1-SRS-RSRP, further discuss the following options based on simulation results. - Option 1: single set of requirements based on [1] sample - Option 2: two sets of requirements based on [1] and [3] samples - Other options are not precluded - For L1-CLI-RSSI, measurement period requirements are defined based on 1 sample.

For L1-SRS-RSRP measurement, we support to follow same assumption as L1-RSRP measurement requirement that both 1 and 3 samples based measurement period requirements can be considered. For L1-CLI-RSSI, we support to use same assumption as legacy CLI-RSSI measurement requirement that 1 sample based measurement period requirements can be considered.

Proposal 2: For L1-SRS-RSRP measurement, both 1 and 3 samples based measurement period requirements can be considered.

Proposal 3: For L1-CLI-RSSI measurement, 1 sample based measurement period requirements can be considered.

Status in the WF [1] and summary [2]:

#### Issue 1-1-8: Measurement reporting - Recommended WF - Further discuss the issue - Proposals - Proposal 1 (vivo): - Define requirements for periodic, semi-persistent reporting. - Proposal 2 (Samsung, CTC, HW, MTK): - Focus on aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1 - Proposal 3 (CATT, Nokia, HW, E///): - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused, and wording can be adapted when reporting scheme design is stable. - Proposal 4 (MTK): - FFS whether L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement. - Recommended WF - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement. - For aperiodic reporting, L1-RSRP measurement reporting requirement can be re-used as baseline.

L1 based UE-to-UE CLI measurement and reporting are based on existing CSI framework, measurement resource can be periodic, semi-persistent, or aperiodic measurement resource, and measurement reporting is at least aperiodic reporting. Rel-19 L1 CLI measurement reporting is different from Rel-16 CLI measurement reporting including periodic, event-triggered periodic, event triggered reporting, therefore, Rel-19 L1 CLI measurement reporting requirements need to be specified. RAN4 shall define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting, and L1-RSRP measurement reporting requirement can be re-used as baseline.

Proposal 4: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting, and L1-RSRP measurement reporting requirement can be re-used as baseline.

Status in the WF [1] and summary [2]:

#### Issue 1-1-9: Measurement accuracy - Recommended WF - RAN4 to define measurement accuracy requirements for L1-SRS-RSRP and L1-CLI-RSSI in the Performance part of the WI. - Proposals for L1-SRS-RSRP - Proposal 1 (vivo): - The accuracy requirement in Rel-16 can be used as baseline. - Proposal 2 (ZTE, CTC): - RAN4 not to reuse legacy UE-to-UE CLI accuracy requirements directly. Determining the accuracy requirements based on the evaluation results. - Proposal 3 (HW, MTK): - RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period. - Proposals for L1-CLI-RSSI - Proposal 1 (HW): - RAN4 to define measurement accuracy requirements for L1-CLI-RSSI in Perf part. - Recommended WF - RAN4 to define measurement accuracy requirements for L1-SRS-RSRP based on the simulation results and agreed side condition and measurement sample.

For L1 UE-to-UE CLI measurement accuracy requirements, it’s proposed to define measurement accuracy requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. Since Rel-16 CLI measurement requirements are based on L3 framework while Rel-19 CLI measurement requirements are based on L1 framework, it’s proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly. The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

Proposal 5: RAN4 to define L1 UE-to-UE CLI measurement accuracy requirements, and it is proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly. The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

- Conclusion

The contribution presents our views on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation, with the following proposals:

Proposal 1: RAN4 to discuss the requirements for UE-to-UE CLI measurements based on Method#1 and Method#2, and differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

Proposal 2: For L1-SRS-RSRP measurement, both 1 and 3 samples based measurement period requirements can be considered.

Proposal 3: For L1-CLI-RSSI measurement, 1 sample based measurement period requirements can be considered.

Proposal 4: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting, and L1-RSRP measurement reporting requirement can be re-used as baseline.

Proposal 5: RAN4 to define L1 UE-to-UE CLI measurement accuracy requirements, and it is proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly. The measurement accuracy requirements for L1-SRS-RSRP can be based on the simulation results and agreed side condition and measurement sample.

- References

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.

- R4-2415661, Topic summary for [112bis][214] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.