# 3GPP TSG-RAN WG4 Meeting #113	R4-2418470

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.3.2

Source:	China Telecom

Title:	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, RRM impacts for SBFD operation for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM impacts for SBFD operation for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

#### Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

In our views, for SBFD-aware UE, since the serving cell SSB based measurement is prioritized to UL transmission, we support to follow same principle for SSB based neighbor cell measurement, i.e. SSB measurement is prioritized over UL transmission.

Proposal 1: It’s proposed that for SBFD-aware UE, existing requirements apply for SSB based neighbor cell measurement, i.e. SSB measurement is prioritized over UL transmission.

Status in the WF [1]:

#### Issue 1-2-2: Requirements for CSI-RS based measurement - Agreements (from online session) - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

For CSI-RS based measurement requirements, it was agreed that RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations, CSI-RS colliding with dynamically scheduled UL transmission, CSI-RS puncturing in frequency domain in SBFD symbols, and CSI-RS distributed over SBFD and non-SBFD symbols.

For CSI-RS colliding with dynamically scheduled UL transmission, in our views, CSI-RS measurement occasion will be dropped and CSI-RS measurement period should be extended. It’s preferred to use a scaling factor to guarantee measurement performance.

For CSI-RS puncturing in frequency domain in SBFD symbols, CSI-RS resources may be split across two DL subbands. If the number of consecutive CSI-RS PRBs which is within one DL subband is no less than 48 PRBs, we support that existing accuracy requirements for CSI-RS based measurements can be applied.

Proposal 2: For CSI-RS colliding with dynamically scheduled UL transmission, CSI-RS measurement occasion will be dropped, CSI-RS measurement period should be extended, and a scaling factor is preferred.

Proposal 3: For CSI-RS puncturing in frequency domain in SBFD symbols, if the number of consecutive CSI-RS PRBs which is within one DL subband is no less than 48 PRBs, it’s supported that existing accuracy requirements for CSI-RS based measurements can be applied.

Status in the WF [1] and summary [2]:

#### Issue 1-2-7: Requirements for UL resource muting - Recommended WF - Interested companies are encouraged to bring proposals on exact RRM impacts, if any - Proposals - Proposal 1 (vivo, LGE, Samsung, HW): - RAN4 to confirm there is no RRM impact due to UL resource muting. - Proposal 2 (E///): - Wait for the further RAN1 conclusion and decide whether there is RRM impacts - Recommended WF - As baseline, there is no RRM impact due to UL resource muting. - This can be revisited if impacts are identified based on further RAN1 agreements.

For UL resource muting requirements, we support there is no RRM requirement impacted by UL resource muting.

Proposal 4: There is no RRM impact due to UL resource muting.

- Conclusion

The contribution presents our views on RRM impacts for SBFD operation for evolution of NR duplex operation, with the following proposals:

Proposal 1: It’s proposed that for SBFD-aware UE, existing requirements apply for SSB based neighbor cell measurement, i.e. SSB measurement is prioritized over UL transmission.

Proposal 2: For CSI-RS colliding with dynamically scheduled UL transmission, CSI-RS measurement occasion will be dropped, CSI-RS measurement period should be extended, and a scaling factor is preferred.

Proposal 3: For CSI-RS puncturing in frequency domain in SBFD symbols, if the number of consecutive CSI-RS PRBs which is within one DL subband is no less than 48 PRBs, it’s supported that existing accuracy requirements for CSI-RS based measurements can be applied.

Proposal 4: There is no RRM impact due to UL resource muting.

- References

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.

- R4-2415661, Topic summary for [112bis][214] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.