# 3GPP TSG-RAN WG4 Meeting #113                             R4-2418481

# Orlando, USA, 18th – 22rd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for regenerative NTN enhancement and downlink coverage enhancement

Agenda item:	7.26.4.2

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. As this is the first meeting for RAN4 RRM to work on this WI, we will provide our initial views on potential impacts on RRM requirements.

Discussion

The objectives of Rel-19 NTN phase 3 enhancement are described in [1] including:

Downlink coverage enhancement - 2.2 Support of Regenerative payload

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objectives. The objectives are divided into several aspects:

  - 2.1 Downlink coverage enhancement

RAN4 shall define the RRM requirements for downlink coverage enhancement based on the latest revised WID as below:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

For this objective, RAN1/RAN2 have discussed several meetings and the goal is to increase the downlink coverage to serve the multiple satellite footprint. RAN1 introduced the beam hopping since the limitation of power budget, the number of RF chains available at the satellite payload  and the number of active beams can not be increased. Thus, we deem that if the downlink coverage is needed to enhance, the beam hopping shall be considered. Based on the latest WID, the SSB periodicity shall be extended and this extension will cause the RAN4 impact. In this section we will analyze the basic scenario of beam hopping and then the potential RAN4 impact with the RAN1/RAN2’s conclusion.

Beam hopping employs TDM approach to serve multiple beam footprints with a single beam, thereby enhancing the coverage area. This technique enables rapid switching of the satellite beam among different geographical regions, effectively boosting the coverage expanse and spectrum utilization efficiency without necessitating additional hardware infrastructure. The beam hopping can be seen as below:

Figure 1 Beam hopping

Based on the above figure, we see that the satellite uses one beam to serve 3 beam footprints, and this single beam using the beam hopping employs the TDM method to cover these beam footprint in order to enhance downlink coverage.

In the latest WID, the SSB periodicity shall be extended. The reason for this enhancement is that a satellite only provides 16 beams simultaneously due to the limitation of power budget or number of RF chains. What we want to realize is that using these 16 beams to serve multiple beam footprints such as 1058 beam footprints. However, when one single beam serve one beam footprint, the integral information of SSB, SIB1, SIB19 etc shall be sent, and a dwell time is needed. Assuming the minimum dwell time is 5ms, the existing default SSB periodicity is 20ms, that is, that one beam only serves 20ms/dwell time 5 ms=4 beam footprints. For all 16 beams, the total serve coverage is 16*4=64 beam footprints which is far less than 1058 beam footprints. Thus, the SSB periodicity shall be extended in order to enhance the downlink coverage, but the concrete values for SSB periodicity depends on RAN1 and they are under discussion.

In the last meeting, RAN4 has the general agreement as below:

### Issue 1-1: SSB periodicity enhancement Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc.

In the last meeting, RAN1 has already reached the agreements as below:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

From my perspective, there are two kinds of SSB periodicity extension shall be clarified:

- (Default) SSB periodicity extension for cell search (initial UE access).  This will affect its initial access and connection establishment.

- If the SSB periodicity extension is larger than 20ms, when attempting to access the network, the UE may require additional time to accurately acquire essential System Information Blocks, paging messages, and signals on other physical channels. This could result in reduced efficiency of the connection establishment process and increased access latency.

- If the SSB periodicity does not align with the transmission cycles of other signals, it may result in a misalignment of the reception window, thereby affecting the reception efficiency of the UE.

- (Configured) SSB periodicity extension for measurement configurations etc. Compared to cell search, the UE will be configured by network and the SMTC periodicity will also be impacted.

- The network configures the SSB periodicity as 5ms, 10ms, 20ms, 40ms, 80ms, or 160ms. If the field is absent, UE assumes the periodicity of 5ms.

- The SMTC periodicity can be either 5ms, 10ms, 20ms, 40ms, 8ms, or 160ms.

We find that the SSB periodicity for UE initial access can be extended at least 160ms form RAN1’s conclusion. From my perspective, there is no requirements on cell selection (initial access) in RAN4’s spec. Thus, there is no RRM impact if the default SSB periodicity is extended to at least 160ms.

To my understanding, some RRM requirements will be impacted if the SSB periodicity is extended for configured SSB. Since the cell re-selection and L1/L3 measurements will be based on SSB signal. However, RAN1 and RAN2 are still discussing downlink coverage enhancement and the concrete values have not been decided yet. For example, for timing error requirementsin NTN, the UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. If SSB periodicity is larger than 160ms, there may be no SSB occurred during the last 160ms and the timing requirements will be impacted.

Observation 1: The SSB periodicity shall be extended in order to enhance the downlink coverage and RAN1/RAN2 are under discussion.

Observation 2: Some RRM requirements will be impacted if the SSB periodicity is extended since the cell re-selection and L1/L3 measurements are based on SSB signal.

Proposal 1: Some RRM requirements shall be updated if RAN1/RAN2 have the concrete conclusions on SSB periodicity extension.

Proposal 2: Timing requirements shall be revisited if the SSB periodicity is extended.

In R18 NES work item, they introduced the cell specific DTX/DRX in order to reduce the power consumption. One key point is that the SSB transmission is not limited in active state or non-active state to guarantee the time and frequency sync and measurement. RAN4 may follow this behaviour in NTN. However, this aspect is still under discussion in other working groups and RAN4 shall wait for other WGs’ conclusion.

Proposal 3: For cell specific DTX/DRX, RAN4 shall wait for other WGs’ conclusion.

In RAN1 previous meeting, one agreement can be seen as below:

For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

We find that RAN1 has already defined the concrete meaning for total number of beam footprints and the following figure can explain this:

Figure 2 Total beam footprints

We find that N1 beam footprints are in state “off” which means NTN service is not-available and UE will not receive any downlink signals including SSB and  CSI-RS which are closely relevant to RAN4 measurements such as L3/L1 measurement in RRC Connected state and cell re-selection measurement in RRC Idle/Inactive state. UEs in N1 coverage will not perform any RRM measurements.

Observation 3: N1 beam footprints are in state “off” which means NTN service is not-available and UE will not receive any downlink signals including SSB and  CSI-RS.

Proposal 4: UEs in N1 coverage will not perform any RRM measurements such as L3/L1 measurement in RRC Connected state and cell re-selection measurement in RRC Idle/Inactive state.

  - 2.2 Support of Regenerative payload

For NR NTN phase 3, the next objective is regenerative payload which is led by RAN3 and we found that there also has not been much progress in RAN2. We can analyze the potential RRM impact although there has been no discussion triggering RAN4 RRM at present based on the latest revised WID as below:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

The following point can be discussed:

- Multiple SMTCs

- RACH-less handover

- Satellite switch

- RRC Re-establishment

- Timing

2.2.1 Multiple SMTCs

In legacy, the multiple SMTCs shall be reused since this feature is not decided by types of UE. However, we also deem that the multiple SMTCs can be reused in this objective. If the elevation angles between different satellites are too large, the time difference of the SSBs arriving at the UE from different satellites is considered significant enough that they would require different SMTCs for reception. Multiple SMTCs would then measure different target satellites. Therefore,  RAN4 shall clarify whether the legacy multiple SMTCs mechanism can be used for regenerative payload or not.

Proposal 5: RAN4 shall clarify whether the legacy multiple SMTCs mechanism can be used for regenerative payload or not.

2.2.2 RACH-less handover

RAN2 has already reached the agreement as below:

- RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario. 2 RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

RAN2 confirms the legacy UEs shall also be supported in the regenerative mode. Also, the existing RACH-less handover procedure for transparent payload scenario can be directly used in regenerative mode. Thus, we deem that the procedure for RACH-less handover is not changed and the existing RACH-less handover requirements shall be reused for regenerative mode.

Observation 4: RAN2 confirms the existing RACH-less handover procedure for transparent payload scenario can be directly used in regenerative mode.

Proposal 6: The existing RACH-less handover requirements shall be reused for regenerative mode.

2.2.3 Satellite switch

The satellite switch has been discussed in RAN plenary meeting, the concrete discussion can be seen as below:

We find that there is no conclusion on whether support the satellite switch with re-sync on regenerative payload or not. Thus, this requirement shall not be considered in this stage from RRM perspective.

Observation 5: There is no conclusion on whether support the satellite switch with re-sync on regenerative payload or not.

Proposal 7: RAN4 shall not consider satellite switch on regenerative payload in this stage.

2.2.4 RRC Re-establishment

For RRC Re-establishment requirements, RAN3 has reached one agreement as below:

Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

We know that RAN3 has supported UE performing RRC connection resume and RRC connection re-establishment for regenerative payload architecture. From RAN4 perspective, the existing requirements could be considered for regenerative payload.

Proposal 8: From RAN4 perspective, the existing requirements could be considered for regenerative payload.

2.2.5 Timing

For regenerative payload, we deem that the discussion of timing advance is one key point. In R17, RAN4 just discussed the UE TA pre-compensation and the reference ponit has been introduced by RAN1. When the reference point is on feeder link, UE will compensate the timing of service link and part of feeder link which is decided by position of reference point.  Compared to transparent payload, the regenerative payload will not consider the feeder link since the gNB is also deployed in payload and only service link has the data transmission between UEs and regenerative payload. In the existing specification we find :

The UE shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

In legacy, the detailed timing analysis can be seen as below:

Figure 3 Timing of transparent payload

NTA: The value NTA of timing advance is indicated in Msg2 which is the timing adjustment be network.

NTA-offset: A UE can be provided a value NTA-offset of timing advance offset for a serving cell.

NTA,adjcommon: It is derived from the higher-layer parameters contained assistance information for the serving cell. It is the 			network-controlled common timing advanced value between reference point and satellite on feeder link.

NTA,adjUE: UE determines it by using the serving satellite position and its own position (GNSS capability) on service link. 			For the serving satellite position, it is calculated by UE based on higher-layer ephemeris parameters for a serving satellite

KMAC:  There is a misalignment between the uplink and downlink frames of the gNB, a scheduling time 				offset (configured offset) is introduced.

Koffset: It is a configured scheduling offset.

For the uplink transmission in the aforementioned highlight formula, the first three parameters are indicated to the UE by the network, while the last parameter is for the UE's own uplink compensation. For the third parameter, it equals zero in the regenerative mode. Based on figure 3, KMAC is also be zero. And RAN2 also has the same understanding as below:

RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

Regarding Te, we still need to consider the relevant issue in the regenerative mode—the gradual timing adjustment and timing advance adjustment accuracy. We understand that these have little impact, and they can be defined based on the understanding of the UE's uplink timing compensation behavior.

Observation 6: Compared to transparent payload, the reference point is only on payload not on the feeder link, where the common TA and KMAC is 0.

Proposal 9: RAN4 shall clarify that there is no difference between transparent payload and regenerative payload on timing advance.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN enhancement phase 3.

Observation 1: The SSB periodicity shall be extended in order to enhance the downlink coverage and RAN1/RAN2 are under discussion.

Observation 2: Some RRM requirements will be impacted if the SSB periodicity is extended since the cell re-selection and L1/L3 measurements are based on SSB signal.

Proposal 1: Some RRM requirements shall be updated if RAN1/RAN2 have the concrete conclusions on SSB periodicity extension.

Proposal 2: Timing requirements shall be revisited if the SSB periodicity is extended.

Proposal 3: For cell specific DTX/DRX, RAN4 shall wait for other WGs’ conclusion.

Observation 3: N1 beam footprints are in state “off” which means NTN service is not-available and UE will not receive any downlink signals including SSB and  CSI-RS.

Proposal 4: UEs in N1 coverage will not perform any RRM measurements such as L3/L1 measurement in RRC Connected state and cell re-selection measurement in RRC Idle/Inactive state.

Proposal 5: RAN4 shall clarify whether the legacy multiple SMTCs mechanism can be used for regenerative payload or not.

Observation 4: RAN2 confirms the existing RACH-less handover procedure for transparent payload scenario can be directly used in regenerative mode.

Proposal 6: The existing RACH-less handover requirements shall be reused for regenerative mode.

Observation 5: There is no conclusion on whether support the satellite switch with re-sync on regenerative payload or not.

Proposal 7: RAN4 shall not consider satellite switch on regenerative payload in this stage.

Proposal 8: From RAN4 perspective, the existing requirements could be considered for regenerative payload.

Observation 6: Compared to transparent payload, the reference point is only on payload not on the feeder link, where the common TA and KMAC is 0.

Proposal 9: RAN4 shall clarify that there is no difference between transparent payload and regenerative payload on timing advance.

- 4References

[1] RP-241667，Rel-19 WID NR NTN phase 3.

[2] TS38.133

[3] TS38.304

[4] TS38.331