3GPP TSG-RAN WG4 Meeting # 113                                R4-2418502

Orlando, US, 18th – 22nd November, 2024

Agenda item:	 7.4.3

Source:        ZTE Corporation, Sanechips

Title:           Discussion on RRM impacts for less than 5MHz BW

Document for:  Approval

Introduction

The work item on NR support for dedicated spectrum less than 5MHz for FR1 was approved at RAN#103. according to the WID, RAN4 shoud define RRM requirements for inter-band CA and DC for combinations introduced in RF part and main RAN4 scopes are summarized as following:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In this contribution, we will provide our views on RRM impacts for less than 5MHz BW.

Discussion

As per the WID in Rel-19, CA and DC are supported and the RRM requirements maybe impact due to introduction of NR in less than 5 MHz bandwidth. RANP agreed to use a PBCH transmission bandwidth is 20 PRBs for 5MHz channel bandwidth and a PBCH transmission bandwidth is 12 PRBs for 3MHz channel bandwidth. For the case that CBW equals to 3MHz, the transmission bandwidth for PBCH is smaller and RAN4 should further discuss how to update relative requirements.In last meeting, we have reached agreements on many issues and there are some leftover issues should be discissed in this meeting [1].

  - SCell configured with less than 5MHz bandwidth

In RAN4#112bis meeting, we have discussed the RRM scope for SCell configured with less than 5MHz bandwidth. And there is a remain issue on whether to consider intra-band CA in R19, copy as follows.

Issue 1-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations. - Option 3: No need to discuss since there is no impact to RRM requirements in Rel-19.

According to the WID, RAN4 should define RRM requirements for inter-band CA and DC for combinations introduced in RF part. It is clearly stated that we should discuss the scenario of inter-band. So, for this issue, we prefer to focus on inter-band CA and wait until RF room has concrete conclusion on intra-band CA combinations. As to option 3, it’s ok of there is no impact to RRM requirements. However, as other company mentioned, intra-band CA may have impact om the requirements, e.g. the SCell activation delay for the case where the SCell being activated is intra-band contiguous to an active cell.

Proposal 1: Focus on inter-band CA and wait until RF room has concrete conclusion on intra-band CA combinations.

Issue 1-1-2: Measurement requirements for deactivated SCC: - Proposals - Option 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table. - Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1) DRX cycle; TSSB_time_index_intra. No DRX; Ceil(7 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. NOTE 1: The requirements also apply to deactivated SCG SCell..

As to Measurement requirements for deactivated SCC, RAN4 needs to specify SSB index identification delay requirements for UE with deactivated SCell measurements on a target cell with less than 5MHz SSB BW in FR1, taking into consideration the impacts from PBCH puncturing.

- In Rel-18, RAN4 has discussed SSB index identification delay for intra-frequency measurement and inter-frequency measurement. Finially, RAN4 has reached concensus that 3+4 samples for intra-frequency measurement and 3+3 samples for inter-frequency measurement to acquire the index of the SSB being measured. The definiation of time period for time index detection for a UE operating on a target cell with 12 PRB SSB has been captured in clause 9.2.5.1, 9.2.6.2, 9.3.4, and 9.3.9, applicable to FR1 intra-f without gap, FR1 intra-f with gap, FR1 inter-f with gap and FR1 inter-f without gap, respectively.

Observation 1: In Rel-18, SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples

- In our understanding, same requirements can be reused for deactivated SCell. In current spec., time period for time index detection for deactivated SCell as follow.

Table 9.2.5.1-6: Time period for time index detection, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Therefore, for SSB index identification delay, we prefer to reuse R18 requirements to compensate the losses caused by punching. Though the SCell is in inter-band, SCell measurement is considered as intra-frequency measurement due to it is a serving cell.

Proposal 2: For the SSB index identification delay requirements for deactivated SCC, time index detection for a UE operating on a target cell with 12 PRB SSB defined in R18 can be reused.

2.2PSCell configured with less than 5MHz bandwidth

Issue 1-2-2a: PSCell change delay requirements: - Proposals - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements. ## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. In the case of NR-DC PSCell change in FR1, when the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target PSCell that is with 12PRB SSB bandwidth, the UE shall transmit PRACH preamble towards the target PSCell no later than the delay requirements specified in clause 8.9.2.. - Option 2: No need to update the text. Issue 1-2-2b: Conditional PSCell change delay requirements: - Proposals - Option 1: In subclause 8.11B.2, add T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW. Issue 1-2-3a: Handover with PSCell: - Proposals - Option 1: SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. - Option 2: The HO requirements with PSCell-PCell and PSCell-PSCell can be reused for NR CA/DC in less than 5MHz.

In the previous meeting, we have discussed and there was no consensus on whether to study the impacts of less than 5MHz, in particular PBCH puncturing, to PSCell change delay requirements. Some company proposed that PScell change requirement is based on PScell addition with additional margin. After checking, we can observe the requirements for PSCell change is same as PSCell addition in clause 8.9.2. Therefore, just need to consider the extension for PSCell addition, and no need to update PScell change requirement.

TS 38.133 clause 8.11 The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than specified in clause 8.9.2 for the case of NR-DC and in TS 36.133 clause 7.31.2 for the case of EN-DC

Observation 2: PScell change requirement is based on PScell addition and no need to update PScell change requirement in specfication.

For handover with PSCell, the requirements for PCell interruption and PSCell change are based on handover and PSCell addition. Therefor, no further extension is needed.

When handover with PSCell from NR-DC to NR-DC is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: - Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2. ... When handover with PSCell from NR-DC to NR-DC is commanded, the PSCell change time shall be less than DHOwithPSCell_PSCell: - DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_PCell + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms Where: - TRRC_delay, Tsearch_PSCell, T∆ and TPSCell_ DU are the same as defined in clause 8.9.2.

Observation 3: Partial components of handover with PSCell requirement is based on PScell addition and handover, no need to update handover with PSCell requirement in specfication.

For issue Issue 1-2-2a and Issue 1-2-3a, in our understanding, there is no conflict here, option 1 and option 2 are essentially the same.

Proposal 3: for PSCell change delay requirements and Handover with PSCell, SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. And no need to update the text in TS 38.133.

However, as to the conditional PSCell change delay and subsequent conditoinal PSCell change delay requirements, we should consider the extension and modification on text.

TS 38.133 clause 8.11B.2 The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Conditional: Where: - Slot n is the last slot overlapping with the PDSCH containing conditional PSCell change. - Tconfig_PSCell_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms ... - T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms. ... TS 38.133 clause 8.11E.2 The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Subsequent_Change_Conditional: Where: - Slot n is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change. - Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms ... - T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms. ...

Proposal 4: Conditional PSCell change delay and subsequent conditoinal PSCell change delay requirements (subclause 8.11B.2 and 8.11E.2), T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW.

Issue 1-2-4: SCG activation delay requirements: - Proposals - Option 1: RAN4 to define new SCG activation requirements for PSCell with 12RB PBCH, and same delay extension as for PSCell addition or PSCell change is re-used. - RAN4 to capture the following text for SCG activation delay: T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

For this issue, we support to define SCG activation requirements for PSCell with 12RB PBCH, which is similar to the PSCell activation delay requirements. In last meeting, we have reached consensus on PSCell addition delay requirements that specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth.

Proposal 5: for SCG activation delay requirements, T∆ = 3*Trs ms for target cells operating with 12 PRB SSB BW.

Issue 1-2-5: Measurement requirements for deactivated PSCell: - Proposals - Option 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table. - Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1) DRX cycle; TSSB_time_index_intra. No DRX; Ceil(7 x Kp) x measCyclePSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra.

Similar as Measurement requirements for deactivated SCC, we support to specify time period for time index detection for a UE operating on a target cell with 12PRB SSB.

Proposal 6: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB,, time index detection for a UE operating on a target cell with 12 PRB SSB defined in R18 can be reused.

Conclusions

In this contribution, we provide our views on the RRRM impacts for less than 5MHz BW and the proposals are:

Proposal 1: Focus on inter-band CA and wait until RF room has concrete conclusion on intra-band CA combinations.

Observation 1: In Rel-18, SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples

Proposal 2: For the SSB index identification delay requirements for deactivated SCC, time index detection for a UE operating on a target cell with 12 PRB SSB defined in R18 can be reused.

Observation 2: PScell change requirement is based on PScell addition and no need to update PScell change requirement in specfication.

Observation 3: Partial components of handover with PSCell requirement is based on PScell addition and handover, no need to update handover with PSCell requirement in specfication.

Proposal 3: for PSCell change delay requirements and Handover with PSCell, SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. And no need to update the text in TS 38.133.

Proposal 4: Conditional PSCell change delay and subsequent conditoinal PSCell change delay requirements (subclause 8.11B.2 and 8.11E.2), T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW.

Proposal 5: for SCG activation delay requirements, T∆ = 3*Trs ms for target cells operating with 12 PRB SSB BW.

Proposal 6: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB,, time index detection for a UE operating on a target cell with 12 PRB SSB defined in R18 can be reused.

References

- R4-2416849, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel