3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418506

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.24.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

Discussion

Compared to R18 LTM, conditional Intra-CU LTM should be supported in R19.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

LTM as introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. In Rel-19, enhancements should be specified so that the system can benefit from both the high robustness and short interruption. It’s benefical to combine LTM and conditional L3 mobility procedures and RAN4 needs to define requirements based on RAN2’s design.

2.1 Conditional LTM delay

From the objective, UE evaluated conditions will be specified by other groups to trigger LTM. RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need be considered. Meanwhile, the requirements are applicable to conditional cell switch to change the NR PCell to another NR cell. The relevant requirements can refer to the conditional handover of L3 in section 6.1.4.

6.1.4.2 NR FR1 – NR FR1 conditional handover The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell. ##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover Tmeasure is the measurements time stated in clause 6.1.4.2.2. TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3. Tinterrupt is the interruption time stated in clause 6.1.4.2.4.

Proposal 1: for conditional LTM delay, CHO requirements framework can be used as baseline.

In last meeting, RAN2 has reached consensus that source cell sends the conditional LTM configuration via RRCReconfiguration to UE. In legacy, the starting point of conditional handover delay is the end of the last TTI containing the RRC command. As to the starting point of conditional LTM delay, we prefer to reuse the definition in conditional handover. That is, the starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

- Conditional LTM Preparation phase - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - Conditional LTM Early sync phase - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. Conditional LTM Evaluation and execution phase The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Proposal 2: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

As to the ending point of conditional LTM delay, RAN2 has the agreement that the LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM. Similarly, the same principle can also be applied to RAN4.

Meanwhile, RAN2 also agreed both RACH-less Conditional intra-CU LTM and RACH based conditional intra-CU LTM were supported. Therefore, we should define ending point for both RACH-based and RACH-less, and defined for Rel-18 intra-CU LTM can be reused for conditional LTM.

Observation 1: Both RACH-less Conditional intra-CU LTM and RACH based conditional intra-CU LTM are supported in RAN2.

Proposal 3: The ending point of Rel-18 LTM can be reused for conditional LTM delay.

- For RACH-based conditiona LTM, the ending point is the first available PRACH occasion in the new cell.

- For RACH-less conditiona LTM, the ending point is the first uplink transmission on the target cell.

For the componets of conditional LTM delay, RAN2 has agreed early candidate TCI State activation/deactivation is supported. Therefore, early TCI state activation delay should be considered. We prefer to use option 1 as baseline.

Issue 1-2-4: The components of conditional LTM delay < Way Forward > FFS the following options: - Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt. (CTC) - Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. (CMCC) - Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. (Apple) - Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. (Nokia) - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design. (Nokia) - Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay. (CATT)

Proposal 4: use DCLTM = TRRC + TEvent_DU +Tmeasure + TCLTM_execution + TLTM-interrupt. as baseline and consider whether/how to capture the time for active the TCI state of candidate cell(s).

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes LTM cell switch to a target cell and interruption time starts.

TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch.

2.2 Early synchronization and TCI state activation

Issue 1-3-1: Early DL sync < Way Forward > FFS the following options: - Option 1: R18 LTM early DL-sync can be as baseline. (CTC) - Option 2: FFS whether new requirements due to CLTM specific DL/UL sync procedures are needed. (Nokia) - Option 3: Wait for RAN2 further agreement on the framework. (E///) Issue 1-3-2: Early UL sync < Way Forward > FFS the following options: - Option 1: For UL-sync, we can consider some situations as follows,(CTC) - Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0. - RAR-based TA acquisition. - UE based TA measurement. - Option 2: How/whether to support early RACH on candidate cells during conditional LTM. (HW) - Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements. (Nokia) - Option 4: If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation. (QC) - Option 5: Wait for more progress from other WGs. (HW, E///) Issue 1-3-3: Early TCI state activation < Way Forward > FFS the following options: - Option 1: How/whether to support early TCI activation on candidate cells during conditional LTM. (HW) - Option 2: FFS whether updates are needed to the Rel-18 early TCI state activation delay requirements. (Nokia) - Option 3: R18 LTM early TCI state activation can be as baseline. (CTC) - Option 4: Wait for more progress from other WGs. (HW, Ericsson)

In last meeting, RAN2 agreed to support early sync for conditional LTM, including DL sync and UL sync. Meanwhile, for UL sync, UE based TA measurement mechanism and PDCCH orderd early TA acquisition are supported. Consequently, RAN4 should consider and define related requirements for early sync. In our understanding, the requirements for R18 LTM can be used as baseline and further discussion is needed on whether/how to update for conditional LTM.

Proposal 5: Early sync is supported for conditional LTM and the related requirements defined for LTM can be used as baseline, e.g., PDCCH ordered RACH delay requirements and early TCI state activation delay requirements.

Conclusions

In this contribution, we put forward the following proposals on conditional LTM.

Proposal 1: for conditional LTM delay, CHO requirements framework can be used as baseline.

Proposal 2: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

Observation 1: Both RACH-less Conditional intra-CU LTM and RACH based conditional intra-CU LTM are supported in RAN2.

Proposal 3: The ending point of Rel-18 LTM can be reused for conditional LTM delay.

- For RACH-based conditiona LTM, the ending point is the first available PRACH occasion in the new cell.

- For RACH-less conditiona LTM, the ending point is the first uplink transmission on the target cell.

Proposal 4: use DCLTM = TRRC + TEvent_DU +Tmeasure + TCLTM_execution + TLTM-interrupt. as baseline and consider whether/how to capture the time for active the TCI state of candidate cell(s).

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes LTM cell switch to a target cell and interruption time starts.

TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch.

Proposal 5: Early sync is supported for conditional LTM and the related requirements defined for LTM can be used as baseline, e.g., PDCCH ordered RACH delay requirements and early TCI state activation delay requirements.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom