3GPP TSG-RAN WG4 Meeting # 113                                R4-2418508

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.3

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on the fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

Introduction

In the RAN #113 meeting, the WID on several enhancements for NR and MR-DC RRM requirements was approval. There are two objectives need to discuss in Rel-19, one is FR2-1 SSB based L3 measurement delay reduction for connected mode, and the other is FR2-1 SSB based L3 measurement delay reduction for connected mode. In this meeting, we need to study the latter according to schedule [1].

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this contribution, we would like to provide our views on fast SCell activation for UE supporting Rel-18 EMR.

Discussion

According the work item, we want to consider valid early measurement report in the existing known/unknown SCell condition during SCell configuration/activation in Release 19, which could reduce delay requirement for the target SCell activation. In Rel-18 mobility, RAN4 has discussed how to reuse the IDLE/INACTIVE mode measurement results which to be reported during RRC connection setup/resume in order to improve SCell/SCG setup delay. We think it’s feasible and beneficial to consider fast SCell activation for UE supporting Rel-18 EMR.

In the last meeting, we have discussed and reached consensus on some issues. First, we will not change the definition of R18 eEMR in the fast SCell activation discussion. The fast SCell activation delay requirements are defined only for the case when the UE supports Rel-18 eEMR and the UE has reported valid results on the SCell to be activated before SCell activation command. We can observe the specific discussion below[2].

##### Issue 1-1-1: Applicability of fast SCell activation delay requirements Proposals: - Option 1: (vivo, CATT, CMCC, LGE, Apple, ZTE, MTK) - RAN4 not to define the fast SCell activation delay requirements for the case when the durations for validity check (i.e., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18) are not configured. - Option 2: (OPPO, Nokia, Samsung, Ericsson) - RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR. - Option 2a: (OPPO) - Consider the following two approaches to support the case: - 1) a new indication of valid measurement results of idle/inactive mode can be introduced, or - 2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration. - Option 2b: (Samsung) - RAN4 not to include the case when measIdleValidityDuration-r18 is not configured - RAN4 to include the case when measReselectionValidityDuration-r18 is not configured ##### Issue 1-2-4: Whether the indication to network is needed Proposals - Option 1: (CATT, Apple, Samsung) - Valid eEMR report in R18 is enough. No need to introduce additional indication to the network for fast or slow SCell activation. - Option 2: (Ericsson, Nokia) - RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

In R18 EMR, we discussed the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured. In this case, UE is not required to perform validity check and report valid measurement results, which the UE behavior is same as R16 EMR. According to WID, fast SCell activation for UE supporting Rel-18 EMR is mentioned. In our understanding, maybe there has benefit to support the case that UE has not been configured with measReselectionValidityDuration-r18, we should study the fast SCell activation based on Rel-18 EMR other than Rel-16, and the different between them just depend on whether validity check is required.

Observation 1: UE is not required to perform validity check and report valid measurement results when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured.

We try to understand the intention of proponents for option2. Option2 isn’t limited to valid EMR and maybe other indication that whether the EMR is valid for SCell activation or not is necessary. Meanwhile, Option1 aims to use valid R18 EMR for SCell activation delay, so it’s necessary to ensure that validity check has been excuted. Therefore, it’s two approaches to support R18 EMR in SCell activation procedure as mentioned in option2b.

Consider the following two approaches to support the case:

1) a new indication of valid measurement results of idle/inactive mode can be introduced, or

2) reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

For the first bullet, a UE indication together with the EMR report on whether UE can do the fast or legacy Scell activation and validity check on R18 eEMR is not needed in this case. Consequently, it can be used when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured. For the second bullet, there is no indication and valid eEMR report in R18 is needed.

We should consider whether valid EMR in Rel 18 can be used for R19 fast SCell activation directly. In our commom understanding, existing validity conditions for Rel-18 maybe not sufficient for known SCell activation due to long validity duration. In Rel 18, EMR is recognized as valid if measurement excuted in validity duration, which is indicated by NW and the candidate values are {5s,10s,20s,50s,100s}. In legacy, SCell is recognized as known within the known window. If we can’t reach consensus on how to ensure the fresh of EMR, UE indication can be considered.

Proposal 1: for the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured,

- if UE indication is introduced together with the EMR report on whether UE can do the fast or legacy Scell activation, validity check in R18 eEMR is not needed.

- if UE indication is not introduced together with the EMR report on whether UE can do the fast or legacy Scell activation, validity check in R18 eEMR is needed.

- RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception. - For FR1, - [Y] equals to [5s] for direct SCell activation - FFS: Y for other SCell activation case. - For FR2, - Reuse the existing value of Y in direct SCell activation - FFS: Y for other SCell activation case.

In current Spec, SCell activation of deactivated SCell, direct SCell activation, fast SCell activation and SCell activation of multiple SCell are supported. As to the scope of fast SCell activation for UE supporting Rel-18 eEMR, direct SCell Activation at SCell addition is more typical, and we have reached consensus that MAC CE based Single SCell Activation is also supported. In current spec., known condition window can be observed blew. .

Table.1 typical SCell scenario and corresponding the definition of ‘known condition window’

|  | SCell activation scenario | FR1 | FR2 |
| --- | --- | --- | --- |
| Scenario 1 | SCell Activation Delay Requirement for Deactivated SCell | During the period equal to max (5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: | During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable) |
| Scenario 2 | Direct SCell Activation at SCell addition | During the last 5 seconds before the reception of the direct SCell configuration command | same as scenario 1 in FR2 |
| Scenario 3 | SCell Activation Delay Requirement for Deactivated PUCCH SCell | same as scenario 1 in FR1 | same as scenario 1 in FR2 |

For SCell activation delay requirement for deactivated SCell, it should consider the time gap between configuration and SCell activation command compared to direct SCell Activation. In our understaning, it’s fine to consider SCell activation delay requirement for deactivated SCell and the requirements of direct SCell activation can be used as baseline. There is no doubt for scenario 2 and scenario 3 in FR2 due to we agreed to reuse the existing value of Y in direct SCell activation, which is similar. As to FR1, we prefer to use the same statement as direct SCell activation.

Proposal 2:RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception.

- For FR1, [Y] equals to [5s] for Single direct SCell activation and MAC CE based Single SCell Activation

- For FR2, Reuse the existing value of Y in direct SCell activation and MAC CE based Single SCell Activation

For continued discussion in next meeting: - For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured: - For FR1: - use existing SSB based known SCell activation delay requirement with [2] SSB samples. - For FR2: - Option1: The fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply provided that measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X] - Option 1 delay components: 1 SSB sample as legacy known case (Apple with short window, OPPO with X=5, MTK, HW) - Option 2: (Ericsson, Nokia) No need to have such side condition in option 1. Need more discussion. - Option 3: use existing SSB based known SCell activation delay requirement with 2 SSB samples and don’t need side condition in option 1 - Option 3 delay components: [2] SSB samples * beam sweeping +1 SSB sample (QC, Apple, OPPO, ZTE, Samsung, VIVO, MTK when the validityduration is configured) - Option 4 (combine option 1 and option 2) delay components:[ K] SSB samples * beam sweeping +1 SSB sample (QC, ZTE, Nokia, HW, CATT) - K = 0 or 1 if when the “validityduration” is smaller than X - K = 1 or 2 otherwise - Option 4a: delay components:[ K] SSB samples * beam sweeping +1 SSB sample (Samsung) - K = 0 or 1 if when the “validityduration” is applied with smaller than X - K = 1 or 2 otherwise - FFS how to test

For single direct SCell activation at SCell addition with R18 valid eEMR, we have discussed SCell activation delay requirements last meeting and it seems there is no doubt about the related requirements for FR1.

As to FR2, there are four options.

For Option 1, if measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured smaller than [X], the fast SCell activation delay requirements with Rel-18 valid eEMR reporting apply. In our understanding, this approach ensures that the measurement results are fresh. R18 eEMR can only be used for SCell activation when the validity duration is less than X, and SCell activation that meets this condition will be very fast.

For Option 2, it is not required that the validity duration be less than X, which means that all results identified as valid in R18 eEMR can be used for fast SCell activation. This essence is certainly good, but we need to consider whether these results are still valid for SCell activation. The majority of companies remain skeptical about these results.

For Option 3, we consider it as a compromise, which has the advantage over option 1 in that it takes into account all valid EMRs rather than focusing solely on cases with small validity duration. However, in return, the SCell activation delay is somewhat longer, offering only a slight improvement compared to the legacy system.

For Option 4, we consider it as an optimization of Option 3. While taking into account all valid eEMR, it splits the SCell activation requirement definition into two parts. If the validity duration is less than X, a very fast activation (i.e., Option 1) is employed; otherwise, a faster activation (i.e., Option 3) is used.

Our viewpoint is that we need to fully utilize R18 eEMR and hope to apply all valid eEMR to the SCell activation process, rather than only being applicable when the configuration is less than a certain value. So, we support option 3 and option 4, and compared to these two, option 4 brings more gain but it will make the implementation and protocol more complex.

Proposal 3: For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples.

  - For FR2:

    - delay components:[ 2 ] SSB samples * beam sweeping +1 SSB sample

Conclusions

In this paper, we have some discussion on fast SCell activation for UE supporting Rel-18 EMR. And we have the following proposals:

Observation 1: UE is not required to perform validity check and report valid measurement results when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured.

proposal 1: for the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured,

- if UE indication is introduced together with the EMR report on whether UE can do the fast or legacy Scell activation, validity check in R18 eEMR is not needed.

- if UE indication is not introduced together with the EMR report on whether UE can do the fast or legacy Scell activation, validity check in R18 eEMR is needed.

Proposal 2:RAN4 to update known condition that the UE has sent a R18 EMR report according to the reporting requirements in 4.7.3 or 5.8.3 and the R18 EMR report is sent within [Y] seconds before SCell activation command reception.

- For FR1, [Y] equals to [5s] for Single direct SCell activation and MAC CE based Single SCell Activation

- For FR2, Reuse the existing value of Y in direct SCell activation and MAC CE based Single SCell Activation

Proposal 3: For single direct SCell activation at SCell addition, provided measIdleValidityDuration-r18/measReselectionValidityDuration-r18 is configured

  - For FR1:

    - use existing SSB based known SCell activation delay requirement with [2] SSB samples.

  - For FR2:

    - delay components:[ 2 ] SSB samples * beam sweeping +1 SSB sample

References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT