# 3GPP TSG-RAN WG4 Meeting #113	R4-2418540

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	4.2.1

Source:	Apple

Title:	On gradual timing adjustment for CA and DC

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In Rel-15 gradual timing adjustment requirements were introduced.  In this contribution we propose update to the gradual timing requirements in CA and DC.

# 2. Discussion

The gradual timing adjustment requirement in §7.1.2.1 is defined as:

The gradual timing adjust requirements are applicable for CA and DC. The UE applied gradual timing adjustment until the transmission timing error between UE and the reference timing is within ±Te for each TAG in DC and CA case. The gradual timing adjust is applied until the timing error with respect to reference timing is within ±Te for each TAG. UE is also expected to meet MTTD requirements between pairs of TAGs or PCell and PScell depending on the configuration. It is possible that with gradual timing adjustment applied on each TAG independently the TTD between the pair of TAGs exceeds MTTD requirement and could impact the UL transmission in certain scenarios.

While the UE is applying gradual timing adjustment, the TTD for the pair of TAGs or between PCell-PSCell could exceed the MTTD requirement

In certain cases there might be an impact to UL transmission if the MTTD is exceeded.

The MTTD requirements are defined for DC and CA for the following:

Inter-band EN-DC: between E-UTRA PCell and PSCell

Async – mandatory, except in FDD-FDD with overlapping spectrum with LTE with capability

Sync - mandatory

Intra-band EN-DC: between E-UTRA PCell and PSCell

Async – optional with capability

NR CA: between pairs of TAGs configured in SA and DC modes

Inter-band NE-DC: between PCell and E-UTRA PSCell

Async FDD-FDD with UE capability

Inter-band NR-DC: between PCell and PSCell

Sync and Async based on UE capability

For EN-DC, NE-DC, NR-DC if asynchronous operation is supported, if the TTD exceeds MTTD might not be an issue. But if asynchronous operation is not supported, then there will be an impact to UL transmission if TTD exceeds MTTD.

In case of NR CA if MTTD is exceeded, there would be an impact to UL transmission

For NR-DC, EN-DC and NE-DC if asynchronous mode is supported there might not be issue with UL transmission if TTD exceeds MTTD.

For NR-DC, EN-DC and NE-DC if asynchronous mode is not supported there would be an impact to UL transmission if TTD exceeds MTTD.

For NR CA if TTD exceed MTTD, there would be an impact to UL transmission.

In the cases above listed above where there would be an impact to UL transmission if TTD exceeds MTTD, we propose that the gradual timing adjust should stop if the transmit timing difference would exceed MTTD with the adjustment.

- The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation
       Inter-band NR-DC if UE doesn’t support asynchronous operation

Our proposal is captured in our companion CR [1].

A similar issue is brought up for gradual timing adjustment in case of multi-DCI mTRP transmission with 2 TA in [2].

# 3. Conclusion

In this paper, we propose an update to the gradual timing requirements in CA and DC. Our observations and proposals are captured below:

- While the UE is applying gradual timing adjustment, the TTD for the pair of TAGs or between PCell-PSCell could exceed the MTTD requirement

- In certain cases there might be an impact to UL transmission if the MTTD is exceeded.

For NR-DC, EN-DC and NE-DC if asynchronous mode is supported there might not be issue with UL transmission if TTD exceeds MTTD.

For NR-DC, EN-DC and NE-DC if asynchronous mode is not supported there would be an impact to UL transmission if TTD exceeds MTTD.

For NR CA if TTD exceed MTTD, there would be an impact to UL transmission.

- The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support       asynchronous operation
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation
       Inter-band NR-DC if UE doesn’t support asynchronous operation

# Reference

- R4-2418541, “Update to gradual timing adjustment R15”, Apple.

- R4-2418548, “On gradual timing adjustment for 2TA”, Apple.