3GPP TSG-RAN WG4 Meeting #113		R4-2418550

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.14.2

Source:	Apple

Title:	On Spatial Channel Modeling for Demodulation Requirements

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#112bis spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution we share our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4.

# 2. Discussion

## General Aspects

In RAN4#112bis the following agreements were made:

Cases for SU-MIMO Agreement: Single-User PDSCH: 8Rx: 8 Layer, MCS 13 on both codewords (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting (FFS Whether it is already covered in PMI) 4Rx: 4 Layer, MCS 13 (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer Single-User PMI 4Rx: 4 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 4 Layer (eType II) – Full Throughput Curves (PMI Follow) 4Rx: 2 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 2 Layer (eType II) – Full Throughput Curves (PMI Follow) Interested companies can use initially 8 CSI-RS Ports, more ports can be used to identify and show relevant impacts. Note: Several companies would prefer to focus on PDSCH until the modelling is stable Cases for MU-MIMO Agreement: Interested companies to bring to RAN4#113 views on the following cases: PDSCH 2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE) Interested companies are encouraged to assess 2+2 layers with R-ML / E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE) Proponents to highlight the difference between a SCM and TDL for MU-MIMO.

For SU-MIMO test cases the open issue is whether to use fixed or random precoder. Companies were encouraged to bring proposals for the fixed precoder if feasible. The PMI is determined by the channel, and unless we have a static channel it would be very difficult to fix the PMI. The best PMI would depend on the channel realization in our understanding.

- The best PMI would depend on the channel realization and is feasible to determine only in static channel without running 2 sets of simulations.

- Finding a fixed precoder for CDL or TDL channel to align across all companies is not practical.

RAN4 employ random precoder for PDSCH TPut test cases for alignment and comparison study of different channel modeling methodologies.

- UE processing is tested sufficiently for cancelling inter-user interference in MU-MIMO scenarios with TDL channel model.

Further justification is needed for considering MU-MIMO for the study.

In case MU-MIMO is agreed the test setup is limited with what RAN4 has used so far – single UE with random PMI for target UE.

In case MU-MIMO is agreed the test setup shall not include PMI feedback and/or multiple UEs.

If determined as necessary and feasible to introduce spatial channel model, RAN4 shall not define any new requirements with it in Rel-19 – for either Rel-19 WIs or earlier WIs.

Hence, we propose to use random PMI for PDSCH throughput evaluation.

For MU-MIMO RAN4 has defined requirements in Rel-17, Rel-18 with TDL channel model. The purpose of introducing requirements with MU-MIMO is to determine if the UE can reject or suppress the inter user interference. Employing TDL channels doesn’t limit testing of UE processing to cancel inter-user interference in case of MU-MIMO. Further justification is needed for studying impact of MU-MIMO performance with spatial channel model.

In case MU-MIMO test cases are agreed for the study, the test setup should be what RAN4 has used so far – with random PMI for target UE, orthogonal/random PMI for co-scheduled UEs. Scenarios with multiple UEs, PMI feedback should be out of scope. There is no use is the test setup in the study cannot be used for the actual test setup when requirements are defined.

If found necessary and feasible to introduce a spatial channel model for requirements in RAN4, we propose that RAN4 doesn’t define demodulation or CSI reporting requirements with it in Rel-19 for ongoing Rel-19 WIs or for features/ requirements introduced in previous WIs. This is a study and not a WI.

## TDL based methodologies

In 38.901 spatial filtering for TDL channel is described. This is an option that could be used to include directional antennas in TDL channel model which otherwise assumes omni directional antennas. With CDL channel model, some antenna pattern typically directional is used. With the same assumptions and simplification to generate TDL channel model, we can use a directional antenna pattern and obtain spatially filtered TDL channel models.

RAN4 to also consider spatially filtered TDL channel models in the study.

RAN4 to also consider spatially filtered TDL channel models described in 38.901 in the study.

In RAN4#112bis companies submitted proposals for TDL based spatial channel modeling methodologies in [2], [3].

For the proposal in [2] with multi-cluster TX and RX beam steering, can model the effect of multiple clusters with dual cluster beam steering. In our opinion it might still lack spatial properties such as AoA/AoD for the rays and possibly cannot reflect the situation where beam direction doesn’t match the cluster direction. This channel model might not capture the physical aspects of the environment we aim to emulate with a spatial channel model.

- Multi-cluster TX RX beam steering over TDL might be limited in modeling the spatial properties as observed.

The proposal in [3] for TDL based modeling uses the suggested approach in TS 38.901 for spatially filtered TDL to generate per-tap correlation matrices. This could be a more feasible approach to to extend TDL channel model.   As suggested different cASA/cASD values can be used for PDSCH T-Put and PMI test cases in the study.

- Introducing per-tap correlation for TDL channel could be a feasible approach for TDL based methodology.

## CDL based methodologies

Radiation Pattern

Proposals:

- Option 1: For the purposes of CDL alignment, disregard radiation pattern parameters included in TR 38.827 Section 7.2 and consider Omnidirectional antenna pattern

- Option 2: The antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1

For the radiation pattern option 2 is more practical and relevant to this study item. With omni directional antenna assumption for BS and CDL channel might be like TDL channel model.

- Assuming directional antenna for BS might be practical and more relevant to this study.

Assume directional antenna with radiation pattern in 38.901 for CDL channel.

Frequency Bands

Proposals:

- Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range

The current models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions on RMS delay spread, UE velocity, etc. RAN4 defines band agnostic demodulation and CSI reporting requirements.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

It would be very important to study if the spatial channel model can be defined for all applicable carrier frequencies/ bands for each FR.

RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

Subcluster Correlation

Proposals:

- Option 1: RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it.

In TS38.901 for CDL channel generation, the following is captured:

For the UMa-CDL-C channel generation the above ray to cluster mapping should be used to avoid correlation between sub-clusters.

Use the ray to sub-cluster mapping in Table 7.5-5 of TS38.901 in UMa-CDL-C channel generation to avoid potential correlation between sub-clusters.

Angles of Departure and Angles of Arrival

Proposals:

- Option 1: Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as  .

We support the option to update mean angle in the last position of departure and arrival angles generated equation by the desired angle. The desired angle μϕ,desired can be set based on the line-of-sight direction of TX to RX. This ensures that the mean ray angles are pointing in the LOS direction.

Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as .

Set μϕ,desired based on the LOS direction between the TX and RX.

Antenna Coordinates, Angles, and other parameters for gNB (for UMa-CDL-C)

Proposals:

- Option 1: For the gNB, use the following parameters for alignment with UMa CDL-C model:

  - GCS Location Coordinates for the gNB:

  - Height = 25m (based on UMa);

  - Azimuth = 0; (placement on the x axis);

  - GCS to LCS Conversion angles  (α,β,γ = bearing, downtilt, slant) for the gNB;

  - α,β,γ=(0,0,0);

- Option 2:

  - Frequency Fc = 3.5 GHz, height BS = 25m, and D2D = 100m.

  - BS orientation: (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). Intention is that BS points towards horizon along x-axis.

- Other options not precluded

Antenna Coordinates, Angles, and other parameters for UE (for UMa-CDL-C)

Proposals:

- Option 1: For the UE, use the following parameters for alignment with UMa CDL-C model:

  - Height = 25m (same as gNB)

  - Azimuth = 0; (placement on the x axis);

  - α,β,γ=(90,0,45);

- Option 2:

  - Frequency Fc = 3.5 GHz, height UE = 1.5m and D2D = 100m.

  - UE orientation: (alpha = -90 degrees, beta = -13.2 degrees, gamma = 45 degrees). Intention is that UE points towards BS and uses "[+ + ... +]" crosspol elements.

- Other options not precluded.

For the purpose of alignment, we propose to assume that the antenna panels are in the Y-Z plane. Other parameters are proposed below:

Height of BS: 25m

LCS to GCS for BS: α,β,γ=(0,10,0);

Height of UE: 1.5 m

LCS to GCS for UE: α,β,γ=(180,0,0);

BS and UE panels are in the Y-Z plane with the following parameters:
Height of BS: 25m
LCS to GCS for BS: α,β,γ=(0,10,0);
Height of UE: 1.5 m
LCS to GCS for UE: α,β,γ=(180,0,0);

BS Antenna Virtualiser choice

Proposals:

- Option 1: Down Tilt Virtualiser : -10 degrees

- Option 2: Broadside

- Option 3: Max Receive Power

- Option 4: DFT Virtualiser according to TR 36.897 with application of panning and tilting angles (as per Table 7.8-1 of 38.901)

On the BS virtualizer we have the following views:

Option 2: Broadside direction. This is only appliable to AAV with sub-array configuration. This is not preferred as there could be an issue to obtain > 2 layers if the sub-arrays are pointing in the same direction.

Option 3: Max Receive Power. This is the method from TS38.827 direction is the most straight forward and convenient to adopt. This is applicable to both AAV without sub-array and with sub-array configuration. It is easy to specify this for different TX port configurations based on the channel model.

Option 4: DFT Virtualiser. This option is not favored for this study as we would need to have alignment on how beams are chosen. Employing different methods for beam selection may lead to diverse results.

For BS antenna configurations with AAV we propose to use the max receive power for BS antenna virtualizer.

For BS antenna configurations with AAV use max receive power based BS antenna virtualizer.

In [1] there were several candidate SCM options for comparison and evaluation. For some of the options with AAV -without sub-array and without AAV the notation was not finalized. The notation for with AAV and sub-array is:

(M,N,P,Ms,Ns)  ; Where

M is the number of antenna elements with the same polarization in each column (vertical size)

N is the number of antenna elements with the same polarization in each row (horizontal size)

P is the number of polarizations

Ms is the number of Subarray Elements with the same polarization in each column (vertical size)

Ns is the number of Subarray Elements with the same polarization in each row (horizontal size)

When there is no AAV and the TX ports = the antenna elements, we don’t need a 5-argument notation. The 5 argument notation can be used to denote AAV and 3-argument notation could be used for without AAV.

- When there is no AAV and #TX ports= BS antenna elements, we don’t need 5-argument notation. 3-arg notation (M,N,P) is sufficient.

- The 5-argument notation can be used for AAV without sub-array.

Use 3-argument notation of (M,N,P) for no AAV configuration.

Use 5-argument notation of (M,N,P,Ms,Ns)  for AAV with no sub-array config where Ms=M, Ns=N.

In Rel-15 we defined simplified TDL channel models with maximum of 12 taps by taking the channel model in 38.901 and combining close by paths and having a resolution of 5ns. The CDL channel models in 38.901 and 38.827 are defined for a large number of clusters – 23/24 for NLOS cases. RAN4 needs to further study if they models can be simplified by taking a similar approach as described in 38.101-4 Annex B.2.1.

RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

## Comparison of methodologies

So far we have agreed on some initial test cases for SU-MIMO, but we have not discussed the metric we would employ to compare the methodologies to rationalize the need for a special channel model for demodulation requirements.

One of the objectives of the SI is to identify the limitations of current channel models and how MIMO performance is impacted:

We need to further discuss the metric used to compare methodologies that help identify limitations in the current methodology that impacts UE/BS demod/ MIMO performance. Without a concrete idea on how to this is done, it would be difficult to move ahead with the agreed comparison.

RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts MIMO performance.

# 3. Conclusion

In this paper, we provide our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4. Our observations and proposals are captured below:

General aspects

- The best PMI would depend on the channel realization and is feasible to determine only in static channel without running 2 sets of simulations.

- Finding a fixed precoder for CDL or TDL channel to align across all companies is not practical.

- RAN4 employ random precoder for PDSCH TPut test cases for alignment and comparison study of different channel modeling methodologies.

- UE processing is tested sufficiently for cancelling inter-user interference in MU-MIMO scenarios with TDL channel model.

Further justification is needed for considering MU-MIMO for the study.

In case MU-MIMO is agreed the test setup is limited with what RAN4 has used so far – single UE with random PMI for target UE.

In case MU-MIMO is agreed the test setup shall not include PMI feedback and/or multiple UEs.

If determined as necessary and feasible to introduce spatial channel model, RAN4 shall not define any new requirements with it in Rel-19 – for either Rel-19 WIs or earlier WIs.

TDL based methodologies

RAN4 to also consider spatially filtered TDL channel models described in 38.901 in the study.

- Multi-cluster TX RX beam steering over TDL might be limited in modeling the spatial properties as observed.

- Introducing per-tap correlation for TDL channel could be a feasible approach for TDL based methodology.

CDL based methodologies

- Assuming directional antenna for BS might be practical and more relevant to this study.

Assume directional antenna with radiation pattern in 38.901 for CDL channel.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

Use the ray to sub-cluster mapping in Table 7.5-5 of TS38.901 in UMa-CDL-C channel generation to avoid potential correlation between sub-clusters.

Replace the mean angle   in the last position of departure and arrival angles generated equation by the desired angle as .

Set μϕ,desired based on the LOS direction between the TX and RX.

BS and UE panels are in the Y-Z plane with the following parameters:
Height of BS: 25m
LCS to GCS for BS: α,β,γ=(0,10,0);
Height of UE: 1.5 m
LCS to GCS for UE: α,β,γ=(180,0,0);

For BS antenna configurations with AAV use max receive power based BS antenna virtualizer.

- When there is no AAV and #TX ports= BS antenna elements, we don’t need 5-argument notation. 3-arg notation (M,N,P) is sufficient.

- The 5-argument notation can be used for AAV without sub-array.

Use 3-argument notation of (M,N,P) for no AAV configuration.

Use 5-argument notation of (M,N,P,Ms,Ns)  for AAV with no sub-array config where Ms=M, Ns=N.

RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

Comparison of methodologies

RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts MIMO performance.

# Reference

- R4-2413606, “Way Forward for [112][327] FS_NR_demod_SCM”, Nokia.

- R4-2415382, “Discussion on spatial channel modelling”, MediaTek.

- R4-2415636, “Discussion on spatial channel modelling methodology”, Huawei, HiSilicon.