3GPP TSG-RAN WG4 Meeting #113                              						         R4-2418577

# Orlando, US, 18th – 22th November, 2024

Source: 	Apple

Title: 	On signalling and UE behaviour for type 4 UE

Agenda Item:	7.5.2.2

Document for:	Discussion

# Introduction

Type 4 UE capability and new BS signaling have been discussed in the past RAN4 meetings and the latest agreements were captured in the WF [1]. The following issues remain open for further study.

- < Issue 2-3: BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated) >

- < Issue 2-4: BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

- < Issue 2-6: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

This contribution will give further analysis on these open issues and presents our views.

# Discussion

## New BS signalling supporting type 4 UE switching

In Rel-18, RAN2 introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for type 2 UE for the case of maxMIMO-Layers with value less than or equal to 2. The description for nonCollocatedTypeNR-CA in TS 38.331 is reproduced as below.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

Though not mentioned in the above Rel-18 BS signalling nonCollocatedTypeNR-CA for CA and nonCollocatedTypeMRDC for EN-DC, it is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4 for type 2 UE.

Observation 1: Rel-18 BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to indicate switch between Type 1 and Type 2 is only specified for the case with maxMIMO-layers less than or equal to 2. It is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4.

When it comes to type 4 UE, a similar approach could be considered for network indication supporting UE switching between 6/8Rx type 1 and type 4a/4b.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

Proposal 1: To support type 4 UE switching between type 1 capability and type 4 capability, consider the following proposals.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

## UE behaviour between different capability set

For type 4 UE, there could be different working mode depending on different operating condition. The switch behaviour between non-collocated type 4 and other main modes is illustrated in figure 2.4-1.

Figure 2.3-1 illustration of UE behaviour and means to switch between different UE capabilities

It should be noted that,

- For non-collocated type 2, there could be further switching between collocated type 2 and collocate type 1 (4Rx). While these are already supported in Rel-18 and is omitted in the figure.

- For collocated type 1 with 4-layers/CC and 8-layers/CC, there will be further switching between them and other x-layers/CC. while it is just done by MIMO layer adaption as usual and is omitted in the figure.

Proposal 2: it is proposed that RAN4 confirm the UE behaviour for type 4 UE as illustrated in figure 2.3-1.

# Summary

This contribution presented our consideration on the open issues for type 4 UE. The following observations and proposals are concluded.

Observation 1: Rel-18 BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to indicate switch between Type 1 and Type 2 is only specified for the case with maxMIMO-layers less than or equal to 2. It is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4.

Proposal 1: To support type 4 UE switching between type 1 capability and type 4 capability, consider the following proposals.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

Proposal 2: it is proposed that RAN4 confirm the UE behaviour for type 4 UE as illustrated in figure 2.3-1.

# References

[1] R4-2417115, WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI, RAN4#112bis