3GPP TSG-RAN WG4 Meeting #113	 R4-2418585

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.22.2.1

Source: 	Apple

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

# Introduction

RAN4 requirements for OD-SSB was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1]. Besides, RAN4 also received an LS [2] from RAN1, in which RAN4 action is included. RAN4 had some preliminary discussion on the LS in the previous RAN4 meeting. However, the issue has not been concluded yet.

In this contribution, we will discuss the RAN1 LS and all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

The first issue we would like to discuss is about RAN1 LS [2]:

RAN1 has made the following agreements for MAC-CE based indication of on-demand SSB (OD-SSB) transmission on SCell. Only the relevant agreements related to RAN1’s question are excerpted. In highlights in the agreements, RAN1 agreed working assumption that the value of T_min for MAC-CE based OD-SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command. RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement. <RAN1 #116> Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for <RAN1 #116 bis> Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. <RAN1 #117> Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable <RAN1 #118> Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min= where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and is as defined in current specification. - RAN4 to confirm that T_min can be equal to - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology..

In summary, RAN1 is asking whether OD-SSB activation can be completed by n+m+3Nslotsubframe+1:

In our understanding MAC-CE processing time shall be same as legacy, e.g. it can be finished before n+m+3Nslotsubframe+1. However, from UE measurement point of view, some additional preparation time is expected since UE may need to update measurement scheme upon OD-SSB activation. As we all know, two separate searchers are assumed in RRM measurement requirements. Measurement on OD-SSB occupies one searcher. Typically once OD-SSB is activated, UE may assume network is going to activate target SCell soon (e.g. scenario 2A). UE may choose to prioritize OD-SSB measurement.

Even if UE doesn’t always prioritize OD-SSB, effectively there is one more MO to measure upon OD-SSB activation:

Observation 1: OD-SSB activation is effectively similar with that network uses MAC-CE to add one more measurement object.

Proposal 1: additional preparation time T△ is needed on top of n+m+3Nslotsubframe+1 for UE to start measurement on OD-SSB.

Proposal 2: additional preparation time T△ is [5ms].

The next issue we would like to discuss is about OD-SSB based deactivated SCell measurement in Case 1

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

- After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 2-1: fallback legacy requirement

  - Option 2-2: stop measurement

  - Option 2-3: No requirement

- Time window means the time duration starting from receiving OD-SSB activation indication

  - FFS the time window configuration or pre-defined

- FFS the fast L3 measurement requirement

- FFS the relation between time window and OD-SSB deactivation command in the time domain

- When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Based on the progress of the last meeting, we need to further discuss definition of fast measurement, slow mode and time window. The minimum sampling internal is the OD-SSB periodicity. However, if the OD-SSB periodicity is too short, it doesn’t bring too much gain in physical layer averaging. Besides, it would significantly increase UE power consumption. One solution is to follow existing non-DRX RRM measurement requirement, i.e. adding a minimum sampling interval like 40ms.

Regarding slot mode, we prefer option 2-1 (follow measCycleSCell). Option 2-2 (stop measurement) is also acceptable but we think probably UE shall measure it again before the cell becomes unknown, assuming network doesn’t deactivate OD-SSB.

As for the window design, it should be long enough to make the SCell known. One solution is to define it same as the minimum requirement for cell identification.

Proposal 3: OD-SSB based deactivated SCell measurement in case 1:

- Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

- Fast measurement:

Measurement period for OD-SSB in FR1

| DRX cycle | TOD-SSB_measurement_period_fast |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x TOD-SSB) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(TOD-SSB,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x max(TOD-SSB, DRX cycle) x CSSFintra |
| NOTE 1: TOD-SSB is the periodicity of OD-SSB. |  |

Measurement period for OD-SSB in FR2

| DRX cycle | TOD-SSB_measurement_period_fast |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x TOD-SSB)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(TOD-SSB,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x max(TOD-SSB, DRX cycle) x CSSFintra |
| NOTE 1: TOD-SSB is the periodicity of OD-SSB. |  |

- Slow measurement:

  - Option 1: fallback to legacy requirement (preferred)

  - Option 2: stop measurement until the cell is becoming unknown

- Time window:

  - Starting point: upon OD-SSB activation command reception

  - Duration: It should be long enough to make the SCell known, e.g. same as the minimum requirement for cell identification based on OD-SSB.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

- Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

- Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

- Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

- Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm)

- Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

We continue supporting option 2. For the sake of progress, we can also consider the following compromised solution.

Before activation of OD-SSB, UE measures the SCell based on always-on SSB following measCycleSCell. The measurement period depends on several aspects, e.g. DRX cycle, number of carriers to measure, length of measCycleSCell, Kp and so on. Upon OD-SSB activation, if the SCell is already known, it is unnecessary for UE to perform fast measurement since the SCell can already be activated in a fast mode (fulfill known definition). But if the cell is somehow unknown upon OD-SSB activation, we can accept to perform fast measurement similarly with case 1 as a compromised solution.

Proposal 4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2:

- Option 1: RAN4 requirement is defined based on legacy measCycleSCell (preferred)

- Option 2:

  - When OD-SSB is activated, UE determines whether to perform fast measurement or not

    - If the serving cell is known (based on RRM on always-on SSB), UE doesn’t need to perform fast measurement. Existing measurement requirements based on measCycleSCell can be applied.

    - If the serving cell unknown, UE performs measurement in a fast mode within a time window. After the time window, UE falls back to slow mode.

      - Design of fast mode, slow mode and time window can be same as that discussed in case 1.

Issue 1-1-4: L1 measurement for deactivated SCell

- Proposals

  - Option 1: Xiaomi, Samsung, China Telecom, Intel

    - RAN4 shall define both L1 and L3 measurement requirements based on OD-SSB.

  - Option 2: Qualcomm, OPPO, Nokia

    - L1 measurements are not supported for deactivated SCell

  - Option 3: Nokia

    - Wait further RAN1 clarification on the applicable scenario

- Recommended WF

  - Notes:

    - From moderator’s understanding, RAN1’s agreement is applied to the L1 measurement after receiving SCell activation command other than before SCell activation.

    - RAN4 only needs to clarify the L1 related requirement can also applied to OD-SSB in SCell.

  - Suggest to further discuss the scenarios below. The proponent companies need to clarify which of the following scenarios are supported by your proposal.

    - L1 measurement before SCell activation:

    - L1 measurement during SCell activation:

    - L1 measurement after SCell activation:

Before SCell activation the SCell is in deactivated mode. Currently L1 measurement on deactivated SCell is not supported, except for LTM. We don’t think L1 measurement for LTM based on OD-SSB is in the scope. During SCell activation, sometimes L1 measurement is needed (e.g. in FR2 unknown case). In legacy it is based on always on SSB. In NES scenario it is feasible to be based on OD-SSB. Similarly, L1 measurement based on OD-SSB after SCell activation is also feasible, RAN4 requirements are defined based on TSSB. We only need to clarify that TSSB refers to OD-SSB periodicity in this case.

Proposal 5: L1 measurement based on OD-SSB before SCell activation is not considered.

Proposal 6: L1 measurement based on OD-SSB during and after SCell activation is feasible. Existing requirement framework can be reused. Clarify that TSSB for this case is the OD-SSB periodicity.

Issue 1-1-5: CSSF

- Proposals

  - Option 1: CMCC, Mediatek

    - The legacy CSSF can be reused

  - Option 2: vivo

    - RAN4 to discuss the searcher assumption of OD-SSB.

- Recommended WF

  - Further discussion.

We don’t see the necessity to discuss searcher assumption for OD-SSB. The WI is for network energy saving. New searcher assumption would result in significant impact on UE side from measurement perspective. Therefore, legacy CSSF shall be reused.

Proposal 7: RAN4 shall not change searcher assumption in this work item.

Issue 1-1-6: OD-SSB accuracy

- Proposals

  - Option 1: Qualcomm

    - No new measurement accuracy requirements should be defined.

- Recommended WF

  - Moderator suggests the group to check whether the following proposal can be agreed.

    - No new measurement accuracy requirements should be defined.

Option 1 is straightforward. We agree with the recommended WF.

Proposal 8:  No new measurement accuracy requirements should be defined specifically for OD-SSB.

Regarding relationship between always-on SSB and OD-SSB in case 2, RAN4 approved an LS to ask RAN1&2 about the supported scenario. Here we would like to emphasize that from RAN4 point of view we see some problem to support different carrier frequencies between always-on SSB and OD-SSB. Based on existing specification, UE is allowed to cause only one interruption during SCell activation procedure (due to RF activation). Currently, scenario wherein always-on SSB and OD-SSB are on different frequencies is not precluded. We would like to point out that if this scenario is included, UE may cause additional interruption during SCell activation. Some examples:

Observation 2: using both always-on SSB and OD-SSB on different frequencies during SCell activation would result in extra interruption to other serving cells.

Proposal 9: do not consider different frequencies between always-on SSB and OD-SSB in SCell activation requirements for case 2 in RAN4.

The most typical use case in our mind is that network uses long periodicity for always-on SSB to save power. Whenever there is need for CA, network activates OD-SSB with short periodicity to facilitate SCell activation. Therefore, it is proposed to prioritize same time offset, different periodicities between always-on SSB and OD-SSB.

Proposal 10: prioritize same time offset, different periodicities between always-on SSB and OD-SSB for case 2 from RAN4 requirement point of view.

The last thing we would like to discuss is how to facilitate SCell activation in case 1. Note that typically in case 1 target cell is unknown to UE since no always-on SSB before OD-SSB activation. The whole SCell activation would be quite long for unknown target cell since UE needs to start from scratch. Note that in R18 SSB-less scenario, there is a reference cell from which UE can borrow some information like AGC and timing so that the activation latency could be reduced:

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent a "default cell" is the reference cell as defined in TS 38.133 [14].

However, referenceCell configuration only applies for inter-band SSB-less SCell. We think it is beneficial to extend the concept to cover SCell with OD-SSB operation.

Proposal 11: extend referenceCell to cover SCell with OD-SSB operation.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for OD-SSB. After discussion, the following conclusions are provided:

Observation 1: OD-SSB activation is effectively similar with that network uses MAC-CE to add one more measurement object.

Proposal 1: additional preparation time T△ is needed on top of n+m+3Nslotsubframe+1 for UE to start measurement on OD-SSB.

Proposal 2: additional preparation time T△ is [5ms].

Proposal 3: OD-SSB based deactivated SCell measurement in case 1:

- Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

- Fast measurement:

Measurement period for OD-SSB in FR1

| DRX cycle | TOD-SSB_measurement_period_fast |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x TOD-SSB) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(TOD-SSB,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x max(TOD-SSB, DRX cycle) x CSSFintra |
| NOTE 1: TOD-SSB is the periodicity of OD-SSB. |  |

Measurement period for OD-SSB in FR2

| DRX cycle | TOD-SSB_measurement_period_fast |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x TOD-SSB)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(TOD-SSB,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x max(TOD-SSB, DRX cycle) x CSSFintra |
| NOTE 1: TOD-SSB is the periodicity of OD-SSB. |  |

- Slow measurement:

  - Option 1: fallback to legacy requirement (preferred)

  - Option 2: stop measurement until the cell is becoming unknown

- Time window:

  - Starting point: upon OD-SSB activation command reception

  - Duration: It should be long enough to make the SCell known, e.g. same as the minimum requirement for cell identification based on OD-SSB.

Proposal 4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2:

- Option 1: RAN4 requirement is defined based on legacy measCycleSCell (preferred)

- Option 2:

  - When OD-SSB is activated, UE determines whether to perform fast measurement or not

    - If the serving cell is known (based on RRM on always-on SSB), UE doesn’t need to perform fast measurement. Existing measurement requirements based on measCycleSCell can be applied.

    - If the serving cell unknown, UE performs measurement in a fast mode within a time window. After the time window, UE falls back to slow mode.

      - Design of fast mode, slow mode and time window can be same as that discussed in case 1.

Proposal 5: L1 measurement based on OD-SSB before SCell activation is not considered.

Proposal 6: L1 measurement based on OD-SSB during and after SCell activation is feasible. Existing requirement framework can be reused. Clarify that TSSB for this case is the OD-SSB periodicity.

Proposal 7: RAN4 shall not change searcher assumption in this work item.

Proposal 8:  No new measurement accuracy requirements should be defined specifically for OD-SSB.

Observation 2: using both always-on SSB and OD-SSB on different frequencies during SCell activation would result in extra interruption to other serving cells.

Proposal 9: do not consider different frequencies between always-on SSB and OD-SSB in SCell activation requirements for case 2 in RAN4.

Proposal 10: prioritize same time offset, different periodicities between always-on SSB and OD-SSB for case 2 from RAN4 requirement point of view.

Proposal 11: extend referenceCell to cover SCell with OD-SSB operation.

# Reference

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R1-2407565, LS on timeline for On-demand SSB operation on SCell, Apple