3GPP TSG-RAN WG4 Meeting #113	 R4-2418590

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	8.3

Source: 	Apple

Title:	Discussion on RAN2 LS on Relaxed measurement

Document for:	Discussion

# Introduction

RAN4 received an LS [1] from RAN2 on R16 UE power saving:

1. Overall Description: RAN2 has discussed about the scenarios for the Rel-16 RRM relaxation in Idle/Inactive state during the RAN2#127bis meeting. As a conclusion, RAN2 has in-principle agreed the CR to TS38.304 [1], removing the description of conditions when measurements are relaxed and adding a reference to RAN4 specification. During the discussion, RAN2 noticed that RAN4 specification defines a behaviour for the condition “if the serving cell measurement is better than s-NonIntraSearch thresholds but the cell-edge criteria is not fulfilled”. From RAN2 point of view, the case “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criteria is not fulfilled” will not happen and cannot happen because of the following field description for s-SearchThresholdP and s-SearchThresholdQ in TS38.331. s-SearchThresholdP, s-SearchThresholdP2 Parameters "SSearchThresholdP" and "SSearchThresholdP2" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP.. s-SearchThresholdQ, s-SearchThresholdQ2 Parameters "SSearchThresholdQ" and "SSearchThresholdQ2" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ.. 2. Actions: To RAN4. ACTION: RAN2 respectfully asks RAN4 to take the above into consideration and inform that RAN4 can update their part of their specs, if they see a need.

In this contribution, we provide our view on this issue.

# Discussion

We believe in “cell-edge criteria is not fulfilled” RAN2 means cellEdgeEvaluation is not met. cellEdgeEvaluation is defined in TS38.304:

5.2.4.9.2 Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

We would like to use the following figure to show different coverages of s-NonIntraSearch and s-SearchThresholdP.

When RAN2 said “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criteria is not fulfilled” we assume RAN2 actually means UE cannot be in zone 1 and zone 3 at the same time. The maximum of SSearchThresholdP is s-IntraSearchP and s-NonIntraSearchP which means there is no zone 2 and zone 1 is adjacent to zone 3.

Indeed, in R16 when RAN4 discussed measurement relaxation RAN4 didn’t know the following restrictions:

- The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP

- The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ.

However, after we went through RAN4 requirements we found that existing RAN4 requirement actually doesn’t intentionally cover the case “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criteria is not fulfilled”. Let’s take a look at section 4.2.2.10.2:

##### 4.2.2.10.2 Measurements for UE fulfilling low mobility criterion This clause contains requirements for measurements on inter-frequency NR cells provided that: - UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or - UE is configured with both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and - UE has fulfilled only the lowMobilityEvaluation [2] criterion. The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running. When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements are defined as follows: - The UE shall be able to evaluate whether a newly detectable inter-frequency NR cell meets the reselection criteria defined in TS38.304 [1] within Ncarrier_Relax * Tdetect,NR_Inter_Relax + Ncarrier_Non_relax * Tdetect,NR_Inter. Cells which have been detected shall be measured at least every Ncarrier_Relax * Tmeasure,NR_Inter_Relax + Ncarrier_Non_relax * Tmeasure,NR_Inter. The UE shall be able to evaluate that an already identified inter-frequency NR cell has met reselection criterion defined in TS 38.304 [1] within Ncarrier_Relax *Tevaluate,NR_Inter_Relax + Ncarrier_Non_relax * Tevaluate,NR_Inter. - When T331 is running, - The parameter Ncarrier_Relax is the total number of NR inter-frequency carriers not configured for idle mode CA/DC measurements. - The parameter Ncarrier_Non_relax is the total number of NR inter-frequency carriers configured for idle mode CA/DC measurements. - When T331 is not running, - The parameter Ncarrier_Relax is the total number of inter-frequency carriers configured for mobility measurements only and the number of inter-frequency carriers configured for both mobility measurement and idle mode CA/DC measurements. - The parameter Ncarrier_Non_relax =0. - For a UE not configured with eDRX_IDLE, Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax are as specified in Table 4.2.2.10.2-1 and Tdetect,NR_Inter, Tmeasure,NR_ Inter and Tevaluate,NR_ Inter are as specified in Table 4.2.2.4.1-1. - For a UE configured with eDRX_IDLE, Tdetect,NR_Inter, Tmeasure,NR_ Inter and Tevaluate,NR_ Inter are as specified in Table 4.2.2.4-3 and Table 4.2.2.4-4 for FR1 and FR2 respectively. - For a UE configured with eDRX_IDLE cycle up to 10.24s, Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax are as specified in Table 4.2.2.10.2-2 and Table 4.2.2.10.2-3 for FR1 and FR2, respectively. - For a UE configured with eDRX_IDLE cycle greater than 10.24s, Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax are as specified in Table 4.2.2.10.2-4 and Table 4.2.2.10.2-5 for FR1 and FR2, respectively, provided eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW length. When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Section 4.2.2.10.2 cover case 1 and case 2. We assume the issue RAN2 has is that when case 2 happens, this will not and cannot happen. Technically we agree with that. However, in our understanding this is just the conditions for UE to apply requirements based on highPriorityMeasRelax. In case 2, conditions of “Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ” cannot be met thus UE doesn’t need to apply the requirements in the rest of the sentence. We have no problem to keep it since we still have case 1. This could happen in case 1.

Observation 1: seems existing RAN4 requirements have no problem.

If companies still want to make it clearer, we can consider the following update in section 4.2.2.10.2 and 4.2.2.11.2:

If UE is only configured with lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion is not configured, and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - if the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. - Otherwise, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Proposal 1: If companies still want to make it clearer, we can consider the following update in section 4.2.2.10.2 and 4.2.2.11.2:

If UE is only configured with lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion is not configured, and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - If the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. - Otherwise, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

We also provide a draft LS back to RAN2 in Annex.

# Conclusion

In this contribution, we provide discussion on RAN2 LS on relaxed measurement. After discussion, the following conclusions are provided:

Observation 1: seems existing RAN4 requirements have no problem.

Proposal 1: If companies still want to make it clearer, we can consider the following update in section 4.2.2.10.2:

If UE is only configured with lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion is not configured, and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - If the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. - Otherwise, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

# Reference

- R4-2417507, LS on Relaxed measurement, RAN2

# Annex

3GPP TSG-RAN WG4 Meeting #113						     R4-24xxxxx

Orlando, USA, 18th – 22nd November, 2024

Title:	LS to RAN2 on relaxed measurement

Response to:	R2-2409308

Release:	Release 16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Qiming Li

E-mail Address:	< li_qiming@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 thanks RAN2 for the LS on relaxed measurement. After discussion in RAN4#113, the following update was agreed in TS38.133 section 4.2.2.10.2:

If UE is only configured with lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion is not configured, and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - If the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. - Otherwise, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

With the above update, the scenario “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criteria is not fulfilled” is excluded in RAN4 requirements.

2. Actions:

To RAN2.

ACTION: 	RAN4 kindly asks RAN2 to take above information into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114		              17 - 21 February 2025 			Athens, Greece

TSG RAN WG4 Meeting #114bis		              7 - 1 April 2025				China