3GPP TSG-RAN4 Meeting #113	R4-2418605

# Orlando, US, 18th – 22nd November, 2024

Title:	Views on LP-WUS BS RF requirements

Source:	Qualcomm Incorporated

Agenda item:	7.23.3

Document for:	Approval

# 1.	Introduction

In RAN4#112bis, discussions continued on BS RF requirements for LP-WUS with some progress, and a WF was agreed in [1] which entailed several agreements and current open issues. In this contribution, we share our view on several of those issues and shed light on development direction considering plenary confirmation about scope of WI and FR2

# 2 	Discussion

## 2.1	Clarification on FR2 support

IN RAN4#111, the way forward [3] entailed the following regarding support of FR2 for LP-WUS feature:

## Whether to consider FR2 for LP-WUS WF - RAN4 focus on FR1 licensed bands for BS RF requirements. - FR2 is not precluded for further study.

In addition, below clarification in the RAN#105 [2] confirms that FR2 is in scope of the normative work in Rel-19

- Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Accordingly, it is important to ensure that the progress on BS RF requirements for LP-WUS is not excluding FR2 support. In terms of additional work to address FR2 work in the normative phase, the ongoing discussions regarding BS RF requirements captured in [1], are applicable to both FR1 and FR2. Thus, it is important that FR2 applicability is not discarded when progressing the BS RF requirements work for the LP-WUS feature.

Observation 1: Based on RAN guidance and previous agreements in RAN4, FR2 support for LP-WUS is within the scope of Rel-19 work item.

Proposal 1: Following RAN guidance, RAN4 to consider FR2 support when discussing BS RF requirements for LP-WUS.

## 2.1	Applicable BS types

In RAN4#112bis, the following regarding applicable BS type and type of requirements to be defined for LP-WUS feature was agreed:

#### Issue 1-6: On applicable BS type for LP-WUS feature - Proposals - Proposal 1: Not to set restriction on applicable BS types to support LP-WUS. (CMCC, CATT, Nokia) - Proposal 2: BS type 1-C, 1-H and 1-O are all applicable BS types for LP-WUS. (Ericsson) - Proposal 3: Use BS Type 1-C as applicable BS type as a start point. (ZTE) - Proposal 4: BS types 1-C, 1-H and 1-O are more like the applicable requirements to a certain BS as the same BS can support both type 1-C and type 1-H requirements, which is up to the manufacturer’s declaration. Focusing on the applicable requirements rather than the BS types (Huawei) - WF Non-AAS, AAS could be considered for LP-WUS feature #### Issue 1-7: Whether to define the LP-WUS requirements only for supported BS types - Proposals - Proposal 1: RAN4 to only define conductive based BS RF requirements for LP-WUS in Release-19. (CATT) - Proposal 2: Only conductive based BS type 1-C and 1-H requirements are considered for LP-WUS. (Huawei) - WF - FFS.

In order not to restrict the adoption of LP-WUS feature to a limited deployment scenario, it is a viable option for RAN4 to consider proposal 1. Note that the ongoing discussions the power boosting definition, maximum and minimum power boosting levels, small channel BW support for the feature, etc., should be agnostic to the BS type.

Proposal 2: RAN4 not to set restriction on applicable BS types to support LP-WUS.

Regarding the BS types, similar rationale should be followed. With the goal of not limiting any foreseeable deployment of the LP-WUS features, RAN4 should define both conducted and radiated requirements for both FR1 and FR2. It is clear that from RAN guidance, the work within BS RF requirements on LP-WUS will be missing if RAN4 did not specify requirements for BS type 2-O. 

Observation 2: Following RAN guidance, normative work in Rel-19 will be missing if no BS type 2-O requirements are not defined. 

Proposal 3: RAN4 to define conducted (i.e., BS type 1-C) and radiated (i.e., BS type 1-H, Bs type 1-O, and BS type 2-O) requirements for LP-WUS.

# Conclusion

In this contribution we provide our views on BS RF requirements for LP-WUS. Our observations and proposals can be summarized as follows:

Observation 1: Based on RAN guidance and previous agreements in RAN4, FR2 support for LP-WUS is within the scope of Rel-19 work item.

Proposal 1: Following RAN guidance, RAN4 to consider FR2 support when discussing BS RF requirements for LP-WUS.

Proposal 2: RAN4 not to set restriction on applicable BS types to support LP-WUS.

Observation 2: Following RAN guidance, normative work in Rel-19 will be missing if no BS type 2-O requirements are not defined. 

Proposal 3: RAN4 to define conducted (i.e., BS type 1-C) and radiated (i.e., BS type 1-H, Bs type 1-O, and BS type 2-O) requirements for LP-WUS.

# Reference

[1]	R4-2416589, “Way Forward for [112bis][308] NR_LPWUS” RAN4#112bis, Huawei.

[2]		R4-2409956, “Way Forward for [111][314] NR_LPWUS”, RAN4#111, Huawei.

[3] 	RP-242361, “Way Forward on LP-WUS for FR2” RAN#105, vivo, , Sept. 2024