3GPP TSG-RAN WG4 Meeting #113	R4-2418612

# Orlando, USA, 18th October – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2551 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.101-1 adding missing PC2 harmonic mixing MSD for CA_n26-n78 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_FR1_TDD_NR_CADC_SUL_R18-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding missing PC2 harmonic mixing MSD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding missing harmonic mixing MSD for CA_n26-n78 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correction is not made |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1 for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 25 | 10 | 2.2 | NOTE 11 | UL4/DL3 |
| n25 | n713 | 5 | 15 | 25 | 5 | 29.5 | NOTE 4 | UL1/DL3 |
| n25 | n713 | 20 | 15 | 25 | 20 | 18.2 | NOTE 4 | UL1/DL3 |
| n39 | n41 | 5 | 15 | 6 | 5 | 10.6 | NOTE 8 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 100 | 4.9 | NOTE 8 | UL4/DL3 |
| n40 | n28 | 5 | 15 | 6 | 5 | 40.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 20 | 15 | 6 | 20 | 33.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 29.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 15 | 24.3 | NOTE 7 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 6.1 | NOTE 9 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 1.5 | NOTE 9 | UL3/DL4 |
| n41 | n77 | 10 | 15 | 6 | 10 | 11.6 | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 3.6 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 12.1 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 3.8 | NOTE 8 | UL4/DL3 |
| n77 | n2 | 10 | 15 | 25 | 5 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 4.4 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 5 | 17.7 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 50 | 3.6 | NOTE 3 | UL2/DL3 |
| n77 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 7.9 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.5 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 14.1 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 10.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n772 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 4 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n78 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n78 | n5 | 10 | 15 | 25 | 5 | [8.1] | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 20 | [1.5] | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 10 | 15 | 25 | 30 | 11.3 | NOTE 1 | UL1/DL5 |
| n78 | n40 | 10 | 15 | 12 | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 4.4 | NOTE 3 | UL2/DL3 |
| n79 | n8 | 10 | 15 | 25 | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: Void NOTE 11: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

---End of changes---