3GPP TSG-RAN WG4 Meeting #113		R4-2418627

Orlando, US, 18th – 22nd November 2024

Agenda item:	7.15.2.1

Source:	Intel Corporation

Title:	Discussion on delay reduction by optimizing Rx beam sweeping factors

Document for:	Discussion

# Introduction

The Rel-19 NR RRM enhancements work item was approved in March 2024 and an updated WID was approved in RANP #105 in [1]. The work item descriptions of objectives of the item are shown in the below box.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last WG meeting, a WF was approved in [2] to capture all the discussions and agreements.

In this paper we provide our analysis on delay reduction by optimizing Rx beam sweeping factors.

# Discussion

## Multi-Rx simultaneous reception enabling delay reduction of FR2-1 SSB based L3 measurements

Faster beam sweeping during handover and RRC re-establishment

In the last WG meeting RAN4 agreed that for handover and re-establishment, UE supporting Rel-19 faster beams sweeping should always apply reduced beam sweeping factor during handover and RRC re-establishment procedures.

Agreement: - For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Regarding handover delay requirements, this agreement means that Tsearch = N * Trs or 3N * Trs when the target cell is an unknown FR2 cell where N should be 4 (compared to 8 in the legacy spec) for FR2-1 and 6 (compared to 12) for FR2-2.

We propose the below TP for Rel-19 specification for FR2 handover delay requirements.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - If the target cell is a known intra-frequency cell, then Tsearch = 0ms. - If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. - Otherwise, the target cell is an unknown inter-frequency cell and Tsearch = N*3* Trs ms if the target cell Es/Iot≥-2 dB. Where N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. If the UE supports [Rel-19 BSF reduction capability] and the network configures handover with reduced beam sweeping factor to the UE, N = 4 when the target cell is in FR2-1, and N = 6 when the target cell is in FR2-2.

Similarly for the re-establishment delay requirements, time to identify target NR FR2 cells for RRC connection re-establishment to NR intra- or inter- frequency cell is reduced in half.

New Table in 6.2.1.2.1: Time to identify target NR cell for RRC connection re-establishment to NR FR2 intra-frequency cell for UE supporting [Rel-19 BSF reduction] Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR2-1; N/A; MAX (1000 ms, [40] x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, [60] x TSMTC)). < -8; FR2-1; N/A; [1760]Note1. < -8; FR2-2; N/A; [2640]Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB..

New Table in 6.2.1.2.1: Time to identify target NR cell for RRC connection re-establishment to NR FR2 inter-frequency cell for UE supporting [Rel-19 BSF reduction] Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR2-1; N/A; MAX (1000 ms, [52] x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, [78] x TSMTC, i)). < -8; FR2-1; N/A; [2000]Note1. < -8; FR2-2; N/A; [3000] Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB..

Proposal 1: Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements.

Conditions for reduced beam sweeping factor in UE L3 measurements

For FR2 L3 measurements the UE processes all the SSB-s configured on the serving frequency within a SMTC period. In a typical 120kHz SCS deployment, 64 SSB-s are configured within a SMTC window for all the cells on the same frequency. How each SSB is linked to the network Tx beam is up to implementation of the network, as the UE is only expected to use one single Rx beam to receive and process correctly one SSB out of all Tx beams during one whole SMTC period long.

We see potential for the group to enhance the measurement delay requirements by assuming that the UE can receive and process correctly multiple SSB-s in that period. And it is natural that we reduce the delay and cut it in half assuming that the UE is capable of receiving two Rx beams at the same time.

But when considering the UE has to process all the results during one measurement period (even within the SMTC/gap occasion), the group should allow reasonable margins on the reduction to accommodate different UE categories.

In the original design of L1 measurements, the main purpose is for UE to manage beams for serving cells. The L3 measurements however, aim to support mobility among cells. This difference results in that the major targets for L1 and L3 measurements are fundamentally different: L1 measurements target on serving cells (neighbour cells added in target list in recent releases for LTM) while L3 measurements target on neighbour cells and on inter-frequencies.

When L1 procedures are carried out, the UE focuses on measurement results on a single cell at a time for processing. However for L3 measurements, the UE processes with all the possible cell ID-s in each of the measurement results per Rx beam per SSB index (Tx beam). This is to say that the baseband resources consumed by L1 procedures and L3 measurements are largely different. Even that the UE apply the same ability to either L1 or L3 measurements in terms of simultaneous receptions of the reference signals, processing the obtained results within the measurement period for L3 in FR2 with large bandwidth (not to mention CA) and many indexes is much more challenging compared to L1.

In Rel-19 we don’t expect significant changes in the UE implementations. From this starting point the group needs to carefully conclude on the preconditions and amplitude for the reduction to apply for L3 measurement delays.

Observation 1: Under the simultaneous multi-Rx reception delay reduction scope, UE baseband processing capabilities have impacts on the amount of reduction applied, considering the existing UE implementation.

To summary here, we need to combine the preconditions for multi-Rx simultaneous reception and for successful parallel processing to apply at the same time. The considered preconditions are:

  - Multi-Rx simultaneous reception active, following Rel-18 definitions as much as reasonable, including,

    - Group based beam reporting is configured for the UE not long ago prior to the expected L3 reporting,

  - Measurement configurations meet processing limitations, including,

    - Single carrier is configured,

    - Measurement periodicity is large enough compared to the number of SSB to be measured in a burst,

  - Existing side conditions for L3 measurements.

Proposal 2: Conditions for UE to apply multi-Rx reception and reduced beam sweeping factor include the below aspects:

  - Multi-Rx simultaneous reception active, following Rel-18 definitions as much as reasonable, including,

    - Group based beam reporting is configured for the UE not long ago prior to the expected L3 reporting,

  - Measurement configurations meet processing limitations, including,

    - Single carrier is configured,

    - Measurement periodicity is large enough compared to the number of SSB to be measured in a burst,

  - Existing side conditions for L3 measurements.

To mention other proposals a bit: mobility status should not be considered as a limitation for UE delay reduction since the purpose of the reduction is to have greater mobility in general so we are not supposed to compromise on mobility status; power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced; whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions.

Proposal 3: Mobility status should not be considered as a limitation for UE delay reduction since the purpose of the reduction is to have greater mobility in general, so we are not supposed to compromise on mobility status.

Proposal 4: Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced.

Proposal 5: Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions.

# Conclusion

In this paper we provide our views on the scope of FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: Beam sweeping factor/total sample number of measurements during handover and re-establishment can be cut half as a baseline while a margin can be added to the requirements.

Proposal 2: Conditions for UE to apply multi-Rx reception and reduced beam sweeping factor include the below aspects:

  - Multi-Rx simultaneous reception active, following Rel-18 definitions as much as reasonable, including,

    - Group based beam reporting is configured for the UE not long ago prior to the expected L3 reporting,

  - Measurement configurations meet processing limitations, including,

    - Single carrier is configured,

    - Measurement periodicity is large enough compared to the number of SSB to be measured in a burst,

  - Existing side conditions for L3 measurements.

Proposal 3: Mobility status should not be considered as a limitation for UE delay reduction since the purpose of the reduction is to have greater mobility in general, so we are not supposed to compromise on mobility status.

Proposal 4: Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced.

Proposal 5: Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple