3GPP TSG-RAN WG4 Meeting #113	R4-2418653

# Orlando, US, 18th – 22nd November, 2024

Agenda item:  7.22.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES OD-SSB

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discuss the RRM impacts of OD-SSB with issues and agreement captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

Regarding the general issues of OD-SSB, RAN4 achieved following agreements in last meeting:

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. - This agreement can be revisited if it is not aligned with further agreement in other WGs. Issue 1-1-3: OD-SSB type Agreement: - RAN4 to first take NCD-SSB as a start point for OD-SSB requirements.

Besides, RAN4 send a LS to RAN1 asking for clarification on relation between OD-SSB and AO-SSB in case 2 [2]. The question and answers are essential for further RAN4 discussion. Thus, RAN4 shall wait for RAN1 LS reply on important assumptions and the corresponding RAN4 agreements/assumption could be updated if needed.

Observation 1: RAN4 shall wait for RAN1 LS reply on important assumptions and the corresponding RAN4 agreements/assumption could be updated if needed in further meeting.

In the following part, we provide our views on RRM impacts for deactivated SCell measurement/SCell activation in case by case manners.

## Deactivated SCell L3 measurement (case 1)

For deactivated SCell L3 measurement in case 1, following agreements are achieved:

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Based on above agreements, the expected UE behavior is shown in following figure.

Fig.1 Deactivated SCell measurement based on OD-SSB in Case 1

Thus, we think following question should be discussed to have a full picture of UE measurement behavior:

- Aspect 1: The length of time window (T1 in the figure)

- Aspect 2: The interval between two activated OD-SSB (T2 in the figure)

For Aspect 1, we think the number of the available OD-SSB bursts shall guarantee at least one-time measurement. Taking existing requirements in clause 9.2.5.1 as reference, in FR2-1, UE needs 24 samples for Cell detection and measurement. Thus, the number of available OD-SSB bursts shall larger than the number of samples needed for Cell detection and/or cell measurement.

Proposal 1: The number of available OD-SSB bursts shall be larger than number of samples needed for Cell detection and/or cell measurement.

Thus, regarding the length of the time window, we think it is straightforward that UE shall performs the OD-SSB based fast L3 measurement for deactivated SCell for at least one complete measurement, and after that there is no requirements on UE measurement behaviour until next OD-SSB activation.

Proposal 2: UE shall perform the OD-SSB based fast L3 measurement for deactivated SCell for at least one complete measurement.

Considering that different UE may have different capabilities on measurement in terms of number of samples needed. Thus, there is no need to configure the length of the window as long as UE complete the measurement.

Observation 2: Different UE may have different capabilities on measurement in terms of number of samples needed.

Proposal 3: Based on P2, no need to configured the length of the window and UE can stop “fast measurement” as long as UE has completed the measurement.

For aspect 2, the deactivated SCell measurement is not a one-shot measurement, instead UE is required monitor the quality of the deactivated SCell periodically for potential SCell modification/activation. Thus, RAN4 shall consider how to perform long-term deactivated SCell monitoring based on OD-SSB.

Observation 3: RAN4 shall consider how to perform long-term deactivated SCell monitoring based on OD-SSB instead of one-shot measurement.

Based on observation 3, two aspect shall be considered:

- Aspect 2-1: UE is not required to perform “fast measurement” to frequently as UE is required to perform measurement per OD-SSB periodicity instead of measCycle.

- Aspect 2-2: The interval between two activated OD-SSB shall not be too far away that UE needs to search the Cell again.

For Aspect 2-1, as mentioned by many companies, in legacy cases, the minimum measurement cycle is derived based on measCycle equal to 160 ms. When take FR2 PC3 as example, UE is required to measurement the FR2 deactivated SCell every 160*24 =3840 ms. For OD-SSB, if gNB triggers another OD-SSB immediately after the previous OD-SSB, UE shall not be required to continuously performing measurement for power consumption issue, as shown in Fig.2. Another issue needs to be considered is the available OD-SSB for measurement should not be too far away from each other. The consequence is that UE may loss tracking of the Cell and UE has to search the Cell before measurement. Thus, NW has no idea whether the SCell be activated as a known cell or fast SCell activated based on Rel-18 enhancement (i.e. L3 report after SCell activation).

Fig.2 Deactivated SCell measurement based on OD-SSB in Case 1 with too short/too long interval

Proposal 4: In order to have clear understanding from NW perspective on how to configure/trigger OD-SSB for deactivated SCell, RAN4 to specify following two conditions for the interval between activated OD-SSB for deactivated SCell measurement:

- The expected minimum interval between activated OD-SSB for deactivated SCell measurement, which can take exiting measurement period when MeasCycle =160 ms as reference. UE is not required to perform “fast measurement” more frequently than the minimum interval.

- The expected maximum interval between activated OD-SSB for deactivated SCell measurement, when UE can perform measurement without Cell detection.

Based on proposal 4, gNB will have better understanding on how to trigger OD-SSB for UE to perform deactivated SCell measurement.

However, another issue is about neighbor cell measurement. In legacy case, UE performs measurement per frequency layer/MO. When there is deactivated SCell on the frequency layer, UE will measurement both serving cell and neighbor cell using measCycle. However, if UE performs measurement for OD-SSB for deactivated SCell as described above, it is not clear how to perform neighbor cell measurement since it is not likely that the SSB of neighbor cell always overlapped with OD-SSB from serving cell.

Proposal 5: For neighbor cell measurement when UE performs deactivated SCell L3 measurement based on OD-SSB, RAN4 to considering following two options:

- UE is not required to perform neighbor cell measurement

- UE performs neighbor cell measurement based MeasCycle.

## Deactivated SCell L3 measurement (case 2)

For deactivated SCell L3 measurement in case 2, RAN4 achieved following agreements in last meeting:

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation - Background From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements. - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different frequencies. Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement. Issue 1-3-2: Deactivated SCell measurement before OD-SSB transmission activation Agreement: - The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) before UE receiving OD-SSB transmission indication. Issue 1-3-3: Deactivated SCell measurement after OD-SSB transmission deactivation Agreement: - The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) after the OD-SSB transmission deactivation. Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

From our understanding, the only difference between case 1 and case 2 is that UE can only rely on AO-SSB, and the measurement behavior can be exactly the same as legacy case. However, we think it is also possible to performance measurement as described in case 1.

Proposal 6: For case 2, the baseline requirements is that UE perform measurement based on AO-SSB, where the legacy requirements can apply.

Apart from the above baseline solution, we think it is also beneficial to perform measurement as described for case 1 from both gNB side and UE side. Specifically, NW can configure long periodicity AO-SSB, and trigger OD-SSB for deactivated SCell measurement occasionally which can obtain network power saving. At the same time without increase UE power consumption.

Proposal 7: For case 2, RAN4 also specify UE measurement behavior/requirements specified for case 1 when UE performs deactivated SCell measurement based on OD-SSB.

## OD-SSB based SCell activation

For OD-SSB based SCell activation, RAN4 achieved following agreements in last meeting.

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

Apart from the high-level principles agreed in last meeting, one important scenario should be considered is the Rel-18 SCell activation enhancement with L3 reporting after SCell activation command, which is mentioned during the online discussion. For unknown SCell activation with L3 report after SCell activation, there is no AGC/Search/L1-RSRP during the SCell activation procedure. However, one possible enhancement is that UE report L3 report based on OD-SSB in deactivated state. In Rel-18 discussion, RAN4 didn’t define the condition w.r.t measurement period that UE can report valid L3 reporting after receiving SCell activation command.

Proposal 8: RAN4 to define requirements for unknown SCell activation with L3 report after SCell activation when the L3 measurement report is based on OD-SSB (Scenario#2) measurement.

## OD-SSB triggering offset

RAN1 also discussed the time offset between the OD-SSB and MAC CE which triggers the OD-SSB, and RAN1 send a LS to RAN4 to ask about the minimum value of the time offset [3]. RAN1 asks RAN4 to confirm the minimum value of T_min. The relevant agreements were further updated in last RAN1 meeting which is shown as follows:

Agreement 118b The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Base on discussion in last RAN4 meeting, companies have different views on whether to have additional processing time on top of RAN1 working assumptions. As commented in last meeting, how to define the value of Tmin highly depends on the conclusion on the LS sent to RAN 1. For case#2 when there is always-on SSB, it is possible that the first OD-SSB is very close to the previous AO-SSB, which is shown in Fig.1. UE may have problem to process the OD-SSB, if UE is required to measure both of them. Another aspect is related to the F/T location of OD-SSB AO-SSB. RAN1 is still under discussion on how to allocate the AO-SSB and OD-SSB. One case is that the AO-SSB and OD-SSB is in different frequency. For this case, UE may need to switch to another frequency for OD-SSB measurement, and RAN4 shall investigate whether there is additional time.

Fig.1 Case#2 when the OD-SSB is very close to AO-SSB

Fig.3 Case#2 when the OD-SSB and AO-SSB is in different frequency

Proposal 9: For the value T_min for OD-SSB triggering, RAN4 confirm the MAC CE processing time as RAN1’s working assumption. RAN4 to further investigate whether to consider additional processing time for following cases, which depends on further RAN1 clarifications on relation between OD-SSN and AO-SSB.

- when the OD-SSB is very close to the last AO-SSB in time domain

- when the OD-SSB and AO-SSB is in different frequency (frequency location w.r.t active BWP).

# Conclusions

Observation 1: RAN4 shall wait for RAN1 LS reply on important assumptions and the corresponding RAN4 agreements/assumption could be updated if needed in further meeting.

Proposal 1: The number of available OD-SSB bursts shall be larger than number of samples needed for Cell detection and/or cell measurement.

Proposal 2: UE shall perform the OD-SSB based fast L3 measurement for deactivated SCell for at least one complete measurement.

Observation 2: Different UE may have different capabilities on measurement in terms of number of samples needed.

Proposal 3: Based on P2, no need to configured the length of the window and UE can stop “fast measurement” as long as UE has completed the measurement.

Observation 3: RAN4 shall consider how to perform long-term deactivated SCell monitoring based on OD-SSB instead of one-shot measurement.

Proposal 4: In order to have clear understanding from NW perspective on how to configure/trigger OD-SSB for deactivated SCell, RAN4 to specify following two conditions for the interval between activated OD-SSB for deactivated SCell measurement:

- The expected minimum interval between activated OD-SSB for deactivated SCell measurement, which can take exiting measurement period when MeasCycle =160 ms as reference. UE is not required to perform “fast measurement” more frequently than the minimum interval.

- The expected maximum interval between activated OD-SSB for deactivated SCell measurement, when UE can perform measurement without Cell detection.

Proposal 5: For neighbor cell measurement when UE performs deactivated SCell L3 measurement based on OD-SSB, RAN4 to considering following two options:

- UE is not required to perform neighbor cell measurement

- UE performs neighbor cell measurement based MeasCycle.

Proposal 6: For case 2, the baseline requirements is that UE perform measurement based on AO-SSB, where the legacy requirements can apply.

Proposal 7: For case 2, RAN4 also specify UE measurement behavior/requirements specified for case 1 when UE performs deactivated SCell measurement based on OD-SSB.

Proposal 8: RAN4 to define requirements for unknown SCell activation with L3 report after SCell activation when the L3 measurement report is based on OD-SSB (Scenario#2).

Proposal 9: For the value T_min for OD-SSB triggering, RAN4 confirm the MAC CE processing time as RAN1’s working assumption. RAN4 to further investigate whether to consider additional processing time for following cases, which depends on further RAN1 clarifications on relation between OD-SSN and AO-SSB.

- when the OD-SSB is very close to the last AO-SSB in time domain

- when the OD-SSB and AO-SSB is in different frequency (frequency location w.r.t active BWP).

# References

[1] R4-2416916 WF on RRM requirements for Netw_Energy_NR_enh_Part1

[2] R4-2416913 LS on SSB relation in OD-SSB and SSB adaptation on Scell

[3] R1-2407565 LS on timeline for On-demand SSB operation on SCell