3GPP TSG-RAN WG4 Meeting # 113	R4-2418670

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on Event triggered L1 measurement reporting

Source: 	Huawei, HiSilicon

Agenda item:	7.24.2.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on 7.24.2.1	Event triggered L1 measurement reporting.

Discussion

It is agreed in RAN2 that it’s up to RAN1 whether the specified L1 filtering is needed or to leave it to UE implementation. RAN1 made the further agreement in RAN1#118bis meeting that no L1 specified filtering for time and frequency domain is introduced. From RAN4 perspective, as one shot sample is applied for CSI-RS based L1-RSRP in serving cell, to ensure the fairness between the RSRP level from candidate cells and that of serving cell, L1 filtering on neighbour cell is NOT expected. Moreover the gain from early L1 measurements is to enable network quick acquire the measurement results, introduction of L1 filter would vanish the gain.

RAN1 Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Proposal 1: From RAN4 perspective, no L1 filtering is needed for event triggered L1 report.

For R19 LTM, both event triggered reporting and event triggered periodic reporting are supported.

RAN1 Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting

RAN2 Agreements on L1 event triggered MR

- Event triggered periodic MR can be supported, based on NW configuration.

Regarding L3 measurement, event triggered periodic reporting and event triggered reporting requirements are specified as below. It is straight forward that the “Event Triggered Reporting” and “Event-triggered Periodic Reporting” for LTM L1-RSRP measurement requirements would be introduced in section 9.14.3 and section 9.15.3 in R19.

Following the legacy definition for L3 measurement, the measurement reporting delay for L1 RSRP can be defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. The event triggered measurement reporting delay shall be less than the L1-RSRP measurement period on the candidate cells.

#### 9.2.4.2 Event-triggered Periodic Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2.4.3. #### 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap. A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

Proposal 2: “Event Triggered Reporting” and “Event-triggered Periodic Reporting” for LTM L1 measurement are to be introduced in section 9.14.3 and section 9.15.3.

Proposal 3: The measurement reporting delay for L1 RSRP can be defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. The event triggered measurement reporting delay shall be less than the L1-RSRP measurement period on the candidate cells.

Conclusion

The contribution provides analysis on Event triggered L1 measurement reporting. The following proposals are proposed:

Proposal 1: From RAN4 perspective, no L1 filtering is needed for event triggered L1 report.

Proposal 2: “Event Triggered Reporting” and “Event-triggered Periodic Reporting” for LTM L1 measurement are to be introduced in section 9.14.3 and section 9.15.3.

Proposal 3: The measurement reporting delay for L1 RSRP can be defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. The event triggered measurement reporting delay shall be less than the L1-RSRP measurement period on the candidate cells.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple