3GPP TSG-RAN WG4 Meeting #113	R4-2418673

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.18.1

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects in AIML mobility

Document for:	Discussion and Decision

- Introduction

According to the revised SID on AIML for mobility in [1], RAN4 will start the study on AI mobility from this meeting until August 2025. In this paper, we discuss the general aspects in AIML mobility.

- General aspects of AI mobility in RAN4

Revised SID on AIML for mobility [1]: Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

For mobility enhancement, AI is introduced to predict the RRM measurement and event prediction. The identified use cases are AI based cell-level/beam-level measurement prediction, AI based HO failure/RLF prediction and AI based measurement event prediction.

Revised SID on AIML for mobility [1]: - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

For the identified use cases, the testability, interoperability, and impacts on RRM requirements and performance will be studied in RAN4.

RAN4 #112bis WF [2]: - Priorities of different use cases - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025). - Priorities of different scenarios - Scenario 2, 3 and 4, the 3 cases can be discussed: 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

According to the WF in [3], RAN4 starts the discussion with RRM measurement prediction use-case, where 3 sub-use cases are identified for further study. The discussion of measurement event prediction use-case and RLF use-case will be discussed if sufficient progress is achieved in RAN2.

General aspects in RAN2 AI mobility #126 [2]: - Conclusion 1: 1st study goal of evaluation is to reduce measurement overhead - Conclusion 2: 2nd study goal of evaluation is to enhance handover performance

As shown above, two study goals in RAN2 are defined for verifying the gain introduced by AI in RAN2. One is to reduce the measurement overhead, another is to enhance handover performance.

2.1 Testing goal

In legacy, the requirement of measurement accuracy is defined in RAN4. For AI based cell-level/beam-level measurement prediction, RAN4 can study the requirement of prediction accuracy. For AI based AI based HO failure/RLF prediction and AI based measurement event prediction, if the eventual prediction is derived from predicted measurement, RAN4 can also study the requirement of prediction accuracy.

In the cases where the measurement can be reduced by AI prediction, there are two options for defining the testing goal.

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy margin requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

Proposal 1: RAN4 can discuss the following two options:

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

With option 1, RAN4 can ensure that the accuracy of the predicted measurement will not be worse than that of legacy measurement. Therefore, no handover performance degradation compared to non AI is expected.

With option 2, whether the predicted measurement will have an impact on handover performance may need to be evaluated. If there is no handover performance degradation or the degradation is smaller enough to accept, then the benefit of AI can still be verified in terms of reducing the cost of RRM measurement.

It is observed that in some cases, for example, in the inter frequency prediction, the measurement reduction may help reduce the measurement gap. If the measurement gap can be reduced, more resources for data transmission can be obtained. In these cases, additional benefit of AI is seen in terms of improving the user experience of services like XR in mobility.

Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

2.2 Training and testing dataset

In RAN2, the AI model is trained by system level channel model and verified in terms of the accuracy of measurement prediction under the same channel model. However, in RAN4 legacy, only link level channel model is considered for defining test cases. There are two potential testing options for RAN4 in AI RRM measurement prediction.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

There are two options for defining the testing and training dataset. One is using the channel model and the sampling rules. Another is that the dataset is specified or partially specified in a sample-by-sample manner.

Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 can discuss the following two options:

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameters and the values, e.g., the prediction window.

Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

References

- R4-2410570, “Revised SID on AIML for mobility in NR”, 3GPP RAN#105, September 9-12, 2024.

- R4-2416855, “WF on AI/ML Mobility”, 3GPP RAN4#112bis, 14th – 18th October, 2024.

- R2-2405941, “Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO)”, 3GPP RAN2#126, May 20th -24th, 2024.