3GPP TSG-RAN4 Meeting # 113	R4-2418690

Orlando, USA, Nov.18th – 22th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Further discussion on R19 PC1.5 Intra-band UL CA

# Agenda item:	7.1.1.1.2

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements/open issues are included in the WF[1]. In this contribution, we give some further discussions on PC1.5 intra-band contiguous and non-contiguous UL CA.

# 2	Discussion

Some agreements on PC1.5 for intra-band UL CA are listed below:

## 1. PC1.5 for intra-band contiguous and non-contiguous UL CA ### 1.1 MPR assumption Agreement: - Keep the following last meeting’s agreement for intra-band non-contiguous ULCA - Define MPR/A-MPR requirements based on equal PSD assumption for intra-band contiguous ULCA - It does not preclude companies to bring evaluation for unequal PSD - MPRc = MPR, which means no change of spec ### 1.2 PCMAX,f,C and PCMAX Agreement: - For a contiguous intra-band combination with PC1.5 capability supporting by txDiversity-r16 - The PCMAX,f,c and the total configured output power PCMAX for the band combination are given by clause 6.2A.4.1.1 for intra-band contiguous CA in the current version of 38.101-1, no change for PC1.5 - For a non-contiguous intra-band combination with PC1.5 capability supporting dualPA-Architecture - For PCMAX,f,c, keep the following last meeting’s agreement, and the detailed wording can be discussed based on the CR in future - For PCMAX, FFS

In general, we think the assumption would be the same with the existing one, i.e. equal PSD assumption since the MPRc is the same for each UL CC and the same as the MPR for the total intra-band power according to the specification. It is true that when the large non-equal PSD between CCs is assumed, then emission out of each CC would be large different, and lower RB would cause higher leakage emission. We think the situation is existed for either intra-band contiguous CA or intra-band non-contiguous CA no matter PC3/2/1.5 power class. In addition to that, RAN5 specify the test configurations of RB allocatation in each CC for the MPR requirement testing. But in RAN4 spec, the MPR requirements should be applied to any UL PSD imbalance across uplink carriers, this is aligned with the current specification, where the PSD information for MSD are not explicated in the texts of MPR definition.

- Proposal 1: To reuse the existing same methodology, i.e. equal PSD assumption to defined the requirements.

For Pcmax the agreements in RAN4 #112 meeting is:

Online agreement with adhoc modification: - For intra-band contiguous ULCA with 2Tx-TxD, the upper bound of Pcmax is 29dBm - For intra-band UL NC CA with dual PA architecture, the upper bound of Pcmax is 29dBm

PCMAX is the total configured maximum output power and is set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

- For PC1.5 UL inter-band CA, it was already agreed that the upper bound of Pcmax is 29dBm for both intra-band contiguous CA and intra-band non-contiguous CA, which 29dBm is the PPowerClass,CA =PC1.5. Hence it seems no changes of specification for Pcmax for both intra-band contiguous ULCA with 2Tx-TxD and intra-band UL NC CA with dual PA architecture.

- Proposal 2: No changes of specification for Pcmax.

- For Pcmax,c the agreements in RAN4 #112 meeting is:

- PCMAX,C Adhoc agreement: - For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC - Whether or how to reflect the above agreement in spec is FFS - For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC - Whether or how to reflect the above agreement in spec is FFS

- For intra-band contiguous CA with TxD, it was already agreed that no changes of specification for Pcmax,c, the current specification can well reflect the the upper bound of Pcmax,c is 29dBm per CC.

- For intra-band UL NC CA with dualPA-architecture, due to the upper bound of Pcmax,c is 26dBm per CC, rather than 29dBm, so some changes should be done to reflect the 3dB lower of the CA power class.

- Observation: Some changes should be done to reflect Pcmax,c is 3dB lower than the CA power class (29dBm) for intra-band UL NC CA with dualPA-architecture

- As stated in the last WF, the changes can be discussed in the CR in future.

# 3 Conclusion

In this contribution, we give further discussions on PC1.5 Intra-band contiguous and non-contiguous UL CA. The proposals and conclusions are:

- Proposal 1: To reuse the existing same methodology, i.e. equal PSD assumption to defined the requirements.

- Proposal 2: No changes of specification for Pcmax.

- Observation: Some changes should be done to reflect Pcmax,c is 3dB lower than the CA power class (29dBm) for intra-band UL NC CA with dualPA-architecture.

# 4	Reference

[1] R4-2417098, WF on HPUE RF requirements, Samsung