3GPP TSG-RAN4 Meeting # 113	R4-2418692

Orlando, USA, Nov.18th – 22th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	On release independence for 3Tx inter-band UL CA for handheld 		UE and FWA UE

# Agenda item:	7.1.1.1.3

# Document for:	Approval

# 1 Introduction

One of the objectives for the R19 NR_ENDC_RF_Ph4 topic is ‘consider release independency, if needed’. In RAN4 #111 meeting, we first discussed the issue about the release independence for 3Tx inter-band UL CA for handheld UE in [1]. However, at that time, the agreement in [2, see below] was to discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WID due to there are lots of general parts should be defined first.

#### 3.4 Release independence Way forward: Discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WI - For FWA, only discuss the configurations that were not introduced in Rel-18

As of the RAN4 #112bis meeting, except the SAR issue which belongs to all the objectives, the other issues have already been agreed thanks to moderator’s good organization.

Therefore, in this contribution, we give the discussions for the release independence for 3Tx band combination for handheld UE and FWA UE.

# 2	Discussion

According to the WF [2], the release independence for 3Tx band combination for handheld UE and FWA includes:

1. FWA UE: for the configurations that were not introduced in Rel-18

2. Handheld UE: All the configurations

FWA UE: for the configurations that were not introduced in Rel-18

For the requirements for 3Tx band combination for handheld UE in Rel-19 NR_ENDC_RF_Ph4, it was already agreed that the requirements defined in R18 FWA requirements for NR CA/ENDC are applied.

In current table 5.2.2-1 in TS38.307, R18 3Tx band combination is release independence from Rel-17 and applied to FWA UE, which is shown in the following.

It can be seen from the table that detail information for UL and DL configurations are included in table, including maximum DL or UL band associated with number of CC, CA BW classes and duplex mode. In current specification, only maximum 2 DL/UL bands with 2UL CCs are included since only 3Tx inter-band CA configurations with total 2CC in either DL or UL 2bands were introduced in Rel-18.

Observation 1. For 3Tx inter-band UL CA release independence in TS38.307, the details information for DL/UL configurations are included in the table, including maximum DL or UL band associated with number of CC, CA BW classes and duplex mode.

In Rel-19, the valid UL configurations were agreed for 3Tx inter-band CA, which are:

- Don not consider 3ULCC with 3Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario

- Do not consider 4UL CC with PC2/PC1.5 2Tx/3Tx inter-band UL NR CA/ENDC scenario.

The above agreements are applied for both FWA UE and handheld UE.

However, for the DL band number, there are no restriction during the discussion in R19 NR_ENDC_RF_Ph4 WID , instead it can be referred to HPUE_NR_CADC_SUL_R19 WID. However, there are no explicit information for the DL band number in revised WID [3]. Ideally, up to 6 DL bands can be considered but it depends on the proponent’s request.

In the draft big CR[4], still there are only two bands 2 DL/UL bands with 2UL CCs are introduced for 3Tx inter-band CA configurations. However, there exists 3DL bands or 2 DL bands with 3DL CC were requested in the excel sheet in the HPUE_NR_CADC_SUL_R19 WID [3].

Due to the request for new configurations for 3Tx inter-band CA will continue for the whole R19, it is impossible to predict which types of DL/UL configurations will be requested by the proponents, and even there are requests but it is impossible to predict when to introduce the specification via basket WID work.

Observation 2. It is impossible to predict which types of DL/UL configurations will be requested, and also it is impossible to predict when to introduce the specification via basket WID work.

But the discussion in R19 NR_ENDC_RF_Ph4 WID should be general to applicable for all of the possible/valid configurations. Unpredictable DL/UL configurations in the observation 2 will make it quite complicated for the release independence discussion if considering the detail information for DL/UL as mentioned in observation 1.

To void the complicated situation, we propose to apply the same release independence for all the configurations of 3Tx inter-band CA for handheld UE and FWA UE, respectively.

Proposal 1. To apply the same release independence for all the configurations of 3Tx inter-band CA for handheld UE and FWA UE, respectively.

With the above proposal 1, for the configurations that were not introduced in Rel-18 for FWA UE, the release independence is from Rel-17, which is same with the existing ones.

Proposal 2. For the 3Tx inter-band UL CA/ENDC configurations that were not introduced in Rel-18 for FWA UE, the release independence is from Rel-17.

For handheld UE, it seems it would be impossible for 3Tx band combination handheld UE to be released independence from R16, the earliest release should be the same as FWA UE, i.e. R17. To make the 3Tx inter-band CA band combinations are valid in the filed as earlier as possible, it is proposed to be release independent from Rel-17.

would be R17, which is same as FWA UE. Meanwhile, from R18, even R19 could also be the options. So we have following proposal.

Proposal 3: For the 3Tx inter-band UL CA/ENDC configurations for handheld UE, the release independence is from Rel-17.

With the above proposals, the only changes are to introduce the new introduced configuration in the table, an example can be found below. The changes depends on the configurations in Rel-19 specification.

It should be noted that if the different release independence for FWA UE and handheld UE, then different NOTEs should be needed for FWA UE and handheld UE, separately.

| Feature | DL/UL | Maximum number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| .... |  |  |  |  |  |  |  |
| Inter-band CA configurations with UL MIMO within FR1 | DL | 2, [3,4,5,6] | 2, [3,4,5,6] | A,[B,C..] | TDD, FDD and TDD, [..] | Rel-17 | Table B.4.2-3 |
|  | UL | 2 | 2, [3] | A, [B,C] | TDD, FDD and TDD, [..] | Rel-17 |  |
| Inter-band CA configurations with Tx Diversity within FR1 | DL | 2, [3,4,5,6] | 2, [3,4,5,6] | A, [B,C..] | FDD and TDD, [..] | Rel-17 | Table B.4.2-4 |
|  | UL | 2 | 2, [3] | A, [B,C] | FDD and TDD, [..] | Rel-17 |  |
| .... |  |  |  |  |  |  |  |
| NOTE 1: Void. |  |  |  |  |  |  |  |

# 3 Conclusion

In this contribution, we give the discussions for the release independence for 3Tx band combination for handheld UE and FWA UE. The proposals and conclusions are:

Observation 1. For 3Tx inter-band UL CA release independence in TS38.307, the details information for DL/UL configurations are included in the table, including maximum DL or UL band associated with number of CC, CA BW classes and duplex mode.

Observation 2. It is impossible to predict which types of DL/UL configurations will be requested, and also it is impossible to predict when to introduce the specification via basket WID work.

Proposal 1. To apply the same release independence for all the configurations of 3Tx inter-band CA for handheld UE and FWA UE, respectively.

Proposal 2. For the 3Tx inter-band UL CA/ENDC configurations that were not introduced in Rel-18 for FWA UE, the release independence is from Rel-17.

Proposal 3: For the 3Tx inter-band UL CA/ENDC configurations for handheld UE, the release independence is from Rel-17.

# 4	Reference

[1] R4-2408352, On R19 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC, ZTE Corporation

[2] R4-2410750, WF on high power UE, Samsung

[3] RP-242003_Revised WID for HPUE_NR_CADC_SUL_R19, China Telecom, China Unicom

[4] R4-2414978, DraftBig CR to 38.101-1 new combinations for HPUE_NR_CADC_SUL_R19, China Telecom, China Unicom