3GPP TSG-RAN WG4 Meeting #113				                       R4-2418724

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.21.2.1

Source: 	CMCC

Title: 	Evaluation methodology for ambient IOT co-existence

Document for:	Discussion

# 1. Introduction

In previous RAN4 meetings, it was agreed to use 5% throughput loss as performance metric as legacy. For AIOT system, it was agreed to consider the following methodology to derive the outage percentage [1].

Agreement: - Consider 3 SINR values for D2R and R2D respectively and derive the outage percentage value/range. - The 3 values can be selected according to following steps: - Select one set SNR/CNR values @ 10% BLER from companies based on RAN1 LLS (refer to latest RAN1 LLS summary) - Set the SNR/CNR range by removing the highest and lowest value from the set in step 1 - Pick 3 values from the range in step 2, e.g. highest, lowest, middle Note: R2D CNR needs to be transformed to SINR

In this contribution, following the above agreement, we provide the SNR values to derive the outage percentage value/range based on the LLS summary in RAN1.

# 2.Discussion

2.1 D2R SNR values

In RAN1, there are many different parameters and aspects considered, which lead to very big difference on the SNR values among companies. We identified some key factors that have big impact on the link lever performance, which are:

- coherent or non-coherent: it has been observed that with non-coherent assumption, there is about 10dB SNR degradation compared to coherent assumption. Hence, it is recommended use the LLS with coherent in RAN4.

- FEC or no FEC: big performance difference had been observed between FEC and no FEC, and for TDL-D (D2T2), all the LLS are based on with FEC. Hence, to keep the alignment, it is recommended to use results with FEC in RAN4.

- Data rate: for example, most results based on data rate range [1-10kbps], however there are some results which use very high data rate (60kbps) and big performance difference is observed. It is recommended not to use LLS with very high data rate (60kbps) in RAN4.

Observation 1: it is recommended use RAN1 LLS for D2R with the following assumption:

- Coherent

- With FEC

- Exclude very large data rate (60kbps)

Then, following the principle in proposal 1, the SNR values for D2R are derived as follows (The values in red are removed as the highest and lowest value.):

|  | D2R coherent TDL-A device1 with FEC (D1T1) (Table 7.1.3.1.1 - 1) | D2R coherent TDL-A device2a with FEC (D1T1) (Table 7.1.3.2.1 - 1) | D2R coherent TDL-D device1 with FEC (D2T2) (Table 7.1.3.1.1 - 2) | D2R coherent TDL-D device2a with FEC (D2T2) (Table 7.1.3.2.1 - 2) |
| --- | --- | --- | --- | --- |
|  | -8.77 | -8 | -11.2 | -11.2 |
|  | -8 | -7.5 | -7.5 | -7.7 |
|  | -7.1 | -7.1 | -6.8 | -7.5 |
|  | -6.4 | -6.3 | -6 | -7.2 |
|  | -6 | -5.9 | -5.9 | -6.8 |
|  | -5.9 | -4.7 | -5.8 | -6.8 |
|  | -5.9 | -4.69 | -5.7 | -6.2 |
|  | -5.3 | -4.27 | -5.4 | -6.03 |
|  | -4.3 | -4.1 | -5 | -5.8 |
|  | -4.1 | -0.9 | -4.66 | -5.7 |
|  | -2.44 | 0.23 | -4.33 | -5.68 |
|  | -2.17 | 0.5 | -3.5 | -3.8 |
|  | -2 |  |  |  |
|  | -0.3 |  |  |  |
|  | 0.23 |  |  |  |
|  | 0.5 |  |  |  |
|  | D2R coherent TDL-A device1 with FEC (D1T1) (Table 7.1.3.1.1 - 1) | D2R coherent TDL-A device2a with FEC (D1T1) (Table 7.1.3.2.1 - 1) | D2R coherent TDL-D device1 with FEC (D2T2) (Table 7.1.3.1.1 - 2) | D2R coherent TDL-D device2a with FEC (D2T2) (Table 7.1.3.2.1 - 2) |
| min | -8 | -7.5 | -7.5 | -7.7 |
| max | 0.23 | 0.23 | -4.3 | -5.68 |
| average | -4.2 | -4.5 | -5.7 | -6.5 |

For D2R, the main difference between device1 and 2a is the SFO assumption. It can be observed that LLS for different devices are very similar for either D1T1 or D2T2. To further simplify the evaluation, the target SNR values can be averaged between device1 and 2a as follows:

|  | D1T1 D2R (dB) | D2T2 D2R (dB) |
| --- | --- | --- |
| min | -7.8 | -7.6 |
| max | 0.23 | -5 |
| average | -4.3 | -6 |

Proposal 1: It is recommended to use following SNR values for D2R to derive outage percentage for AIOT system

|  | D1T1 D2R (dB) | D2T2 D2R (dB) |
| --- | --- | --- |
| min | -7.8 | -7.6 |
| max | 0.23 | -5 |
| average | -4.3 | -6 |

2.2 R2D SNR values

For R2D LLS, there are not many results as D2R. For example, for TDL-D results, there are only two companies provice results. Hence, it is recommended to use TDL-A results for R2D in RAN4.

Also, it was agreed in last meeting that for R2D, SINR at antenna connector is used as the starting point, i.e interference and noise bandwidth is 10MHz. Hence, CNR can be transformed to SNR by : SNR=CNR+10*LOG10(180/10000).

|  | TDL-A device 1 (Table 7.1.4.1.1 - 1) |  | TDL-A device 2a (Table 7.1.4.2.1 - 1) |  |
| --- | --- | --- | --- | --- |
|  | -8.1 |  | 1.73 |  |
|  | 2.6 |  | 3.37 |  |
|  | 3.99 |  | 4.73 |  |
|  | 6.5 |  | 6.23 |  |
|  | 9.22 |  | 6.38 |  |
|  | 9.74 |  | 8 |  |
|  | 10 |  | 8.65 |  |
|  | 12 |  | 21 |  |
|  | 12.83 |  | 22.5 |  |
|  | 15.5 |  | 37 |  |
|  | 26.7 |  |  |  |
|  | 39.7 |  |  |  |
|  | TDL-A device 1 CNR | R2D SNR for device 1 (dB) | TDL-A device 2a CNR | R2D SNR for device 2a (dB) |
| min | 2.6 | -14.8 | 3.37 | -14.1 |
| max | 26.7 | 9.3 | 22.5 | 5.1 |
| average | 10.9 | -6.5 | 10.1 | -7.3 |

Observation 2: it is recommended use RAN1 LLS for R2D with the following assumption:

- TDL-A

- SNR=CNR+10*LOG10(180/10000).

Proposal 2: It is recommended to use following SNR values for R2D to derive outage percentage for AIOT system

|  | R2D SNR for device 1 (dB) | R2D SNR for device 2a (dB) |
| --- | --- | --- |
| min | -14.8 | -14.1 |
| max | 9.3 | 5.1 |
| average | -6.5 | -7.3 |

# 3. Conclusions

In this contribution, we provide the SNR values to derive the outage percentage value/range based on the LLS summary in RAN1.

Observation 1: it is recommended use RAN1 LLS for D2R with the following assumption:

- Coherent

- With FEC

- Exclude very large data rate (60kbps)

Proposal 1: It is recommended to use following SNR values for D2R to derive outage percentage for AIOT system

|  | D1T1 D2R (dB) | D2T2 D2R (dB) |
| --- | --- | --- |
| min | -7.8 | -7.6 |
| max | 0.23 | -5 |
| average | -4.3 | -6 |

Observation 2: it is recommended use RAN1 LLS for R2D with the following assumption:

- TDL-A

- SNR=CNR+10*LOG10(180/10000).

Proposal 2: It is recommended to use following SNR values for R2D to derive outage percentage for AIOT system

|  | R2D SNR for device 1 (dB) | R2D SNR for device 2a (dB) |
| --- | --- | --- |
| min | -14.8 | -14.1 |
| max | 9.3 | 5.1 |
| average | -6.5 | -7.3 |

# 4. Reference

- R4-2417196, WF on co-existence study for A-IOT, CMCC

- R4-2414304, WF on Co-existence study for ambient IoT, CMCC

- R4-2412833, Topic summary for [112][131] FS_Ambient_IoT_solutions_part1

- R4-2406714, WF on Ambient IoT in NR, CMCC