# 3GPP TSG-RAN WG4 Meeting #113	R4-2418772

# Orlando, US, 18th – 22nd November, 2024

Title: 	On RRM requirements of on-demand SSB SCell operation

Source: 	OPPO

Agenda item:	7.22.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved OD-SSB based requirements [].

In this contribution, we will provide discussion on RRM requirements for On-demand SSB SCell operation.

Discussion

RAN4 has agreed to define requirements for OD-SSB based deactivated Scell measurement and Scell activation. And RAN4 will first focus on the MAC CE based single SCell activation case as starting point.

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Dynamic OD-SSB on/off transmission may not bring too much network energy saving gain but cause more UE power consumption, compared to legacy deactivated SCell measurement based on meascycleSCell.

If SCell activation command is not coming later than OD-SSB activation quickly, then this scenario does not make too much sense for both network energy saving and UE power saving. In addition, from RAN4 requirements perspective, it should restrict the time gap between OD-SSD activation command and SCell activation command to avoid too long waiting time and too much UE power consumption on deactivated SCell measurement. For RRM requirements, we think the OD-SSB is mainly to speed up SCell activation delay, and it may provide some extra information of to be activated SCell for network to decide whether to activate this Scell. We suggest the gap between two MAC CE commands should be no longer than T0, i.e, 5s, which comes from the reporting validity time in current known SCell definition.

In our view, once the OD-SSB activation indication is transmitted, e.g., MAC CE triggering, UE is expected to receive the first OD-SSB no later than T_min after the slot where the UE receives the indication. T_min at least includes HARQ processing delay and decoding time. Then UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. During this window, OD-SSB is not allowed to be deactivated. The time window is the smaller one of T0 and time gap between OD-SSD activation command and SCell activation command(Tgap), i.e., min(T0, Tgap).

Therefore, there may exist two kinds of time windows, the 1st one is the duration for UE receiving OD-SSB and performing measurements, and the 2nd one is the duration for Network transmitting OD-SSB. Both time windows start from UE receiving OD-SSB activation indication for one deactivated SCell.

Proposal 1: OD-SSB is not allowed to be deactivated during the time window in which UE performs the OD-SSB based fast L3 measurement for deactivated SCell.

Proposal 2: For Scenario 2, the time window is defined as the time duration starting from OD-SSB activation indication reception, and ending at the earlier one between SCell activation command reception and 5s from OD-SSB activation indication reception. The time duration is no longer than 5s.

For Scenario 2A, since the OD-SSB activation and Scell activation are indicated together, we think the time window is not needed to consider. The time window is to be introduced for Scenarios 2.

For Scenario 2, on case of the time gap between OD-SSB activation command and SCell activation command is too large, e.g., no less than 30s (much larger than measurement period of deactivated SCell), then UE is allowed to perform measurements on deactivated SCell with fulfilling legacy requirements, or stop measurements, because UE and network may not believe this deactivated SCell is ready to be activated (e.g., due to bad radio link quality or other implementation issues). From our side, we prefer UE to stop the measurement. It is also aligned with RAN4’s agreement that UE can perform deactivated SCell measurement with a slow mode after the time window.

Proposal 3: For Scenario 2, if SCell activation command is received much later than the time windows, UE is allowed to stop measurements on deactivated SCell after the time windows.

In addition, RAN4 is going to discuss the relation between time window and OD-SSB deactivation in the time domain. Since RAN1 and RAN2 are still discussing the solution of OD-SSB deactivation, e.g., configured signalling or timer, OD-SSB deactivation command may be not needed and we prefer to wait for their conclusion. From our side, we prefer the option 5 agreed in RAN1 (in the below agreement) that the OD-SSB transmission is deactivated when SCell activation is completed. That means, UE can perform the OD-SSB based measurement for deactivated SCell and/or for SCell activation, assuming network is not likely to only trigger OD-SSB transmission on target deactivated SCell but not trigger this SCell to be activated.

For scenarios 2 and 2A, we think it is possible to support faster L3 deactivated Scell measurement before Scell activation and during the SCell activation. The OD-SSB transmission is kept at least until the end of Scell activation delay, as proposed in option 5 above.

Proposal 4: For both Scenario 2 and Scenario 2A, once OD-SSB transmission is triggered, the OD-SSB is assumed to be not deactivated until the SCell activation is completed.

Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

For Case 1 and Case 2, we prefer a unified solution to define the RRM requirements. Before the SCell activation in Scenario 2, if OD-SSB is triggered, the time window defined for case 1 can be reused for case 2, since RAN4 already agreed UE to use OD-SSB to perform fast deactivated Scell measurements.

Proposal 5: Define unified solutions for both case 1 and case 2, including the time window and OD-SSB deactivation solution (in P1-P3 and P4 respectively).

During Scell activation, the similar logic of R18 faster Scell activation can be reused so that OD-SSB periodicity can be used to replace current Trs or Tfirst_SMTC.

After SCell activation, the L3 measurement on normal activated SCell should be unchanged due to UE power saving. Any OD-SSB should be directly taken as deactivated from UE side. For both Case 1 and case 2, we only expect to enhance requirements for SCell activation related procedure/purpose, but not L3 activated or deactivated SCell measurement themselves.

Proposal 6: During Scell activation, OD-SSB periodicity can be used to replace current Trs or Tfirst_SMTC for both case 1 and case 2.

Since RAN4 agreed to discuss both known and unknown Scell cases, we prefer to reuse the legacy framework at least including the effective time windows for reporting and SSB detection. We think the known/unknown Scell definition can be updated considering OD-SSB based measurement report and detection.

For scenario 2, if measurement report of deactivated Scell is report within X (e.g., 5s for FR1) before the point of Scell activation reception, and OD-SSB is kept detectable with fulfilling the updated measurement period requirements, then the Scell is taken as known. Otherwise it is unknown. The above can apply for both case 1 and case 2.

For Scenario 2a, since always-on SSB exists for case 2, the legacy known/unknown definition can be unchanged. But for case 1, the OD-SSB starts to transmit after Scell activation command and UE needs Tmin to be ready to receive OD-SSB, the Scell anyway can be taken as unknown. In addition, whether R18 unknown Scell activation enhancement with L3 report can be applied, RAN4 need more discussion. At this moment, we are open to such optimization based on R18 work.

Proposal 7: For scenario 2, the known/unknown SCell definition can be updated with OD-SSB based measurement period for the condition of “SSB is detectable”, and keeping measurement results report’s available time window unchanged. It applies for both case 1 and case2.

Proposal 8: For Scenario 2a, the legacy known/unknown definition can be unchanged for case 2, and RAN4 to only consider unknown case for case 1 and discuss whether to enhance based on R18 work on FR2 unknown Scell activation.

Conclusions

In this paper, we provided our views as follows:

Proposal 1: OD-SSB is not allowed to be deactivated during the time window in which UE performs the OD-SSB based fast L3 measurement for deactivated SCell.

Proposal 2: For Scenario 2, the time window is defined as the time duration starting from OD-SSB activation indication reception, and ending at the earlier one between SCell activation command reception and 5s from OD-SSB activation indication reception. The time duration is no longer than 5s.

Proposal 3: For Scenario 2, if SCell activation command is received much later than the time windows, UE is allowed to stop measurements on deactivated SCell after the time windows.

Proposal 4: For both Scenario 2 and Scenario 2A, once OD-SSB transmission is triggered, the OD-SSB is assumed to be not deactivated until the SCell activation is completed.

Proposal 5: Define unified solutions for both case 1 and case 2, including the time window and OD-SSB deactivation solution (in P1-P3 and P4 respectively).

Proposal 6: During Scell activation, OD-SSB periodicity can be used to replace current Trs or Tfirst_SMTC for both case 1 and case 2.

Proposal 7: For scenario 2, the known/unknown SCell definition can be updated with OD-SSB based measurement period for the condition of “SSB is detectable”, and keeping measurement results report’s available time window unchanged. It applies for both case 1 and case2.

Proposal 8: For Scenario 2a, the legacy known/unknown definition can be unchanged for case 2, and RAN4 to only consider unknown case for case 1 and discuss whether to enhance based on R18 work on FR2 unknown Scell activation.

Reference

- R4-2416916	WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson