3GPP TSG-RAN WG4 Meeting #113	R4-2418773

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.4.1

Source:	OPPO

Title:	Simulation results for NR LPWUS

Document for:	Approval

- Introduction

Previous RAN4 meetings discussed simulation assumptions for LP-SS measurements, and reached some agreements on as well as the target baseband accuracy requirements, i.e., {3.5, 2.5} dB, in [1][2][3]. This contribution will give our simulation results for LP- RSRP based on the updated simulation assumptions.

Issue 2-1-4: Accuracy baseline for simulation Agreement: Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: - For FR1: - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session - Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

- Discussion

Agreement in RAN1 #118bis meeting To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values: - M=1, L= {4,6,8} - M=2, L= {8,12,16,24} - M=4, L= {16,24,32,56} - FFS whether one or more than one L value (s) applied to each of M values M=1,2,4 - FFS the L values applied to other applicable SCS(s) - FFS the L values if other M values in addition to M=1,2,4 are supported. - FFS whether the time estimation averaged across multiple LP-SS occasions is applied - Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS) - Note 2: This doesn’t preclude any of three agreed sequence types Above applies at least for both 15kHz and 30kHz.

Based on the latest RAN1 progress as shown above, at least 4 OFDM symbols are required for LP-SS sequences. And we simulated the following candidate combinations {M, L} for LP-SS sequences:

- OOK-4 M=1, LP-SS sequence length L=4, i.e. 4 OFDM symbols

- OOK-4 M=1, LP-SS sequence length L=8, i.e. 8 OFDM symbols

- OOK-4 M=2, LP-SS sequence length L=8, i.e. 4 OFDM symbols

- OOK-4 M=2, LP-SS sequence length L=16, i.e. 8 OFDM symbols

- OOK-4 M=4, LP-SS sequence length L=16, i.e. 4 OFDM symbols

- OOK-4 M=4, LP-SS sequence length L=32, i.e. 8 OFDM symbols

In our simulation, the SINR = -3dB is considered, with both 9dB and 6dB power difference between the serving cell and the interference cell. The simulation results are summarized in Table-1 and Table-2 respectively, and the related figures could be found in Annex.

Observation 1: With at least 4 OFDM symbols for LP-SS, the RSRP measurement could achieve 3.5 accuracy at SINR = -3dB.

Table 1: RSRP accuracy @ Es/Iot = -3dB with 9dB power difference (SNR = [-2.7, -11.7] dB)

| Channel | AWGN |  |  |  | TDL-C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of samples | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| OOK-4 M=1 length L=4 | 2.36 | 1.9 | 1.7 | 1.6 | 3.32 | 2.65 | 2.32 | 2.16 |
| OOK-4 M=1 Length L=8 | 1.99 | 1.71 | 1.59 | 1.48 | 2.95 | 2.42 | 2.12 | 1.98 |
| OOK-4 M=2 length L=8 | 1.61 | 1.31 | 1.14 | 1.06 | 2.19 | 1.74 | 1.52 | 1.4 |
| OOK-4 M=2 length L=16 | 1.45 | 1.19 | 1.1 | 1.05 | 1.87 | 1.51 | 1.33 | 1.26 |
| OOK-4 M=4 length L=16 | 1.49 | 1.19 | 1.06 | 0.99 | 2.64 | 2.13 | 1.82 | 1.69 |
| OOK-4 M=4 length L=32 | 1.31 | 1.09 | 1.01 | 0.95 | 2.27 | 1.84 | 1.64 | 1.51 |

Table 2: RSRP accuracy @ Es/Iot = -3dB with 6dB power difference (SNR = [-2.4, -8.4] dB)

| Channel | AWGN |  |  |  | TDL-C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. of samples | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| OOK-4 M=1 length L=4 | 2.34 | 1.91 | 1.69 | 1.6 | 3.38 | 2.63 | 2.31 | 2.14 |
| OOK-4 M=1 Length L=8 | 1.98 | 1.72 | 1.59 | 1.49 | 2.99 | 2.41 | 2.12 | 1.97 |
| OOK-4 M=2 length L=8 | 1.58 | 1.31 | 1.16 | 1.06 | 2.19 | 1.77 | 1.55 | 1.39 |
| OOK-4 M=2 length L=16 | 1.43 | 1.21 | 1.11 | 1.04 | 1.87 | 1.52 | 1.35 | 1.26 |
| OOK-4 M=4 length L=16 | 1.50 | 1.20 | 1.05 | 1.0 | 2.60 | 2.11 | 1.80 | 1.69 |
| OOK-4 M=4 length L=32 | 1.31 | 1.10 | 1.02 | 0.96 | 2.24 | 1.82 | 1.67 | 1.51 |

- Conclusion

This contribution gave some simulation results for LP-RSRP measurement and the following observation.

Observation 1: With at least 4 OFDM symbols for LP-SS, the RSRP measurement could achieve 3.5 accuracy at SINR = -3dB.

References

[1] R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo, RAN4 #111

[2] R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo, RAN4 #112

[3] R4-2416861, WF on RRM requirements for NR_LPWUS, vivo, RAN4 #112bis

Annex

| AWGN channel | TDL-C channel |
| --- | --- |

Figure-1, RSRP accuracy @ Es/Iot = -3dB (SNR = [-2.7, -11.7] dB)

| AWGN channel | TDL-C channel |
| --- | --- |

Figure-2, RSRP accuracy @ Es/Iot = -2dB (SNR = [-2.4, -8.4] dB)