3GPP TSG-RAN WG4 Meeting #113	R4-2418801

# Orlando, United States,18 – 22 Nov., 2024

Source: 	Huawei, HiSilicon

Title:	On Ambient IoT general aspects

Agenda Item:	7.21.1

Document for:	Approval

1	Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. A WF was agreed last meeting [2]. This contribution we discuss the open issues and provide a text proposal for TR 38.769 on Ambient IoT system parameters based on endorsed TP [3].

2	Discussion

Based on the WF, for Ambient IoT system parameter, the TBD requirements include channel spacing for standalone deployment, channel raster for device and intermediate UE. [2]

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device | Intermediate UE |
| Operating band |  | YES |  |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |  |
|  | channel bandwidth | YES，as RAN4#112 agreed |  |  |
| Channel arrangement | Channel spacing | TBD for standalone NA for in-band/Guard-band |  |  |
|  | Channel raster | YES | TBD | TBD |
|  | Synchronization raster | NO for device 1/2a，as RAN4#112 agreed |  |  |

## 2.1	Spectrum usage

In RAN1#108bis, it was agreed that the spectrum usage is as follows: [3]

Update the D2R spectrum and R2D spectrum columns in Table 4.2.1-1 of TR38.769 as follows,

| Scenario | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- |
| D1T1-A1 | Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL) | Same as CW | At least DL |
| D1T1-A2 | Same as D1T1-A1 | Same as CW | At least DL |
| D1T1-B | Case 1-4 (outside topology, UL) | Same as CW | At least DL |
| D1T1-C | N/A | UL | At least DL |
| D2T2-A1 | Case 2-2 (inside topology, UL) | Same as CW | UL |
| D2T2-A2 | Same as D2T2-A1 | Same as CW | UL |
| D2T2-B | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW | UL |
| D2T2-C | N/A | At least UL | UL |

A simple workflow of the Ambient IoT system can be described as follows:

1）RF  Energy power  is provided for a device.

2）Reader sends configuration information to the CW.

3）After that, the reader sends R2D signal to a device that falls within its coverage area.

4）The CW then sends the carrier wave to the device within its coverage area,

5） the Device scatters signals from the CW.

6）Finally, the reader receives the device backscatter signal that carries uplink information.

From the above workflow, we can see that Readers can be turned on and off, as does the CW. The R2D transmitting is non-concurrent with the CW transmitting. Device 1 and 2a cannot backscatter signal until they receive the CW. After the CW is transmitted, device1 and 2a backscatters the D2R signal to the Reader. There is a slight time delay between the CW2D transmission and the R2D backscattering signal receiving. It should be carefully considered the switching performance of the Ambient IoT system.

Observation 1: The R2D transmitting is non-concurrent with the CW transmitting.

Observation 2: There is a slight time delay between the CW2D transmission and the R2D backscattering signal receiving.

Proposal 1: While the Ambient IoT system operates in the FDD band, it should be carefully considered the switching performance of the Ambient IoT system.

## 2.2	Channel bandwidth

The R2D waveform is agreed to be OFDM-based OOK waveform. One or multiple PRBs are under consideration in RAN1.

The R2D channel bandwidth can be defined as one or multiple PRBs with guardband if necessary. The value for this can be determined during the WI phase.

For D2R by backscattering, the waveform is provided by the inside or outside carrier wave, and is non-OFDM. For D2R channel, the transmission bandwidth of 15kHz or more is being discussed. The D2R channel bandwidth can be defined as 15kHz or more with guardband if it is necessary. The value for this can be determined during the WI phase.

## 2.3	Channel spacing

For devices with RF-ED receivers, the channel spacing is unnecessary as the device cannot distinguish between different channels.

On the other hand, for Ambient IoT BS and intermediate UE, channel spacing may be needed for deployment purpose, for example, to co-deploy with existing NR network.

## 2.4	Channel raster

For devices with RF-ED receivers, the channel raster is unnecessary as the device cannot distinguish between different channel raster.

On the other hand, for Ambient IoT BS and intermediate UE, channel raster may be needed for deployment purpose, for example, to co-deploy with existing NR network.

In summary, we propose that:

Proposal 2: For the following system parameters, we recommend:

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device | Intermediate UE |
| Channel arrangement | Channel spacing | NA for in-band/Guard-band YES for standalone | NA for in-band/Guard-band NO for standalone | NA for in-band/Guard-band YES for standalone |
|  | Channel raster | YES | NO | YES |

3	Conclusion

Based on the analysis in this contribution, we have the following observation and proposal:

Observation 1: The R2D transmitting is non-concurrent with the CW transmitting.

Observation 2: There is a slight time delay between the CW2D transmission and the R2D backscattering signal receiving.

Proposal 1: While the Ambient IoT system operates in the FDD band, it should be carefully considered the switching performance of the Ambient IoT system.

Proposal 2: For the following system parameters, we recommend:

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device | Intermediate UE |
| Channel arrangement | Channel spacing | NA for in-band/Guard-band YES for standalone | NA for in-band/Guard-band NO for standalone | NA for in-band/Guard-band YES for standalone |
|  | Channel raster | YES | NO | YES |

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

[2] R1-2409223 Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things) in NR，HW，RAN1#118bis，October，2024

[3] R4-2417193, A-IoT general overview, Ericsson, RAN4#112bis，October，2024

5	Text Proposal to TR 38.769

<Start of Text Proposal>

6.7.1	System parameters

6.7.1.1	Operating band

According to the SID (RP-240826), Ambient IoT uses FR1 licensed spectrum in FDD. NR band n8 can be used as an example band.

[Table 6.7.1.1-1 summarized the A-IoT system spectrum usage] It can be observed that The spectrum usage for A-IoT operation can be flexible, and it depends on several factors ,such as whether the reader is A-IoT BS  or intermediate UE, and whether CW transmission is deployed inside or outside topology. For the case where different spectrum is used for D2R and R2D, the FDD band definition can be kept. As CW transmission may interfere the R2D reception, the R2D and D2R should be TDM and therefore, for FDD band definition, it should be half duplex mode.

[Table 6.7.1.1-1: Spectrum usage for Ambient IoT]

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | Comment |
| --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | [D1T1-A2, D1T1-A1] |
|  | DL spectrum | UL spectrum | UL spectrum | [D1T1-B] |
|  | UL spectrum | DL spectrum | DL spectrum | [D1T1-A2, D1T1-A1] |
|  | UL spectrum | UL spectrum | UL spectrum | [D1T1-B] |
| UE as reader | UL spectrum | UL spectrum | UL spectrum | [D2T2-A2] |
|  | UL spectrum | DL spectrum | DL spectrum | [D2T2-B] |

While the Ambient IoT system operates in the FDD band, it should be carefully considered the switching performance of the Ambient IoT system.

6.7.1.2	Channel bandwidth

The R2D waveform is agreed to be OFDM-based OOK waveform, therefore the channel bandwidth definition can follow the channel bandwidth definition in NR.  The R2D channel bandwidth can be defined as one or multiple PRBs with guardband if necessary. The value for this can be determined during the WI phase.

For D2R by backscattering, the waveform is provided by the inside or outside carrier wave, and is non-OFDM. For D2R channel, the transmission bandwidth of 15kHz or more is being discussed. The D2R channel bandwidth can be defined as 15kHz or more with guardband if necessary.The value for this can be determined during the WI phase.

6.7.1.3	Channel arrangement

For devices with RF-ED receivers, the channel spacing and channel raster are unnecessary as the device cannot distinguish between different channels.

On the other hand, for Ambient IoT BS and intermediate UE, channel spacing and channel raster may be needed for deployment purpose, for example, to co-deploy with existing NR network.

A summary for Ambient IoT system parameters is captured in the table below.

| System parameter for A-IoT |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | A-IoT BS | A-IoT device | Intermediate UE |
| Operating band |  | YES |  |  |
| Channel bandwidth | Transmission bandwidth | YES the value needs be discussed in WI phase depending on other WG conclusion. |  |  |
|  | channel bandwidth | YES |  |  |
| Channel arrangement | Channel spacing | NA for in-band/Guard-band [YES for standalone] | NA for in-band/Guard-band [NO for standalone] | NA for in-band/Guard-band [YES for standalone] |
|  | Channel raster | YES | [NO[ | [YES[ |
|  | Synchronization raster | [To be discussed in WI phase] | NO for device 1/2a | [To be discussed in WI phase] |

<End of Text Proposal>