3GPP TSG-RAN WG4 Meeting #113	R4-2418803

# Orlando, United States,18 – 22 Nov., 2024

Source: 	Huawei, HiSilicon

Title:	On the RF requirements for Ambient IoT BS

Agenda Item:	7.21.3.1

Document for:	Endorsement

1	Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. A WF on the RF requirements for Ambient IoT was agreed last meeting [2]. This contribution we discuss the open issues and propose a text proposal for TR 38.769 on Ambient IoT BS.

2	Discussion

During the A-IoT study, some companies expressed interest in scenarios which A-IoT BS and legacy NR BS share hardware or co-locate.

However, this could cause interference issues. Ambient IoT devices, specifically Device 1 and Device 2a, may have a significant sensitivity difference compared to legacy NR UEs. The interference signal from R2D can reach levels of about -36dBm, while the blocking level for the NR UE receiver is -56dBm. This means that some NR UE receivers may be blocked by the Ambient IoT R2D signal, while a few NR UE that receive strong NR signals and are not blocked by the Ambient IoT BS may interfere with the Ambient IoT BS receiver. For hardware sharing, another problem is that initial simulations show that a PSD (power spectral density) difference of more than 20 dB is needed to avoid affecting envelope detection demodulation, which is challenging compared to the 6dB power boosting of NB-IoT.

Based on these considerations, it is recommended to focus on non-collocated scenarios in Release 19, with legacy NR gNBs outdoors and Ambient IoT BSs indoors.

Proposal 1: Focus on non-collocated scenarios in Release 19, and hardware sharing is not prioritized because of the significant difference in PSD and interference.

## 2.1	TX

Based on the WF [2], for Ambient IoT BS, the TX TBD requirements include RE power control dynamic range, total power dynamic range, transmitter OFF power, transmitter transient period and transmitter intermodulation.

RE power control dynamic range and total power dynamic range are applied to legacy constellation modulation, which are not applicable to OOK signal for Ambient IoT BS.

In current BS specification, transmit OFF power and transmitter transient period are defined. It is a TDD requirements and not applicable to Ambient IoT BS.

For the case BS generates R2D and CW signal in the sequential way in D1T1-A1 and D1T1-A2, the switch is an implementation issue and no need to define it as a minimal RF requirement.

For the transmit ON/OFF time mask, refer to the RFID specification, a settling time requirement is used to assess signal ripple characteristics. It can be determined whether or not this settling time is necessary in WI phase.

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. for indoor deployment. For the same band it is not possible to have co-location transmitter since it will have interference to the device on the RF-ED. Hence no need to define transmitter intermodulation for Ambient IoT BS.

Proposal 2: For A-IoT BS- TX requirements, we recommend:

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Output power dynamic | RE power control dynamic range | NO |
|  | Total power dynamic range | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | NO |
|  | Transmitter transient period | Discussed in the WI phase |
| Transmitter intermodulation |  | NO |

## 2.2	RX

Based on the WF [2], the TBD Ambient IoT BS RX requirement is In-channel selectivity.

For NR and LTE, in-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density. For Ambient IoT, if FDM operation is supported, ICS may be needed. It can be discussed in the WI phase.

Proposal 3: For A-IoT BS- RX requirements, we recommend:

| A-IoT BS- RX RF Requirement | Potentially needed or not |
| --- | --- |
| In-channel selectivity | Discussed in the WI phase |

3	Conclusion

Based on the analysis in this contribution, we have the following proposals:

Proposal 1: Focus on non-collocated scenarios in Release 19, and hardware sharing is not prioritized because of the significant difference in PSD and interference.

Proposal 2: For A-IoT BS- TX requirements, we recommend:

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Output power dynamic | RE power control dynamic range | NO |
|  | Total power dynamic range | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | NO |
|  | Transmitter transient period | Discussed in the WI phase |
| Transmitter intermodulation |  | NO |

Proposal 3: For A-IoT BS- RX requirements, we recommend:

| A-IoT BS- RX RF Requirement | Potentially needed or not |
| --- | --- |
| In-channel selectivity | Discussed in the WI phase |

4	References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

- R4-2417088, WF on impacts of Ambient IoT on RF requirements, Moderator (Huawei), RAN4#112bis, Oct.,2024

5	Text Proposal to TR 38.769

<Start of Text Proposal>

6.7.2	Ambient IoT BS

In D1T1 deployment scenarios, Ambient-IoT BS belongs BS type 1-C, and its BS class is Micro-BS.

The scenarios which A-IoT BS and legacy NR BS share hardware or co-locate could cause interference issues. Ambient IoT devices, specifically Device 1 and Device 2a, may have a significant sensitivity difference compared to legacy NR UEs. The interference signal from R2D can reach levels of about -36dBm, while the blocking level for the NR UE receiver is -56dBm. This means that some NR UE receivers may be blocked by the Ambient IoT R2D signal, while a few NR UE that receive strong NR signals and are not blocked by the Ambient IoT BS may interfere with the Ambient IoT BS receiver.

For hardware sharing, another problem is that initial simulations show that a PSD (power spectral density) difference of more than 20 dB is needed to avoid affecting envelope detection demodulation, which is challenging compared to the 6dB power boosting of NB-IoT.

Based on these considerations, it is recommended to focus on non-collocated scenarios in Release 19, with legacy NR gNBs outdoors and Ambient IoT BSs indoors.

6.7.3.1 TX

- Base station output power

BS output power is a key parameter for coverage. In existing NR specification, the rated carrier output power of the BS type 1-C is 38 dBm for MR BS, which is characterised by requirements derived from Micro Cell scenarios. This can be applicable to Ambient IoT BS.

- Output power dynamics

In current legacy NR BS specification, the general output power dynamic range requirements include RE power control dynamic range and total power dynamic range. Power boosting for NB-IoT operation in NR in-band is also defined.

RE power control dynamic range and total power dynamic range are applied to legacy constellation modulation, which are not applicable to OOK signal for Ambient IoT BS.

For power boosting, it is only applicable when Ambient IoT BS and NR BS share the same hardware. Initial simulation showed that power boosting need be larger than 20 dB for device 1 and 2a to decode the R2D signal within certain conditions. But we know current specification define a 6dB power boosting for NB-IoT in-band operation. It is challenge to implementation. The power boosting for BS support both Ambient IoT and NR can be discussed in future release.

- Some requirements like Transmitter ON/OFF power

In current BS specification, transmit OFF power and transmitter transient period are defined. It is a TDD requirements and not applicable to Ambient IoT BS.

For the case BS generates R2D and CW signal in the sequential way in D1T1-A1 and D1T1-A2, the switch is an implementation issue and no need to define it as a minimal RF requirement.

For the transmit ON/OFF time mask, refer to the RFID specification, a settling time requirement is used to assess signal ripple characteristics. It can be determined whether or not this settling time is necessary in WI phase.

- Transmitted signal quality

In current BS specification, transmitted signal quality includes Frequency error, EVM (Modulation quality) and TAE.

For NR BS, frequency error for both MR BS and LA BS is ±0.1 ppm. It can be  considering that adopt ±0.1 ppm frequency error for Ambient IoT BS.

EVM requirement for legacy NR BS cannot be applied to Ambient IoT BS, as OOK signal is not mapped to the legacy constellation. As an alternative, the RF envelope metric used by RFID can be used as a reference to ensure modulation quality.

TAE requirement is not applicable for Ambient IoT BS.

- Unwanted emissions

Unwanted emissions for Ambient IoT BS include occupied bandwidth, ACLR, Operating band unwanted emissions and transmitter spurious emissions.

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 (=0.5%) of the total mean transmitted power. Occupied bandwidth: Potentially needed for Ambient IoT BS. The specific value is dependent on the physical layer design of R2D, which has not yet been finalized at this stage.

ACLR can be defined based on the final RAN1 design and co-existence study.

Operating band unwanted emissions and existing transmitter spurious emission requirement for legacy NR BS can be applicable, since this is coming from the ITU-R recommendation.

- Transmitter intermodulation

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. for indoor deployment. For the same band it is not possible to have co-location transmitter since it will have interference to the device on the RF-ED. Hence no need to define transmitter intermodulation for Ambient IoT BS.

A summary for TX part requirements is captured in the table below.

| RF Requirement for Ambient IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | [NO] |
|  | Total power dynamic range | [NO] |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | [NO] |
|  | Transmitter transient period | [Discussed in the WI phase] |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
|  | TAE | NA |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES, |
| Transmitter intermodulation |  | [NO] |

6.7.3.2 RX

- Reference sensitivity level

The residual noise from CW interference is higher than the noise floor. So it should be considered when defining receiver sensitivity. Reference sensitivity shall be specified according following formula:

PREFSENS(dBm)=-174dBm+10*log10BW+NF+IM+ICW+SNR

Where:

-	BW is the maximum transmission bandwidth

-	NF is BS noise figure, equal to 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2dB.

-	Icw is IoT due the residual CW interference, value will be defined in the WI.

-	SNR is the SNR value for which we reach 10% BLER target. Each company provided simulation results, and average will be done for each BW.

- Dynamic range

For NR and LTE, receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. The interference level is derived from simulation of co-channel interference from the same operator. It measures the effects of base station receiver impairments. For AMBIENT IOT indoor deployment, the co-channel interference comes from co-channel device transmission and CW interference. The necessity and the value can be studied at WI phase.

- Adjacent Channel Selectivity

ACS requirement and the interference level depend on co-existence study.

The value can be studied in the WI phase.

- In-band blocking

In-band blocking for legacy NR BS includes general blocking and narrowband blocking.

In-band blocking need more study after ACS is defined. It is potentially needed for Ambient IoT BS.

- Out-of-band blocking

Out of band blocking is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking. Existing out-of-band blocking requirement may be applicable.

- Receiver spurious response

Existing RX spurious emission requirement can be applicable.

- Receiver intermodulation

Receiver intermodulation for legacy NR BS includes General intermodulation and Narrowband intermodulation.

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. For AMBIENT IOT deployment, the typical case is that there in only one operator. Hence there is may no such an interference scenario. It can be further discussed at WI phase.

- In-channel selectivity

For NR and LTE, in-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density. For Ambient IoT, if FDM operation is supported, ICS may be needed.

A summary for RX part requirements is captured in the table below.

| Ambient IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES |
| Dynamic range |  | Discussed in the WI phase |
| Adjacent Channel Selectivity |  | YES |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES |
| Receiver intermodulation | General intermodulation | Wait for RAN1 conclusion. |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | [Discussed in the WI phase] |

<End of Text Proposal>