3GPP TSG-RAN WG4 Meeting#113		R4-2418842

Orlando, United States, Nov 18th -22nd, 2024

Agenda item:	7.1.1.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on SRS antenna switching requirements for 6RX UE

Document for:	Discussion

Introduction

In RAN4#112bis meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

ΔTRxSRS values

Issue 2-2-1: Proposed ∆TRxSRS values for t1r6, t2r6, and t1r6-t2r6 Agreement: RAN4 agrees to adopt the values for Bands n79 and n104 as specified below. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Band n79; 5.5; 5.0; 6.0. Band n104; 6.5; 5.5; 7.0. Issue 2-2-2: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Agreement for RAN4#112bis: As this is the first RAN4 meeting since the confirmation that RAN1 will consider t3r6, companies are encouraged to provide ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 at RAN4#113 for further discussion with the Apple and Qualcomm proposals in R4-2415324 and R4-2416264, respectively.

A conclusion was reached at RAN-P #104 meeting that RAN1 should study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. SRS antenna switching to support 4T6R is precluded. In RAN4#111 and RAN4#112 meeting, ΔTRXSRS value for 1T6R, 2T6R, 1T6R-2T6R for PC3 has reached agreement. Hence, we need to discuss ΔTRXSRS value for 3T6R, 1T6R-3T6R, 2T6R-3T6R, and 1T6R-2T6R-3T6R. It is observed that 3T6R can be divided into three1T2R. Thus we can get the value of ΔTRXSRS for 3T6R is 3 dB. For 1T6R-2T6R-3T6R, we can see the difference with 1T6R (in Figure1 and Figure2 in Annex) is the length of SP4T and the length of PCB (0.5dB), thus we can get ΔTRXSRS of 1T6R-2T6R-3T6R is 5.5dB. 1TR6-3T6R and 2TR6-3T6R is similar with 1T6R-2T6R, thus we can get ΔTRXSRS of 1TR6-3T6R and 2TR6-3T6R is 4.5dB. ΔTRXSRS of 1T6R is 4.0dB and ΔTRXSRS of 1T6R-2T6R is 4.5dB based on the RAN4#112 meeting agreements. For n79, we can add 1.5dB based on n41/n77/n78. However, an additional breakpoint is introduced for n104, we can add 1.0dB based on n79.

Proposal 1: Adopt the value of ΔTRXSRS in Table 1 for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

Table 1: ΔTRXSRS for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

| Operating bands | ΔTRXSRS 3T6R(dB) | ΔTRXSRS 1T6R-3T6R(dB) | ΔTRXSRS 2T6R-3T6R(dB) | ΔTRXSRS 1T6R-2T6R-3T6R(dB) |
| --- | --- | --- | --- | --- |
| N41/N77/N78 | 3 | 4.5 | 4.5 | 5.5 |
| N79 | 4.5 | 6 | 6 | 7 |
| N104 | 5.5 | 7 | 7 | 8 |

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: Adopt the value of ΔTRXSRS in Table 1 for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

Table 1: ΔTRXSRS for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

| Operating bands | ΔTRXSRS 3T6R(dB) | ΔTRXSRS 1T6R-3T6R(dB) | ΔTRXSRS 2T6R-3T6R(dB) | ΔTRXSRS 1T6R-2T6R-3T6R(dB) |
| --- | --- | --- | --- | --- |
| N41/N77/N78 | 3 | 4.5 | 4.5 | 5.5 |
| N79 | 4.5 | 6 | 6 | 7 |
| N104 | 5.5 | 7 | 7 | 8 |

- Reference

- R4-2417209, WF on requirements for 6RX, AT&T.TSG-RAN4#112bis meeting.

- Annex

Figure 1: an example of 1T6R architecture                         Figure 2: an example of 1T6R-2T6R-3T6R architecture