3GPP TSG-RAN WG4 Meeting #113	R4-2418856

Orlando, US, 18th – 22nd November, 2024

Source:	China Telecom

Title:	TP to 37.887 General part on the note for DL interruption for dynamic Tx switching

Agenda Item:	6.10.2

Document for:	Approval

- Background

This contribution provides a TP for TR 37.887 to add General part on the note for DL interruption for dynamic Tx switching, based on the approved WF [1] in last RAN4 meeting.

- Ref

[1] R4-2417057, WF on notes for downlink interruption clarification, China Telecom

- Text Proposal

<Start of Text Proposal for TR 37.887>

# 5			Specific Band Combinations

## 5.0	Criterias

Considering the following criterias when deciding to mandate no DL interruption applicability for TX switching among 3 or 4 bands

> 	Frequency separation between the DL band and the bands involved in the Tx switching for FDD+TDD band combinations and the corresponding higher order combinations

> 	Whether the same UL-DL duplex mode is used for TDD+TDD band combinations and - the corresponding higher order combinations

-	Whether simultaneous RX/TX is mandated between the DL band(s) and the UL band(s)

>	Whether same antenna group is used for band combinations and the corresponding higher order combinations

-	Number of TX chains to be switched

Using the formats and the notes in Table 5.0-1 and 5.0-2 to introduce mandating no DL interruption clarification for Tx switching across up to 4 bands in TS 38.101-1[2]

Table 5.0-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band | DL interruption allowed (Note 1) |
| --- | --- | --- |
| CA_nX-nY-nZ | nX, nY, nZ | No for nX-nY |
| NOTE 1: Applicable when dynamic Tx switching is triggered across any 2 UL bands among the configured 2, 3 or 4 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The nX-nY in the requirement of “No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching. |  |  |

Table 5.0-2: Inter-band CA operating bands involving FR1 (four bands)

| NR CA Band | NR Band | DL interruption allowed (Note 1) |
| --- | --- | --- |
| CA_nX-nY-nZ-nK | nX, nY, nZ, nK | No for nX-nY |
| NOTE 1: Applicable when dynamic Tx switching is s triggered across any 2 UL bands among the configured 2, 3 or 4 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. The nX-nY in the requirement of “No for nX-nY” refers to the two UL Bands nX and nY that are involved in dynamic Tx switching. |  |  |

<end of TP >