3GPP TSG RAN WG4 Meeting#113                                    R4-2418858

Orlando, US, 18th - 22nd November 2024

Title:	RRM Core and performance requirements on Further NR mobility enhancements

Source:	China Telecom

Agenda item:			5.23.1

Document for:	Discussion

- Introduction

In the RAN4 #112 meeting, the L1/L2 based inter-cell mobility were discussed and WF [1] has been approved. In this paper, some issues are further discussed.

- Discussion

Issue 1-4-5-1: Whether are cell switch delay requirements applicable to FR2 without L1 measurement report?

< Way Forward > FFS the following options: - Option 1 (OPPO): - Yes when SNR of the TCI state≥ -3dB and TCI state has been activated, it is also required that the RS associated to the target TCI state is available at least every 1280ms after TCI state activation command. - Option 2 (Nokia): Yes with the following conditions: - The target TCI state in the cell switch command is activated not more than 1280 ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or - The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every 1280 ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB - Option 3 (Ericsson, QC): Yes with the following conditions: - [- The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or] - [- The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or] - Option 4 (MTK): - Not to extend cell switch delay requirements to FR2 without L1 measurement report

For this issue, we have extended cell switch delay requirements to FR1 without L1 measurement report with minor modification compared to previous version. But for FR2, more details need to be discussed, we prefer option4.

Proposal 1: Not to extend cell switch delay requirements to FR2 without L1 measurement report.

LS on the fast RRC processing for LTM

- As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? - Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? - Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

For question 2, we think UE does not need to know the number of SpCells and SCells.

For question 3, we think the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations could not exceed the UE reported maxNumberStoredConfigCells

Proposal 2:

- UE does not need to know the number of SpCells and SCells.

- The number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations could not exceed the UE reported maxNumberStoredConfigCells

- Summary

In this paper, we provide our views on L1/L2 based inter-cell mobility. From this discussion we have derived the following proposals:

Proposal 1: Not to extend cell switch delay requirements to FR2 without L1 measurement report.

Proposal 2:

- UE does not need to know the number of SpCells and SCells.

- The number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations could not exceed the UE reported maxNumberStoredConfigCells

- References

[1] R4-2413960, WF on Further NR mobility enhancements, MediaTek Inc.