3GPP TSG RAN WG4 Meeting#113                                    R4-2418859

Orlando, US, 18th - 22nd November 2024

Title:	RRM Core requirements on event triggered L1 measurement reporting for LTM

Source:	China Telecom

Agenda item:			7.24.2.1

Document for:	Discussion

- Introduction

In the RAN4 #112bis meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on event triggered L1 measurement reporting for LTM.

- Discussion

Issue 1-5: filtering assumption in event evaluation

- Candidate solutions: - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT). - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC) - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected. - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline. - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC)

For event triggered SSB based L1 measurement and CSI-RS based L1 measurement, the related requirements on filtering is shown as follows

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected The event triggered measurement reporting delay, measured without L3 filtering shall be less than the CSI-RS based measurement defined in clause 9.10.2.5. When L3 filtering is used an additional delay can be expected.

However, RAN2 only assumes the filtering at present, it is necessary to wait for the conclusion of RAN1. As we think, it is possible that the filtering is up to UE implementation. So, we think it is better to wait for the conclusion of RAN1.

Proposal 1: For filtering, we think it is better to wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report.

Issue 1-7: beam level or cell level measurement result for new LTM event evaluation

- Candidate solutions: - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung)

In previous RAN2 meetings, some conclusions have been reached on event trigger L1 measurement report. It should be designed for selecting candidate beam(s)/cell(s) to trigger early synchronization and selecting target beam/cell to trigger LTM cell switch procedure.

Usually, L1 measurement is used to beam level, so we think we can discuss the requirements for the case where use beam level measurement result for event evaluation.

Proposal 2: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

Issue 1-9: L1 event triggered reporting delay

- Candidate solutions: - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE) - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi) - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS. - Option 3: (CMCC) - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements. - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple) - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung) - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia) - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

RAN2 has introduced the event triggered L1 measurement reporting is introduced in RAN2, it is necessary to define related measurement reporting requirements.

We have defined the measurement reporting requirements for event triggered reporting of SSB based L3 measurements and CSI-RS based L3 measurements. And it is copied as follow,

Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). Reported CSI-RSRP, CSI-RSRQ, and CSI- SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.3, 10.1.3.3, 10.1.7.2, 10.1.8.2, 10.1.12.2 and 10.1.13.2. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. If a cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra

So, we think the existing framework in L3 event triggered reporting RRM requirements can be as a starting point.

For L1 event triggered report, we think PSS/SSS detection and the index of the SSB is already known. So, the Tidentify intra without index or Tidentify intra with index and TSSB_time_index_intra could be excluded. In this case, the event triggered measurement reporting delay shall be less than L1-RSRP measurement period based on SSB [or CSI-RS].

Proposal 3: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point

Proposal 4: The event triggered measurement reporting delay shall be less than L1-RSRP measurement period based on SSB [or CSI-RS].

Issue 1-10: where to capture event triggered L1 report requirement

- Candidate solutions: - Option 1: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. (CATT, Apple, vivo, HW)

We are ok with option1, so new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency.

Proposal 5: New requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: For filtering, we think it is better to wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report.

Proposal 2: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

Proposal 3: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point

Proposal 4: The event triggered measurement reporting delay shall be less than L1-RSRP measurement period based on SSB [or CSI-RS].

Proposal 5: New requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency.

- References

[1] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.