3GPP TSG-RAN WG4 Meeting #112bis	R4-2418884

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Remaining aspects for NR_MG_enh2

Agenda item:	5.12.1

Document for:	Discussion

Introduction

This paper presents Nokia’s views on measurements without gaps in Rel 18, in particular regarding behaviour of DRX ON misalignment in relation to SMTC window.

Paper highlights

As part of this paper the following is identified and proposed

- The possible network configurations that avoid interruptions during DRX ON are limited if the distance between DRX ON duration and SMTC window has to be larger than 4 ms to avoid interruptions during DRX ON.

- It is proposed to reduce the minimum distance between SMTC occasions and DRX ON, to make network configuration more flexible.

Discussion

In the last RAN4 meeting, the following was agreed regarding DRX [1]

### Sub-topic 4-1 DRX Issue 4-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements - Background (agreement): - Interruption ratio requirement not based on DRX-on duration - Not define the interruption location Agreement: - Interruptions are not allowed during DRX ON duration - Exclude the time extended due to drx-inactivityTimer - if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration.

Table 1: WF from RAN4#112 on DRX.

An example of this agreement for SMTC with 20ms periodicity is shown in Figure 1.

Figure 1: Example of agreed solution for DRX-ON placement in when the SMTC repetition period is 20ms.

If the DRX-ON period is with the suggested [4ms] periods around an SMTC occasion, interruptions are allowed during the DRX-ON duration.

As seen, this leaves a 7ms period to place the DRX-ON period, which should be spread among the population of UE’s to spread the PDCCH occasions between UE’s.

For a DRX-on period larger than 7ms, this basically allows interrupts during DRX-ON, which could be the case for larger DRX cycle lengths, fx used for browsing. An example is shown in Figure 2.

Figure 2: Example of DRX configuration with 80ms cycle length and 10 ms DRX-ON duration.

If the SMTC period is a typical 20ms, the agreement does not allow a lot of room to place DRX-ON, distributing PDCCH among the population.

DRX-ON period above 7ms basically allows interruptions during DRX-ON always.

For shorter DRX ON periods, for example used for voice calls, for example 3ms, will only allow placing the DRX-ON period on (7-3/20) ~ 20% of the time axis, if the network would like to avoid the risk of interruptions during DRX-ON, which is a good idea for voice calls.

Therefore, Nokia believes the time before and after the DRX-ON period in which the presence of an SMTC will allow interruption during DRX-ON should be as short as possible. At the same time, it does not make much sense to place interruptions far from SMTC’s.

Revise the value of the time period before and after the DRX-ON cycle for which the presence of SMTC allows interruptions during the DRX-ON period to 1 ms.

If the value remains large, it could be considered extending the DRX-ON period, allowing more scheduling opportunities if collisions occur.

RAN4 to consider methods to extend the DRX-ON period if collisions occur, if the time period before and after the DRX-ON period is large, for example 4ms, for when to allow interruptions during the DRX-ON period.

It may be that some devices are able to avoid interruptions during PDCCH occasions. It would be a great benefit for the network to know this capability, if possible.

RAN4 to consider allowing devices not causing interruptions during PDCCH occasions to share this capability with the network to allow greater flexibility in the DRX configurations.

Conclusion

The paper presents Nokia’s views regarding interruption requirements for measurements without gaps when DRX is configured. As part of the discussion the following observations and proposals are derived:

Observation 1: If the SMTC period is a typical 20ms, the agreement does not allow a lot of room to place DRX-ON, distributing PDCCH among the population.

Observation 2: DRX-ON period above 7ms basically allows interruptions during DRX-ON always.

Proposal 1: Revise the value of the time period before and after the DRX-ON cycle for which the presence of SMTC allows interruptions during the DRX-ON period to 1 ms.

Proposal 2: RAN4 to consider methods to extend the DRX-ON period if collisions occur, if the time period before and after the DRX-ON period is large, for example 4ms, for when to allow interruptions during the DRX-ON period.

Proposal 3: RAN4 to consider allowing devices not causing interruptions during PDCCH occasions to share this capability with the network to allow greater flexibility in the DRX configurations.

- References

- R4-2414109, WF on RRM requirements for NR_MG_enh2, MediaTek, Maastricht, NL, 19-23 August 2024