3GPP TSG-RAN WG4 Meeting #113	R4-2418888

Orlando, US, 18-22 November 2024

Source:	Nokia

Title:	On L3 beam sweeping factor reduction with multi-Rx

Agenda item:	7.15.2.1

Document for:	Discussion

Introduction

In this contribution we continue the discussion about L3 beam sweeping factor optimization for FR2 with multi-Rx based on the RAN4#112bis meeting way forward [1].

Discussion

In the last meeting the following agreement was made for the framework of FR2 beam sweeping factor reduction with multi-Rx:

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Criteria/conditions to trigger FBS for scenario 1-4

RAN4 agreed that network configures the conditions to trigger FBS through RRC. Conditions to be used remained for further study. It has been proposed by companies at least to reuse L3 event-triggered reporting conditions or RLM/BFD relaxation conditions.

As we have proposed in the previous meetings, we think it would be simple and straightforward to reuse the conditions for event-triggered L3 reporting. Here the conditions that would tell about serving cell quality would be:

- Event A1 (Serving becomes better than threshold)

- Event A2 (Serving becomes worse than threshold)

Event A2 can be used to tell that the UE has moved to the cell edge or cell edge like conditions. Event A1 would then tell that the UE is no longer in cell edge or cell edge like conditions. We think either event A2 alone or a combination of event A1 and A2 can be used to define when the UE is required to do FBS. The approach can be for example:

- Event A2 triggers FBS and if event A2 condition is no longer fulfilled, the UE can return back to normal beam sweeping.

- Event A2 triggers FBS. Event A1 allows the UE to return back to normal beam sweeping.

When event-triggered reporting conditions are used for FBS triggering, the network will also be aware of when the UE is doing FBS, as the UE will send a report to the network. Network also has the option to configure UE to send another report when the event condition is no longer met (through reportOnLeave), so even in the former option listed above the network would also know when the UE returns back to normal beam sweeping. Hence, no additional signalling to indicate which beam sweeping mode the UE is using would be needed.

When L3 event-triggered reporting condition is used to trigger FBS, the network knows based on the report that UE has moved to FBS mode.

Network knows based on event A1 or based on report on leave that the UE returns back to normal beam sweeping.

Use L3 event-triggered reporting conditions for triggering FBS.

Use at least event A2 to trigger FBS.

UE is allowed to return back to normal beam sweeping when event A2 is no longer met, or when event A1 is met, depending on network configuration.

Whether some type of additional signaling should be added for the network to inform the UE about the use of reporting events to trigger FBS is needed in the RRC configuration can in our view be discussed.

RAN4 to discuss whether any additional signaling would be needed from network side to indicate that a L3 reporting event is expected to trigger FBS, and send an LS to RAN2 based on the conclusion.

Earlier we have also proposed that the UE would be required to do FBS when conditional handover is configured to the UE. We think this proposal is still reasonable based on the RAN4#112bis agreements, and we continue to propose this.

Conditional handover configuration is used to trigger FBS.

Value for reduced BSF

Logically, if the assumption is that a Rel-19 multi-Rx capable UE would be using 2 Rx to perform measurements, the BSF should be reduced from 8 to 4. Hence, at least N = 4 should be supported. However, Rel-18 multi-Rx BSF reduction supports also values N = 2 and N = 6. RAN4 should discuss if these (or some other) values should be considered also for Rel-19 L3 BSF reduction. If more than one reduced BSF value are supported, then related capability signalling should be introduced.

At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported.

The need for requirements for transition period during the switch between FBS and non-FBS depends on how switching between these states is to happen. We do not think RAN4 needs to discuss this before agreeing about the conditions under which FBS applies.

How to capture the reduced BSF

Scenario 1-4 (measurement delay):

Beam sweeping is taken into account in the M-factors, where for PC3 the factor is defined as:

Mpss/sss_sync_w/o_gaps = 24

Mmeas_period_w/o_gaps = 24

Same values apply for intra- and inter-frequency measurements with and without gaps. The BSF 8 is hence merged in the M-values. When BSF is reduced, either the M value needs to be changed or an additional BSF reduction factor needs to be defined.

Scenario 5 (handover delay):

In handover requirements, beam sweeping is taken into account in the Tsearch component, where for an unknown intra- and inter-frequency target cell in FR3:

Tsearch = N* Trs ms (N=8)

Here the beam sweeping factor reduction can either be implemented similar to Rel-18 multi-Rx beam sweeping factor reduction by defining different values for N, or through an additional factor for a BSF reduction capable UE.

Scenario 7 (RRC re-establishment delay):

For RRC re-establishment, the impact of faster beam sweeping would apply for the definition of Tidentify_intra_NR and Tidentify_inter_NR, i. These are defined in tables 6.2.1.2.1-1 and 6.2.1.2.1-1.

Here RAN4 will need to discuss how to take into account BSF reduction in both values of the max-function. An additional factor for faster beam sweeping capable UE may be the simplest solution to capture the reduction.

If RAN4 wants to aim for a similar way to capture the beam sweeping factor reduction across the specification for all the applicable requirements, then we think an additional factor, such as Nreduced = NRel-19-BSF/8, where NRel-19-BSF is the UE reported N-factor, may be the clearest solution.

By applying beam sweeping factor reduction, depending on the number of simultaneous receptions (Nreduced < 1) of the UE i.e., the time for identifying for intra/inter cells for FR1 and FR2-1 will change as below:

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2-1 | N/A | MAX (1000 ms, Nreduced x 80 x TSMTC) |
|  |  | Tidentify_inter_NR, i [ms] |  |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (1000 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2-1 |  | MAX (1000 ms, Nreduced x 104 x TSMTC, i) |

However, for some combinations of TSMTC and Nreduced the max(.) function value might only consider the fixed value part e.g., if TSMTC=20 and Nreduced =1/2, the following values will be obtained from the above table:

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR2-1 | N/A | MAX (1000 ms, 800 ms) |
|  |  | Tidentify_inter_NR, i [ms] |  |
| ≥ -8 | FR2-1 |  | MAX (1000 ms, 1040ms) |

Thus, the current fixed values of the max(.) function are diminishing the impact of shorter delays achieved through “beam sweeping factor reduction”. In this regard it is beneficial to scale the fixed value of the max function with Nreduced as well.

The fixed value in the max(.) function overweight the reduced value of Tidentify_intra_NR and Tidentify_inter_NR obtained from BSF.

Thus, the new values of Tidentify_intra_NR and Tidentify_inter_NR could take the following format:

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR2-1 | N/A | MAX (Nreduced x 1000 ms, Nreduced x 80 x TSMTC) |
|  |  | Tidentify_inter_NR, i [ms] |  |
| ≥ -8 | FR2-1 |  | MAX (Nreduced x 1000 ms, Nreduced x 104 x TSMTC, i) |

To apply the scaling factor Nreduced to the fixed value of Tidentify_intra/inter_NR e.g., MAX (Nreduced x 1000 ms, Nreduced x 104 x TSMTC, i).

RAN4 to cover the impact of Rel-19 BSF reduction through an additional scaling factor across all supported requirements. This factor may depend on UE indicated N if multiple values for N are supported.

Such approach can also be applied to any other requirements impacted by the BSF reduction, if RAN4 agrees to support more than the agreed requirements for BSF reduction during the Rel-19 work item.

SSB processing delay

It has been brought up by some companies that a UE receiving multiple beams in a SMTC window will, due to UE hardware design, need additional time for processing the received SSBs. The SSB processing time is assumed to be 2 ms as in multiple legacy requirements. If UE receives 2 different beams in a SMTC, then it can process one SSB during/after the SMTC window (while receiving), and another SSB can be post-processed in the subsequent time period. Hence, when looking at the total cell identification delay, the additional SSB processing time would only need to be added once to the total delay to cover the processing of the last measurement.

If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as

  - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms

  - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms

  - where TSSB_processing = 2 ms

DRX

There has been discussion about whether FBS applies while the UE is in DRX mode. We do not see a need to distinguish between DRX and non-DRX for BSF reduction purposes. The reduction may equally well apply for both DRX and non-DRX.

Rel-19 BSF reduction is enabled independent of DRX status.

Conclusion

The following observations and proposals were made in this paper:

Observation 1: When L3 event-triggered reporting condition is used to trigger FBS, the network knows based on the report that UE has moved to FBS mode.

Observation 2: Network knows based on event A1 or based on report on leave that the UE returns back to normal beam sweeping.

Proposal 1: Use L3 event-triggered reporting conditions for triggering FBS.

Proposal 2: Use at least event A2 to trigger FBS.

Proposal 3: UE is allowed to return back to normal beam sweeping when event A2 is no longer met, or when event A1 is met, depending on network configuration.

Proposal 4: RAN4 to discuss whether any additional signaling would be needed from network side to indicate that a L3 reporting event is expected to trigger FBS, and send an LS to RAN2 based on the conclusion.

Proposal 5: Conditional handover configuration is used to trigger FBS.

Proposal 6: At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported.

Observation 3: The fixed value in the max(.) function overweight the reduced value of Tidentify_intra_NR and Tidentify_inter_NR obtained from BSF.

Proposal 7: To apply the scaling factor Nreduced to the fixed value of Tidentify_intra/inter_NR e.g., MAX (Nreduced x 1000 ms, Nreduced x 104 x TSMTC, i).

Proposal 8: RAN4 to cover the impact of Rel-19 BSF reduction through an additional scaling factor across all supported requirements. This factor may depend on UE indicated N if multiple values for N are supported.

Proposal 9: If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as

- Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms

- Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms

- where TSSB_processing = 2 ms

Proposal 10: Rel-19 BSF reduction is enabled independent of DRX status.

- References

[1] R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, Anhui, China, 14th – 18th October, 2024.