3GPP TSG-RAN WG4 Meeting #113	R4-2418889

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Other aspects for NR MIMO Phase 5 RRM requirements

Agenda item:	7.19.3.2

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the WID RP-242394, in particular about asymmetric DL sTRP/UL mTRP deployment scenarios (Objective 5) and the aspects related to CJT calibration reporting (Objective 3).

… - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH … - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Discussion on two TAs

Regarding asymmetric DL sTRP/UL mTRP with two TAs enhancement, the following was discussed in the last RAN4#112bis meeting [1]:

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

When considering the RRM requirements for single TA, we understand that the existing requirements also apply for asymmetric DL sTRP/UL mTRP deployment.

Existing RRM requirements with single TA apply for asymmetric DL sTRP/UL mTRP deployment.

Regarding the two TAs enhancement, RAN1 has made the following initial agreement:

Agreement (RAN1#118bis Hefei) Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Regarding the two TAs enhancement, it is worth recalling that Rel-18 introduced the two TAs enhancement for multi-DCI multi-TRP scenario, that allow UL transmissions from a UE toward two TRPs whose links may be characterized by a large difference in the propagation delays with respect to the CP length. The most relevant Rel-18 enablers are the following ones:

- Two TAGs can be configured for a serving cell configuration, each corresponding to a TRP.

  - To allow the UE to know which beam/TCI state is associated to which TAG, the TAG ID has been introduced as a field in the TCI state IE.

  - That allows a UE to receive a TA command TA1 for first TRP and a TA command TA2 for second TRP, for both intra-cell and inter-cell multi-TRP.

- The UE is required to monitor two DL reference timings, i.e., one DL reference per TAG.

  - As with legacy single TA requirements, it is up to the UE to decide which RS, among the RSs transmitted by each TRP, to use as DL reference timing for each TAG.

The Rel-19 asymmetric DL sTRP / UL mTRP scenario is specified as a single-DCI multi-TRP scenario. In this scenario the UL transmissions are scheduled by the first TRP, i.e., the DL/UL capable TRP, which is the only TRP sending data and control information to the UE.

There has been an LS [2] from RAN4 to RAN1 to clarify the downlink functionalities of an “UL-only TRP”, and more specifically if an UL-only TRP may or may not transmit RSs as SSBs.

In our understanding, an UL-only TRP as discussed in RAN1 does not transmit any RS. On the other hand, it is not fully clear from the WID whether RRM requirements with two TAs shall be defined also for the case that the second TRP, which does not transmit any data nor control to the UE, may still send reference signals like SSBs.

From the RAN4 perspective, the Rel-18 and Rel-19 two TAs enhancements are very similar and in the RAN4 context the main difference may be in the number of DL reference timings that the UE shall monitor, as shown in Figure 1. From that figure, we may identify two cases for Rel-19 depending on whether the second TRP is sending SSB or not:

- Case 1 (second TRP as UL-only TRP without any DL signal): If the second TRP, which is not sending data or control to the UE, is an UL-only TRP in the strict sense (i.e., it does not transmit anything), then there simply is no DL RS associated to the second configured TAG (assuming the first configured TAG is associated to the DL/UL capable TRP), and the UE shall monitor just one DL reference timing related to the DL/UL capable TRP, as captured in the RAN1#118bis agreement. As a result, the UE applies both TA commands based on the same DL reference timing to determine the UL transmission timings toward the two TAGs. We also highlight that most of the work in RAN1 has focused on this case to define two closed-loop power control adjustments for SRS, where the UE monitors only one DL pathloss RS, which is transmitted by the DL/UL capable TRP, and receives a “pathloss offset update” by the network, which is used by the UE for setting the power of the UL transmission toward such UL-only TRP.

- Case 2 (second TRP that sends some DL RSs): If the second TRP, which is not sending data or control to the UE, is a TRP that also transmits some DL signals, for example transmits SSBs, then the UE can measure independent DL RSs for each of the two configured TAGs. As a result, the UE shall monitor two DL reference timings, as in Rel-18 enhancement, one per TAG or TRP, and apply each TA command to the respective DL reference timing to determine the UL transmission timing toward that TAG. In that case, the system performance would be improved by using the independent reference timing from both TRPs, since the UL timing would be more robust for UE movement.

RAN1 agreements do not preclude the case that two TAs with single-DCI are used with two TRPs, each transmitting DL RS.

If in the example of Figure 1 both TRPs transmit SSBs, the UE would need less timing advance commands if it monitors two DL reference timings. Let’s consider one example where a UE is located in between the two TRPs. If the UE only uses the DL signals of the TRP containing PDCCH, the UE will perform autonomous timing adjustments only based on the first TRP. In that case, all the adjustments made by this UE may increase the UL timing error towards the second TRP since adjustments should be performed in the opposite direction for the second TRP. On the other hand, if the second TRP also contains SSB, which is monitored by the UE for the second TAG, the UE will be able to perform autonomous timing adjustments appropriately for the second TAG.

In another example if the second TRP contains reference signals and the legacy definition of reference timing is used, there is an ambiguity on which TRP is being used for reference timing. The UE might use RS from first TRP or second TRP at different time instants. Therefore, it is important to define some rules to determine how the UE can differentiate from the reference signals for first TRP and second TRP in a single-DCI scenario.

If only one DL reference timing is used for single-DCI with two TAs, there is an ambiguity on the reference timing if both TRPs send SSB.

Several options can be used to define the condition under which the UE shall monitor just one DL reference timing, i.e., indicative that the second TRP is an UL-only TRP and therefore does not transmit any DL signal, and we think we should down-select from the following ones (although other options are not precluded):

- The UE receives explicit information about the second TRP, for example, via RRC, that the second TRP is a strictly UL-only TRP.

  - That, however, needs to be clarified if aligned with the current WID that states the specification to be done “without changing existing cell definition or defining a new cell (e.g. UL-only cell)”.

- There is at least one separate UL or joint DL/UL TCI state configured or activated or indicated at the UE to which the RAN1 Rel-19 introduced “pathloss offset” is associated.

  - That implicitly informs the UE that the second TRP does not transmit any DL RS.

Specify the conditions under which a UE shall monitor one or two DL reference timings for single-DCI multi-TRP with two TAs enhancement.

Figure 1. One or two DL reference timings in asymmetric DL sTRP/UL mTRP scenario.

Discussion on TCI switching requirements

Regarding asymmetric DL sTRP/UL mTRP TCI switching, the following was agreed and discussed in the last RAN4#112bis meeting [1]:

Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI? Way forward: - Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states. - Option 2: RAN4 to specify UL TCI state switching requirements. - Option 3: FFS.

An asymmetric DL sTRP/UL mTRP deployment scenario is a single-DCI scenario where the symmetric DL/UL TRP will transmit and receive the PDCCH and the PUCCH respectively. In Rel-18 MIMO evo, RAN4 has defined downlink as well as uplink TCI state switching delay for unified TCI state switch for single-DCI mTRP.

In case of downlink TCI state switching delay requirements, we see no difference in the downlink TCI state switching behavior and hence see no need to define new set of downlink TCI state switch delay requirements.

Similar to the downlink TCI state switching delay requirements, there is no difference in the uplink TCI state switching behavior either when the UE is indicated to switch to a target uplink TCI state associated to the uplink only TRP. Hence, the legacy RAN4 unified TCI state switching delay requirements for UL TCI state can be reused.

Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Although the delay for UL TCI state switch does not need to be changed from the legacy, the requirements still need some updates for the UL-only TRP TCI state switch case.

The uplink TCI state switching delay requirements specified up to Rel-18 have been defined for an uplink TCI state which is associated with a downlink reference signal. For a UL-only TRP’s TCI state, the reference signal configured as QCL-D is an SRS, which is an uplink signal. The existing UL TCI state switching requirements consider only the case of a TRP which transmits downlink reference signal. In the case of asymmetric DL sTRP/UL mTRP, since the uplink only TRP does not transmit any downlink reference signal, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements. Hence, the applicability of the existing uplink TCI state switch delay requirements should be extended to include uplink TCI states which are associated to an uplink reference signal (SRS).

Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

In the case of asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any downlink reference signal, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements.

Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal.

The legacy TCI state switch delay requirements have been defined based on a TCI state being known or unknown. For a TCI state to be considered as known there are set of conditions defined which are primarily centered around a downlink reference signal being transmitted by the TRP. The known/unknown conditions are used to determine if the UE has reported a valid recent measurement from the target TCI state before making the switch. In case the target TCI state is unknown the UE is expected to perform measurements on the target TCI state, in order to determine transmission parameters such as spatial filter of the target TCI state.

However, in the case of an asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any reference signal, this implies that the conditions defined up to Rel-18 for an uplink TCI state to be known cannot be used in the context of an asymmetric DL sTRP/UL mTRP.

Legacy uplink TCI state switching delay requirements are based on a TCI state switch being known or unknown.

Conditions defined for a TCI state to be considered as known or unknown are based on a recent measurement of a downlink reference signal.

For an uplink-only TRP, since there is no downlink reference signal being transmitted, the legacy known/unknown conditions cannot be reused.

RAN4 to define UL TCI state switch delay requirements for an uplink-only TRP with no distinction between known and unknown TCI states.

Reporting for CJT calibration

In RAN4 # 112-bis, the topic of time averaging for CJT calibration quantities was discussed for DO, FO and PO reporting quantities. Here, one of the main questions that appear is how much better accuracy can be achieved by time averaging of a set of measurements and, at the same time, the delay that it might imply to obtain the final estimated value. Based on previous results from our RAN 1 simulations, we may analyze how much benefit was obtained for DO and FO with a single shot estimation.

In Figure 2 we present simulations to analyze the case for DO reporting with different CJT eType II configurations (i.e. different bit overhead) for 500m ISD, with Time Alignment Error (TAE) uniformly distributed from 0 to the Cyclic Prefix (CP), P-CSI RS each 5ms, 4 antennas at the UE and 16 antenna ports per TRP. The UE distribution consists of 20% outdoor (30kmph), 80% indoor (3kmph). CJT is enabled with up to 4 TRP joint transmission. For CSI feedback, CJT eType II mode-2 is used (i.e. with no tap realignment during FD compression and common FD-basis).

As seen in the results of Figure 2, with TAE and the respective compensation with/without quantization (5 bits i.e. MD=32) is possible to nearly obtain ideal results as in the case with no TAE. Therefore, one might think that time averaging in this setup would not bring any significant enhancement as the room for improvement between the actual reported DO estimations and the ideal case show a very negligible difference.

Figure 2 mean UE Thrp vs. CJT Type II bit overhead under different TAE estimation/compensation conditions without averaging (TRP selection might be slightly affected and then bit overhead).

The results for FO reporting are shown in Figure 3. Likewise for the case with FO reporting in the same scenario and with CFO distribution uniformly distributed for all TRPs into (-ΔfCFO,max, ΔfCFO,max) with ΔfCFO,max corresponding to an error of 0.05 ppm, we observe a similar result. As seen, the actual estimations and ideal estimations (i.e. GA, genie-aided) with quantization (4 bits i.e. MFO=16) and the ideal case with no CFO, yield no major differences in performance. Therefore, we have the same conclusion as previously: the difference of performance lead us to think that time averaging would not bring any additional advantage in comparison to a single shot estimation.

Based on our initial simulations, DO and CFO CJT estimation and compensation seem to count already with a good performance with single shot, which does not give the room for improvement to introduce time averaging.

Figure 3 mean UE Thrp vs. CJT Type II bit overhead under different single-shot CFO estimation and compensation conditions without averaging.

However, the study could be extended by utilizing different simulation parameters to stress the estimation methods. For instance, one may increase ISD to increase the DO or increasing the error or maximum offset in the distributions for CFO and DO. By doing this we may see observe a larger gap between the ideal CJT case without impairments and the CJT case using a calibration quantity for estimation and compensation of the offsets.

Study new scenarios in which the simulation parameters could be stressed to see a potential room for improvement and then time averaging could be used and properly justified.

The previous examples for DO and CFO CJT calibration have a point in common. They can be measured and reported at one side of the communication link, more particularly at the UE side. Thus, we may say that DO and CFO CJT calculations are one-sided estimation cases. For the case of UL/DL PO CJT calibration the situation is a bit more complicated as part of the phase measurements and calculations can be carried out in the UE with CSI-RS and another part of these measurements and calculations need to be run in the NW side based on the SRS measurements received from the UE.

During the RAN 1 studies different type of averaging have been explored for UL/DL PO CJT. In frequency domain, as matter of fact initially the method was conceived to be wideband (averaging in frequency). However, in order to improve the method in presence of increased delay spread across TRP measurements, a subband reporting alternative was proposed to report multiple PO values for different sub-bands and tackle the frequency selectivity. On the other hand, measurements with different number of SRS antenna ports, i.e., PSRS>1 were proposed to increase the accuracy and robustness against noise (antenna domain averaging). However, there was no strong evidence to support that antenna domain averaging would provide gains. As a result the UL/DL PO estimation method agreed in RAN1 requires only one UE antenna, i.e.,  PSRS=1.

Besides antenna averaging, there are other mechanisms that can be studied to improve performance with PO calibration. For instance, another way to combat the effect of SRS noise might be the time averaging. In the simulation from Fig. 3 we have somehow managed to obtain a good estimation and compensation, because we have limited the ISD to only 200m. However, for larger ISD the impact of noise in the measurements and estimations may result in degraded performance. The SRS in comparison to CSI-RS exhibits lower values of SNR with approximately 20dB gap due to the existing UL/DL power imbalance. Thus, SNR will be worse if the ISD is increased as users might be located at a longer distance from the associated TRPs.

Figure 4 Comparison between wideband PO calculation measured from one UE antenna. Network implementation with non-precoded CSI-RS for calibration measurement and ISD = 200 m.

Different mechanisms of time averaging at the UE and NW side can take place. For instance, one question would be: are several PO measurements in time to be UE reported and the time averaging takes place in the NW side? In that case, the number of estimations to be sent must be determined and configured in the UE in order to send these PO estimates toward the NW. However, one disadvantage of this option is the fact that quantization could introduce some error and it requires more bit overhead than a single time CJT calibration report. Alternatively, these measurements could be instead buffered in the UE and the NW in order to exchange a single CJT PO estimate of an averaged value in the UE. As depicted in Fig. 4, the buffering could happen after the CJT calibration triggering and thus the UE and NW sends SRS and CSI-RS respectively by means of periodic/semi-persistent schemes to enable a number of measurements for UL and DL estimations. The number of measurements to take at each side and other averaging related parameters might be needed to be signaled to the UE and sent from the NW. Otherwise the buffering could start before and the CSI-RS and SRS might have been preconfigured before the triggering. Also in this case the NW must indicate the number of measurements to include in the time-averaging and any other related parameters. Notice that in any of the previously mentioned possible implementations, a pairing/linkage in time domain is required between pairs of CSI-RS/SRS transmission occasions in order to have a consistent final PO estimate from the averaged UL and DL measurements. This means that each pair of CSI-RS/SRS must be located the closest possible to each other.

In contrast to the DO and CFO estimations which only requires estimations at UE side,   UL/DL PO CJT estimation and compensation requires measurements and calculations both in the UE and the NW side. Therefore, if time-averaging is used there should be consistency for the calculation.

For UL/DL PO CJT estimation and compensation with time-averaging, further study the necessary NW configuration signaling to ensure consistent averaging parameters at both sides.

Figure 5 Illustration of CSI-RS and SRS transmissions for (a) when the measurements are collected and averaged after the triggering and (b) when the measurements are collected and averaged before the triggering.

Conclusion

In this paper, the following Observations and Proposals were made:

Proposal 1: Existing RRM requirements with single TA apply for asymmetric DL sTRP/UL mTRP deployment.

Observation 1: RAN1 agreements do not preclude case that two TAs with single-DCI is used with two TRPs each transmitting DL RS.

Observation 2: If only one DL reference timing is used for single-DCI with two TAs, there is an ambiguity on the reference timing if both TRPs send SSB.

Proposal 2: Specify the conditions under which a UE shall monitor one or two DL reference timings for single-DCI multi-TRP with two TAs enhancement.

Observation 3: Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

Proposal 3: For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Observation 4: Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

Observation 5: In the case of asymmetric DL sTRP/UL mTRP, since the uplink only TRP does not transmit any downlink reference signal, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements.

Proposal 4: Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal.

Observation 6: Legacy uplink TCI state switching delay requirements are based on a TCI state switch being known or unknown.

Observation 7: Conditions defined for a TCI state to be considered as known or unknown are based on a recent measurement of a downlink reference signal.

Observation 8: For an uplink only TRP, since there is no downlink reference signal being transmitted, the legacy known/unknown conditions cannot be reused.

Proposal 5: RAN4 to define UL TCI state switch delay requirements for an uplink only TRP with no distinction between known and unknown TCI state.

Observation 9: Based on our initial simulations, DO and CFO CJT estimation and compensation seem to count already with a good performance with single shot, which does not give the room for improvement to introduce time averaging.

Proposal 6: Study new scenarios in which the simulation parameters could be stressed to see a potential room for improvement and then time averaging could be used and properly justified.

Observation 10: In contrast to the DO and CFO estimations which only requires estimations at UE side,   UL/DL PO CJT estimation and compensation requires measurements and calculations both in the UE and the NW side. Therefore, if time-averaging is used there should be consistency for the calculation.

Proposal 7: For UL/DL PO CJT estimation and compensation with time-averaging, further study the necessary NW configuration signaling to ensure consistent averaging parameters at both sides.

- References

- R4-2416921, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4#112bis, Hefei, Anhui, China, 14th ‒ 18th, October, 2024.

- R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, RAN4#112bis, Hefei, Anhui, China, 14th ‒ 18th, October, 2024.