3GPP TSG-RAN WG4 Meeting #113	R4-2418893

# Orlando, USA, 18 Nov. – 22 Nov., 2024

Source: 	vivo

Title: 	Consideration on simulation assumptions and results for LP-WUR

Agenda Item:	7.23.4.1

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2]. The initial discussion on simulation assumptions was triggered at RAN4 111 meeting and the simulation assumptions were further discussed at RAN4 112 meeting. In this contribution we provide our further consideration on simulation assumptions and also provide further simulation results from our side.

The objectives of WI are duplicated as below:

The objectives of the work item are the following:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

At RAN4 112bis meeting, we have the following agreements:

Issue 2-1-3: Time/frequency error

Agreement:

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4 |  |

Issue 2-1-4: Accuracy baseline for simulation

Agreement:

Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only:

- For FR1:

  - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

  - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

- Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session

- Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

Discussion

The WF and contribution summary for this WI at RAN4 112bis meeting are [3] and [4]. In this contribution, we provide our considerations on LP-WUR simulation assumptions and initial simulation results.

Consideration on simulation assumptions

Measurement metrics

For the measurement metrics, RAN1 had the following agreements:

RAN1 #117

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

RAN1 #118

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

Working Assumption

The definitions of LP-RSSI for LP-RSRQ is further updated as follows:

LP-RSSI is the linear average of total received power in all the ON and OFF LP-SS OOK symbols over the frequency resources that carry LP-SS.

In RAN4’s simulation assumption, the measurement metrics will follow RAN1’s conclusion. It is suggested that the LP-RSRP and LP-RSRQ follow RAN1’s latest update in simulation.

Issue 2-1-2: Measurement metrics

Recommendations:

Follow RAN1’s conclusion

Proposal 1: The LP-RSRP and LP-RSRQ shall follow RAN1’s latest update in simulation.

Time/frequency error

For the time/frequency error, we have the following agreement at RAN4 112bis meeting based on RAN1 agreement:

Residual timing error:

T= 5us for OOK-1

T= 2us for OOK-4 with M=2

T= 1us for OOK-4 with M=4

To our understanding, in the simulation some companies will perform sync before RRM measurement hence the residual timing error will be input from sync part to the RRM measurement part and the maximum value should be less than the above agreements. Some companies may simulate the RRM measurement performance directly and the above agreed residual timing error can be used as the input for the RRM measurement part. In our view it is not necessary to mandate the simulation methodology, i.e., either perform sync before RRM measurement or not, hence the above agreed residual timing error will be viewed as the upper bound of the timing error after sync. In practical simulation the value can still be better than these values.

- The agreed residual timing error is the upper bound of the timing error after sync process.

Subcarrier spacing in simulation

In the current simulation assumptions, the SCS is:

| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| --- | --- |

At RAN1 118bis meeting the following agreement is available:

Agreement

From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz

However, considering the simulation working amount, it is suggested to still focus on 30K Hz SCS.

Proposal 2: In simulation, SCS is still based on 30K Hz.

Regarding ideal RSRP

Regarding how to obtain ideal RSRP, especially on ideal RSRP under fading channel (TDL-C), traditionally the RSRP signal is passed the fading channel and with the assumption that perfect channel estimation will be applied to the ideal RSRP, the impact from the fading channel can be eliminated in the end, which is a reasonable assumption for traditional MR type receiver which has strong capability. However, for the OOK based receiver, if RSRP is passed by the fading channel, due to the limitation of the OOK receiver, even with the perfect channel estimation, the effect of the fading channel may not be fully eliminated in the end. Hence there could be two methods on the ideal RSRP:

- Method 1: The first one is the RSRP is pass the fading channel and with the perfect channel estimation, i.e., fully knowledge of the fading channel, and the real RSRP pass the same channel. Although the OOK receiver does not have any advanced capability to counteract the impact of the fading channel, due to the same fading channel is used for ideal RSRP and real RSRP, the fading channel impact will be almost cancelled each other. however still assumes the impact of the fading channel can be fully removed, hence in simulation the ideal RSRP need not pass the fading channel.

- Method 2: The second one assumes the impact of the fading channel can be completely removed hence the ideal RSRP does not need pass the channel whereas the real RSRP will pass the fading channel. Due to the fading channel between ideal and real RSRP does not counteract each other, this method, to some extent, shows the impact of the fading channel on the performance due to limited capability of the receiver (OOK based). Actually, this method is close the worst case due to the limited capability of the OOK receiver.

Proposal 3: Suggest to discuss and align the ideal RSRP calculation.

Simulation results

Our initial simulation results are provided below and consider the latest RAN4 agreement on accuracy for simulation purpose.

- For FR1:

  - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

  - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

We provide results on both methods described in section 2.1.4 and the performance of the first method is better compared with that of the second one.

- For RRM measurement purpose, UE can satisfy RSRP/RSRQ measurement accuracy (3.5dB) by at least 3 LP-SS samples at SINR = -3dB under TDL-C channel based on 320ms periodicity of LP-SS, no matter which ideal RSRP method is used. Same number of samples is sufficient to satisfy corresponding requirement under AWGN channel.

- For RRM measurement purpose, UE can satisfy RSRP/RSRQ measurement accuracy (3.5dB) based on 1 SSB samples at SINR = -3dB under AWGN channel based on 320ms periodicity of LP-SS.

Table 1 Delta RSRP/RSRQ value for OOK receiver, asynchronous case, AWGN Channel with 3ms delay for neighbour interference (ideal RSRP method 2)

| AWGN(3ms Delay) |  | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.072 | 1.026 | 1.103 | 0.999 |
|  | M=2,LP-SS=8 | 0.915 | 0.819 | 0.912 | 0.763 |
|  | M=4,LP-SS=16 | 0.951 | 0.844 | 0.886 | 0.764 |
| Set2 | M=1,LP-SS=8 | 1.066 | 1.019 | 1.371 | 1.149 |
|  | M=2,LP-SS=8 | 0.917 | 0.822 | 1.200 | 0.906 |
|  | M=4,LP-SS=16 | 0.946 | 0.852 | 1.008 | 0.794 |

Table 2 Delta RSRP/RSRQ value for OOK receiver, synchronous case, AWGN Channel with CP/2 delay for neighbor interference (ideal RSRP method 2)

| AWGN(CP/2 Delay) |  | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.058 | 1.012 | 1.016 | 0.968 |
|  | M=2,LP-SS=8 | 0.920 | 0.819 | 0.881 | 0.776 |
|  | M=4,LP-SS=16 | 0.969 | 0.870 | 0.908 | 0.810 |
| Set2 | M=1,LP-SS=8 | 1.062 | 1.006 | 0.977 | 0.936 |
|  | M=2,LP-SS=8 | 0.927 | 0.824 | 0.868 | 0.770 |
|  | M=4,LP-SS=16 | 0.978 | 0.888 | 0.887 | 0.797 |

Table 3 Delta RSRP/RSRQ value for OOK receiver, asynchronous case, TDL-C Channel with 3ms delay for neighbor interference (ideal RSRP method 2)

| TDL-C(3ms Delay) |  | RSRP 2 sample NR interference (unit dB) | RSRQ 2 sample NR interference (unit dB) | RSRP 2 sample LP-SS interference (unit dB) | RSRQ 2 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 3.421 | 2.741 | 3.473 | 2.738 |
|  | M=2,LP-SS=8 | 3.490 | 2.419 | 3.463 | 2.353 |
|  | M=4,LP-SS=16 | 3.487 | 2.435 | 3.505 | 2.367 |
| Set2 | M=1,LP-SS=8 | 3.419 | 2.756 | 3.524 | 2.762 |
|  | M=2,LP-SS=8 | 3.474 | 2.411 | 3.530 | 2.365 |
|  | M=4,LP-SS=16 | 3.483 | 2.440 | 3.539 | 2.395 |

Table 3-1 Delta RSRP/RSRQ value for OOK receiver, asynchronous case, TDL-C Channel with 3ms delay for neighbor interference (ideal RSRP method 1)

| TDL-C(CP/2 Delay) |  | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.5014 | 1.9487 | 1.5123 | 1.9129 |
|  | M=2,LP-SS=8 | 1.2737 | 2.0079 | 1.2458 | 1.9880 |
|  | M=4,LP-SS=16 | 1.3910 | 1.9920 | 1.3507 | 1.9693 |
| Set2 | M=1,LP-SS=8 | 1.4189 | 1.8937 | 1.4835 | 1.9056 |
|  | M=2,LP-SS=8 | 1.2274 | 1.9791 | 1.2598 | 2.0155 |
|  | M=4,LP-SS=16 | 1.3077 | 1.9752 | 1.3509 | 2.0119 |

Table 4 Delta RSRP/RSRQ value for OOK receiver, synchronous case, TDL-C Channel with CP/2 delay for neighbor interference (ideal RSRP method 2)

| TDL-C(CP/2 Delay) |  | RSRP 2 sample NR interference (unit dB) | RSRQ 2 sample NR interference (unit dB) | RSRP 2 sample LP-SS interference (unit dB) | RSRQ 2 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 3.416 | 2.759 | 3.443 | 2.740 |
|  | M=2,LP-SS=8 | 3.488 | 2.407 | 3.453 | 2.366 |
|  | M=4,LP-SS=16 | 3.511 | 2.466 | 3.526 | 2.404 |
| Set2 | M=1,LP-SS=8 | 3.412 | 2.759 | 3.443 | 2.718 |
|  | M=2,LP-SS=8 | 3.485 | 2.418 | 3.461 | 2.341 |
|  | M=4,LP-SS=16 | 3.511 | 2.481 | 3.496 | 2.376 |

Table 4-1 Delta RSRP/RSRQ value for OOK receiver, synchronous case, TDL-C Channel with CP/2 delay for neighbor interference (ideal RSRP method 1)

| TDLC(CP/2 Delay) | Ideal RSRP pass channel | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.5014 | 1.9487 | 1.5123 | 1.9129 |
|  | M=2,LP-SS=8 | 1.2737 | 2.0079 | 1.2458 | 1.9880 |
|  | M=4,LP-SS=16 | 1.3910 | 1.9920 | 1.3507 | 1.9693 |
| Set2 | M=1,LP-SS=8 | 1.4189 | 1.8937 | 1.4835 | 1.9056 |
|  | M=2,LP-SS=8 | 1.2274 | 1.9791 | 1.2598 | 2.0155 |
|  | M=4,LP-SS=16 | 1.3077 | 1.9752 | 1.3509 | 2.0119 |

- LP-WUR based on SSB signal (OFDM)

Table 5 Delta RSRP/RSRQ value for OFDM receiver, synchronous case, AWGN Channel with 3ms delay for neighbor interference (ideal RSRP method 2)

| AWGN(3ms Delay) | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- |
| Set1 | 0.2925 | 0.2885 | 0.2915 | 0.2866 |
| Set2 | 0.2881 | 0.2880 | 0.2901 | 0.2857 |

Table 6 Delta RSRP/RSRQ value for OFDM receiver, synchronous case, AWGN Channel with CP/2 delay for neighbor interference (ideal RSRP method 2)

| AWGN(CP/2 Delay) | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- |
| Set1 | 0.2902 | 0.2879 | 0.2917 | 0.2867 |
| Set2 | 0.2919 | 0.2867 | 0.2905 | 0.2857 |

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- The agreed residual timing error is the upper bound of the timing error after sync process.

- For RRM measurement purpose, UE can satisfy RSRP/RSRQ measurement accuracy (3.5dB) by at least 3 LP-SS samples at SINR = -3dB under TDL-C channel based on 320ms periodicity of LP-SS, no matter which ideal RSRP method is used. Same number of samples is sufficient to satisfy corresponding requirement under AWGN channel.

- For RRM measurement purpose, UE can satisfy RSRP/RSRQ measurement accuracy (3.5dB) based on 1 SSB samples at SINR = -3dB under AWGN channel based on 320ms periodicity of LP-SS.

Proposal 1: The LP-RSRP and LP-RSRQ shall follow RAN1’s latest update in simulation.

Proposal 2: In simulation, SCS is still based on 30K Hz.

Proposal 3: Suggest to discuss and align the ideal RSRP calculation.

- References

# [1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

# [2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] R4-2415664	Topic summary for [112bis][217] NR_LPWUS, RAN4 112bis,vivo

[4] R4-2416861,	WF on RRM requirements for NR_LPWUS, RAN4 112bis, vivo

# [5] TS38.133, V18.4.0

- Appendix

- Simulation assumptions

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | [Data, SSB and LP-SS have the same SCS] |
| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| Measurement period (in number of measurement samples) | [4, 5, other number could be studied upon a need] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |