3GPP TSG-RAN WG4 Meeting #113	R4-2418894

# Orlando, USA, 18 Nov. – 22 Nov., 2024

Source: 	vivo

Title: 	Consideration on RRM requirements for UE-to-UE CLI handling

Agenda Item:			7.20.3.1

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD.

- Discussion

This WI was initially discussed at RAN4 112 meeting. At previous meeting, based on the WF [2], the following agreements are available:

Issue 1-1-1: Measurement quantities

- Agreements (from online session)

  - RAN4 to define requirements for L1-SRS-RSRP and L1-CLI-RSSI.

  - Requirements for other measurement quantities can be considered based on further RAN1 agreement.

Issue 1-1-6: Side condition

- Agreements (from online session)

  - Send LS to RAN1: in addition to intra-cell scenario, whether inter-cell scenario is considered in this WI. Further discuss the details in the LS.

  - For SRS configuration, re-use Rel-16 side condition as baseline.

Issue 1-1-14: Measurement timing for L1-SRS-RSRP

- Agreements (from online session)

  - As baseline, for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

  - This can be revisited if it proves to be conflicting with further RAN1 agreements.

The following topics are open for discussion.

Issue 1-1-3: Measurement methods

- Background

Agreement (from RAN1#118) For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

- Proposals

  - Proposal 1 (vivo, ZTE, HW, MTK):

    - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

  - Proposal 2 (LGE):

    - The UE to support L1 based UE-to-UE CLI measurement has to capable to measure the CLI measurement with sub-band unit rather than DL-active BWP unit.

  - Proposal 3 (Nokia):

    - RAN4 to define the CLI measurement requirements in SBFD as the following:

      - SRS-RSRP measurement requirements for Method#2.

      - CLI-RSSI measurement requirements for Method#1 and Method#3.

  - Proposal 4 (Samsung, CTC, QC):

    - RAN4 shall specify L1 CLI measurement requirements at least for below methods

      - Method#1: UE measures RSSI within DL subband

      - Method#2: UE measures RSRP of aggressor UE within UL subband

    - Additional methods can be considered based on further RAN1 agreements.

- Recommended WF

  - RAN4 shall specify L1 CLI measurement requirements at least for below methods:

    - Method#1: UE measures RSSI within DL subband

    - Method#2: UE measures RSRP of aggressor UE within UL subband

  - Additional methods can be considered based on further RAN1 agreements.

  - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

For the measurement method, currently two methods have been confirmed by RAN1. The difference between method 1 and method 2 is in method 1, the UE will measure the strength of the leakage signal and method 2 measures the UL signal transmitted by aggressor UE directly. Differentiation on requirements based on these two different methods should be considered if there is no further down-select on these two methods from RAN1.

Proposal 1: For issue 1-1-3, OK with the recommend way forward and differentiation on requirements based on different methods should be considered.

Issue 1-1-5: Measurement resources

- Background

Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting

- Proposals

  - Proposal 1 (CATT):

    - Configuration with two discontinuous DL subbands should be taken into account when simulating and defining measurement accuracy requirements for CLI RSSI measurements

  - Proposal 2 (LGE, HW, MTK, Ericsson):

    - RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

  - Proposal 3 (Nokia):

    - RAN4 should define new requirements for the sub-band CLI-RSSI reporting if this feature is supported by RAN1.

  - Proposal 4 (Samsung):

    - RAN4 focuses on wideband L1-CLI-RSRI and L1-SRS-RSRP for L1 CLI measurement.

- Recommended WF

  - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement based on wide-band reporting. FFS on sub-band L1-CLI-RSSI pending on further RAN1 conclusion.

  - Further discuss whether and how L1-CLI-RSRI measurement requirements would be impacted by measurement resource configuration or measurement reporting configuration.

For the measurement resources, depending on the configuration, it is possible that the measurement resources locates at two different discontinuous DL subbands. To our understanding under this scenario either sub-band reporting or wide-band reporting can be configured by RAN1. For the wide-band reporting, how to combine the measurement results from different discontinuous DL subbands could be up to UE implementation. From requirements point of view, the current CSI accuracy requirements are applicable when at least 48 RBs are used for measurement. For SBFD operation, since some spectrum will be used for the UL, for either wide-band reporting or sub-band reporting, RAN4 needs to discuss the applicability of requirements when the number of RBs used for measurement is less than 48.

Proposal 2: Either sub-band reporting or wide-band reporting can be configured. For the wide-band reporting when measurement results from different discontinuous DL subbands need be combined, how to combine the measurement results could be up to UE implementation

Proposal 3: RAN4 needs to discuss the applicability of requirements when the number of RBs used for measurement is less than 48.

Issue 1-1-6: Side condition

- Proposals for timing offset

  - Proposal 1 (vivo, CATT, LGE, Nokia, Samsung, E///):

    - Consider intra-cell scenario only.

    - Remove cell phase error from Rel-16 SRS-RSRP side condition.

  - Proposal 2 (CMCC, Apple, ZTE, MTK):

    - Consider both intra-cell and inter-cell scenarios.

    - Keep cell phase error as in Rel-16 SRS-RSRP side condition as starting point.

  - Proposal 3 (Samsung, Huawei):

    - Consider both intra-cell and inter-cell scenarios.

    - Two side conditions with and without cell phase error are considered at least for simulation, FFS to keep both or down-select for defining requirements.

  - Proposal 4 (Apple):

    - Send LS to clarify with RAN1 if the L1 based UE-to-UE CLI measurement and reporting is limited to intra-cell case only.

- Proposals for SRS Es/Iot

  - Proposal 1 (vivo, CATT, LGE, Samsung, HW, MTK, QC):

    - Re-use Rel-16 side condition (SRS Es/Iot = 1dB) as baseline.

  - Proposal 2 (Apple, Nokia):

    - Consider a higher SRS Es/Iot.

  - Proposal 3 (ZTE):

    - Determine the side condition of SRS Es/Iot according to the evaluation results.

- Proposals for SRS configuration and maximum/minimum SRS-RSRP

  - Proposal 1 (vivo, CATT, LGE, Apple, Nokia, Samsung, HW, MTK, QC):

    - Re-use Rel-16 SRS configuration and maximum/minimum SRS-RSRP as baseline.

  - Proposal 2 (ZTE):

    - For the side condition of maximum RSRP, FFS whether reusing the value in R16 SRS-RSRP CLI measurement, depending on whether the assumptions on SRS Tx power, antenna gain and pathloss changed or not.

- For the time offset between DL timing and SRS arrival timing, a LS has sent to RAN1 to clarify whether the inter-cell UE-to-UE L1 CLI measurement needs be considered or not. Upton RAN1’s reply, to our understanding, the question on whether inter-cell UE-to-UE L1 CLI measurement is supported or not makes sense or not rely on whether the NW can differentiate an aggressor UE is intra-cell or inter-cell. Based on our understanding on RAN1 design, the NW has the capability to differentiate an intra-cell aggressor UE or an inter-cell aggressor UE. However this is the information from NW side and it does not mean the victim UE can have this information or not. From UE perspective, the requirements may need consider the worst case hence the 3us cell phase synchronization error is likely to be considered.

- Proposal 4: From UE perspective, the differentiation on inter-cell aggressor UE or intra-cell aggressor UE maybe not possible even the NW side has this capability. The 3us cell phase synchronization error may still need be considered for the worst case.

Issue 1-1-7: Measurement period

- Proposals for L1-SRS-RSRP

  - Proposal 1 (vivo):

    - Requirements based on 1 sample measurement assumption should be used.

  - Proposal 1a (QC, Ericson):

    - RAN4 to define the measurement delay for aperiodic reporting of L1-SRS-RSRP at least based on a single sample.

  - Proposal 2 (CATT):

    - Re-use the framework of R16 CLI measurement period except for the sample number.

  - Proposal 3 (Nokia):

    - Follow L1-RSRP measurement approach, and both 1 and 3 samples are considered based on network configuration

  - Proposal 3a (Samsung):

    - RAN4 can evaluate L1 SRS-RSRP measurement performance considering both single shot and multiple shots measurements if different side condition identified

- Proposals for L1-CLI-RSSI

  - Proposal 1 (vivo, Nokia, HW, QC):

    - Requirements based on 1 sample measurement assumption should be used

  - Proposal 2 (E///):

    - Rel-16 CLI-RSSI requirement framework can be used as a baseline. Further updates can be made when RAN1 finalized the measurement RS configuration and method.

- Recommended WF

  - For L1-SRS-RSRP, further discuss the following options based on simulation results.

    - Option 1: single set of requirements based on [1] sample

    - Option 2: two sets of requirements based on [1] and [3] samples

    - Other options are not precluded

  - For L1-CLI-RSSI, measurement period requirements are defined based on 1 sample.

For issue 1-1-7, we support option 1 for L1-SRS-RSRP measurement since a quick feedback is the intention to have this kind of measurement. In addition the accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed.

- Proposal 5: For issue 1-1-7, support option 1 for L1-SRS-RSRP measurement. In addition the accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed if necessary.

Issue 1-1-8: Measurement reporting

- Proposals

  - Proposal 1 (vivo):

    - Define requirements for periodic, semi-persistent reporting.

  - Proposal 2 (Samsung, CTC, HW, MTK):

    - Focus on aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1

  - Proposal 3 (CATT, Nokia, HW, E///):

    - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused, and wording can be adapted when reporting scheme design is stable.

  - Proposal 4 (MTK):

    - FFS whether L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

- Recommended WF

  - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting.

  - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

  - For aperiodic reporting, L1-RSRP measurement reporting requirement can be re-used as baseline.

- For the measurement aperiodic reporting, it is fine to consider L1-RSRP measurement reporting requirement can be re-used as baseline and wait for further RAN1 agreement on requirements for periodic and semi-persistent reporting.

- Proposal 6: Consider L1-RSRP measurement reporting requirement can be re-used as baseline and wait for further RAN1 agreement on requirements for periodic and semi-persistent reporting

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: For issue 1-1-3, OK with the recommend way forward and differentiation on requirements based on different methods should be considered.

Proposal 2: Either sub-band reporting or wide-band reporting can be configured. For the wide-band reporting when measurement results from different discontinuous DL subbands need be combined, how to combine the measurement results could be up to UE implementation

Proposal 3: RAN4 needs to discuss the applicability of requirements when the number of RBs used for measurement is less than 48.

- Proposal 4: From UE perspective, the differentiation on inter-cell aggressor UE or intra-cell aggressor UE maybe not possible even the NW side has this capability. The 3us cell phase synchronization error may still need be considered for the worst case.

- Proposal 5: For issue 1-1-7, support option 1 for L1-SRS-RSRP measurement. In addition the accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed if necessary.

- Proposal 6: Consider L1-RSRP measurement reporting requirement can be re-used as baseline and wait for further RAN1 agreement on requirements for periodic and semi-persistent reporting

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2416858, WF on RRM requirements for NR_duplex_evo, RAN4 112bis, Huawei