3GPP TSG-RAN WG4 Meeting #113	R4-2418902

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on RRM requirements of NTN with channel BW less than 5MHz

Agenda Item:	7.8.3.4

Document for:	Discussion

# Introduction

In the WI NR Enhanced requirements and test methodology for NR and IoT NTN, the objective: RRM of NTN with channel BW less than 5MHz was discussed in the last meeting, the consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on those issues as follows.

# Discussion

#### Issue 1-2-1-2 CHO delay:

Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs]

Regarding CHO with L3 measurement, there are two parameters, T∆ and Tmeasure, are concerned to be extended due to the bandwidth limitation. Our concern in the last meeting is that since an additional time has been added for Tmeasure, the extension to T∆  may not be necessary.

Proposal 1:  Since of an additional time having been added for Tmeasure, the extension to T∆  may not be necessary.

#### Issue 1-2-4-2: How to define the new requirements for SSB time index detection

Agreements/Wayforward - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. - For X and Y, the values from TN less than 5MHz as starting point.

Proposal 2: For X and Y, the values from TN less than 5MHz can be used directly.

# Conclusion

Proposal 1:  Since of an additional time having been added for Tmeasure, the extension to T∆  may not be necessary.

Proposal 2: For X and Y, the values from TN less than 5MHz can be used directly.

# References

- R4-2416851, WF for RRM of NTN with channel BW less than 5MHz, Xiaomi