3GPP TSG-RAN WG4 Meeting #113	R4-2418903

Orlando, US, November 18th-22nd, 2024

Source: 	Ericsson

Title:  	Discussion on RRM requirements of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

Agenda Item:	7.27.3

Document for:	Discussion

# Introduction

The WI Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 targeting release Rel-19 was approved, which can be found in [1].

In this contribution, we present our views on the underlying impacts to RRM requirements mandated by the objectives in the WI with respect to the recent progress in RAN2 and SA2.

# Discussion

## Topic #1: Store&Forward

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2# 126 meeting, the agreements below were reached.

- For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again - S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed - RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In RAN2#127 meeting, the agreements below were reached.

- RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In RAN2#127bis meeting, the agreements below were reached.

- When present, the “S&F operation” indication has two possible settings: - ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) - ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In terms of the bunch of agreements in RAN2, to our understanding, in the S&F operation, relying on regenerative payload, the satellite may temporarily lose the feeder link connection and/or the service link connection. The S&F operation may be applied during the time span during the unavailable feeder link connection and/or service link connection respectively. The below are some examples of such S&F operation.

- For MO data, when the feeder link is unavailable and the service link is available, the satellite may store the UL data and forward the UL data to the NTN gateway when the feeder link becomes available.

- For MT data, when the feeder link is available and service link is invalid or UE is out of coverage, the satellite may store the DL data and later forward the DL data to UE when service link is valid or the UE enters the satellite coverage.

The UE is able to know whether the serving satellite is currently operating in S&F via System Information broadcast regarding RAN2 agreements.

The UE is likely to be able to know when the S&F mode will end and what satellite shall be attached after S&F, based on the information provided by SA2, which is listed as follows.

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires.The Satellite IDs are based on the SIB information broadcasted by eNB.

It appears that RAN4 has defined the requirements for the discontinuous coverage in Rel-18 IoT NTN, it is straightforward to check the requirements for S&F mode in a same way.

From RRM perspective, the impacted requirements may depend on at the least the below various scenarios which may occur in S&F mode:

- The feeder link is invalid.

- The service link is invalid.

- There is available data to be transmitted or received.

- There isn’t available data to be transmitted or received.

- The UE is in RRC_CONNECTED state.

- The UE is in RRC_IDLE/INACTIVATE state.

The impacted RRM requirements may contain at least the below:

- Keep/suspend/relax/resume measuring SSBs and monitoring paging or not.

- Update cell reselection rule for cells (the serving cell and/or neighbour cells)in S&F mode.

Observation 1: RRM requirements may be impacted by S&F mode, in a same manner as introducing requirements during the discontinuous coverage in Rel-18 IoT NTN.

Observation 2: The Impacted RRM requirements may include the following, depending on the specifics of S&F mode scenarios:

- Keep/suspend/relax/resume measuring SSBs and monitoring paging or not.

- Cell-reselection for cells (the serving cell and/or neighbour cells)in S&F mode.

# Conclusion

Observation 1: RRM requirements may be impacted by S&F mode, in a same manner as introducing requirements during the discontinuous coverage in Rel-18 IoT NTN.

Observation 2: The Impacted RRM requirements may include the following, depending on the specifics of S&F mode scenarios:

- Keep/suspend/relax/resume measuring SSBs and monitoring paging or not.

- Cell-reselection for cells (the serving cell and/or neighbour cells)in S&F mode.

# References

- R4-2416867, Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek