3GPP TSG-RAN WG4 Meeting #113	R4-2418943

# Orlando, US, 18 - 22 Nov, 2024

Title: 	Discussion on NTN testing for NGSO channel model

Source: 	Huawei,HiSilicon

Agenda item:	7.8.4

Document for:	Discussion

Background

During RAN4#112bis meeting, Way Forward [1] for NTN_testing_NGSO_channel_model was approved. In this contribution, we share our view on NTN testing for NGSO channel model.

Discussion

TE emulated channel model for satellite mobility

Methodology for Time varying Doppler and Delay shifts modelling

- RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay model matching a satellite motion with following option: - Satellite motion emulated using the two-body Keplerian propagator model, [i.e., spherical gravity model] - RAN4 further discuss how to derive the Doppler shift and propagation delay variation based on Keplerian model with the necessary parameters and details procedure - Companies are encouraged to provide the two-body Keplerian propagator model to be implemented in the TE, to derive the Doppler shift and propagation delay.

Here we provide an example procedure to derive the Doppler shift and propagation delay, as following Table 2.1.1-1.

Table 2.1.1-1 Doppler shift and propagation delay derivation procedure

Step 1: Set the coordinate system The coordinate system definition can be aligned with TS 38.331. The IE EphemerisInfo provides satellite ephemeris. Ephemeris may be expressed either in format of position and velocity state vector in ECEF or in format of orbital parameters in ECI. Note: The ECI and ECEF coincide at epochTime, i.e., x,y,z axis in ECEF are aligned with x,y,z axis in ECI at epochTime.. Step 2: Set constant values The constant values can be aligned with NIMA TR 8350.2 that is referenced by TS 38.331. Parameters; Notation; Unit; Value. earth semi-major axis; aE; m; 6378137.0. earth first eccentricity squared; eE2; -; 6.69437999014e-3. earth angular velocity; ωE; rad/s; 7292115.1467e-11. earth gravitational constant; μE; m3/s2; 3986004.418e8. velocity of light; c; m/s; 299792458. Step 3: Set input parameters For UE side, assuming UE is static relative to the earth, longitude λ, latitude ϕ, altitude hu should be the input parameters. For satellite side, initial ephemeris information should be the input parameters. To aligned with TS 38.331, either type of ephemeris as following may be expressed. - Position and velocity state vector in ECEF - position state vector s0=sx0, sy0, sz0T - velocity state vector v0=vx0, vy0, vz0T - Orbital parameters in ECI - semi major axis α - eccentricity e - argument of periapsis ω - longitude of ascending node Ω0 - inclination i - mean anomaly M0 In addition, the ephemeris transmission period Te and the carrier frequency fc should be the input parameters. Step 4: UE position transformation Transform UE longitude λ, latitude ϕ, altitude hu into position state vector in ECEF u=ux, uy, uzT N=aE1-eE2*sin2ϕ uxuyuz=N+hu*cosϕ*cosλN+hu*cosϕ*sinλN*1-eE2+hu*sinϕ , where N is the earth prime vertical radius of curvature. Step 5: Initial ephemeris information transformation (only in case that “Position and velocity state vector in ECEF” ephemeris type is indicated) Firstly, transform satellite position and velocity state vector in ECEF s0=sx0, sy0, sz0T, v0=vx0, vy0, vz0T into position and velocity state vector in ECI s'0=sx'0, sy'0, sz'0T, v'0=vx'0, vy'0, vz'0T. sx'0sy'0sz'0=sx0sy0sz0 vx'0vy'0vz't0=vx0-ωE*sy0vy0+ωE*sx0vz0 Secondly, transform satellite position and velocity state vector in ECI s'0=sx'0, sy'0, sz'0T, v'0=vx'0, vy'0, vz'0T into orbital parameters in ECI α,e,ω,Ω0,i,M0. x=1, 0, 0T y=0, 1, 0T z=0, 0, 1T h=s'0×v'0 e=v'0×hμE-s'0s'0 n=z×h φ0=arccose∙s'0es'0if s'0∙v'0>0 2π-arccose∙s'0es'0if s'0∙v'0<0 E0=2*arctan1-e1+e*tanφ02 , where h is the angular momentum vector, e is the eccentricity vector, n is the ascending node vector, φ is the true anomaly, E is the eccentric anomaly, then α=-μE2*v'022-μEs'0 e=e ω=arccose∙nenif e∙z>0 2π-arccose∙nenif e∙z<0 Ω0=arccosn∙xnif n∙y>0 2π-arccosn∙xnif n∙y<0 i=arccosh∙zhif h∙z>0 arccosh∙zh-πif h∙z<0 M0=E0-e*sinE0 Step 6: Propagation For each given time point t, calculate the true anomaly φt and the longitude of ascending node Ωt. Other orbital parameters are constant. Mt=M0+μEα3*tMt=Et-e*sinEt φt=2*arctan1+e1-e*tanEt2 Ωt=modΩ0-tTe*ωE*Te,2π If “Orbital parameters in ECI” ephemeris type is used, at the time point k*Te k=1,2,3…, the updated ephemeris information can be derived by orbital parameters in ECI α,e,ω,Ωk*Te,i,Mk*Te. Step 7: Satellite position transformation Firstly, transform satellite orbital parameters in ECI α,e,ω,Ωt,i,φt into position and velocity state vector in ECI s't=sx't, sy't, sz'tT, v't=vx't, vy't, vz'tT sx'tsy'tsz't=cosΩt-sinΩt0sinΩtcosΩt00011000cosi-sini0sinicosicosω-sinω0sinωcosω0001α*1-e2*cos(φ(t))1+e*cosφtα*1-e2*sin(φ(t))1+e*cosφt0 vx'tvy'tvz't=cosΩt-sinΩt0sinΩtcosΩt00011000cosi-sini0sinicosicosω-sinω0sinωcosω0001-μEα*1-e2*sinφtμEα*1-e2*e+cosφt0 Secondly, transform satellite position and velocity state vector in ECI s't=sx't, sy't, sz'tT, v't=vx't, vy't, vz'tT into position and velocity state vector in ECEF st=sxt, syt, sztT, vt=vxt, vyt, vztT sxtsytszt=cosωE*modt,TesinωE*modt,Te0-sinωE*modt,TecosωE*modt,Te0001sx'tsy'tsz't vxtvytvzt=cosωE*modt,TesinωE*modt,Te0-sinωE*modt,TecosωE*modt,Te0001vx't+ωE*sy'tvy't-ωE*sx'tvz't If “Position and velocity state vector in ECEF” ephemeris type is used, at the time point k*Te k=1,2,3…, the updated ephemeris information can be derived by position and velocity state vector in ECEF sk*Te=sxk*Te, syk*Te, szk*TeT, vt=vxk*Te, vyk*Te, vzk*TeT. Step 8: Calculate Doppler fdt and delay Tt trajectory fdt=fc*vt∙u-stc*u-st Tt=u-stc

Consider above Doppler shift and propagation delay derivation procedure in Table 2.1.1-1.

Assumption of Satellite altitude

- Prioritize the LEO-600 orbit for TE emulated channel modelling - FFS on the LEO-1200 orbit for TE emulated channel modelling pending on the test purpose

For RF and RRM testing, based on WID, there is only frequency error and uplink timing requirements to be tested. In our view, for above test purpose, it is enough to only consider LEO-600 orbit. For demodulation requirement testing, only LEO-600 orbit is considered in Rel-17 and Rel-18 since there is similar UE processing for LEO-600 orbit and LEO-1200 orbit from baseband implementation point of view. Therefore, we propose to not consider LEO-1200 orbit for all NGSO testing.

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

Do not consider LEO-1200 orbit for all NGSO testing.

Assumption of UE motion & UE position

- Same UE location information should be set for both DUT and TE before the test start with the following initial position - Assuming UE is stationary during the demodulation and RF requirement testing - FFS on the assuming UE is in motion during the RRM requirement testing

Based on WID, there is only uplink timing requirements to be tested for RRM, so it is not necessary to assume UE in motion during the RRM requirement testing.

Do not assume UE in motion during the RRM requirement testing.

Assumption of Elevation angle range

- Option 1 - In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE. - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator. - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test. - Option 2: To align with Rel-17 NR NTN channel model discussion, the elevation angle with 30 degrees and 90 degrees can be considered for testing. Large testing period is required for IoT NTN testing - Other options are not precluded.

In Rel-17 and Rel-18, only elevation angle of 30 degrees is considered. So we propose to align with Rel-17/Rel-18, i.e. only consider elevation angle around 30 degrees for all NGSO testing.

Only consider elevation angle around 30 degrees for all NGSO testing.

Assumption of updated granularity for Satellite ephemeris information

- Companies are encouraged to provide the view for RAN4#113 - Satellite ephemeris information will be updated at least every 10.24 seconds as the worst case according 38.331 - NOTE: other (smaller) value may be considered depending on the test purpose

In Rel-17, RRM requirements are defined with ephemeris information as 2.56s for NGSO, so we propose to use the same value to align with Rel-17 requirements.

Table A.3.36.2-1: Test cases for NTN specific requirements Parameter; Unit; Test 1. Interval between adjacent epoch time; s; LEO: 2.56 GEO: 10.24. ntn-UlSyncValidityDuration; s; LEO: 5 GEO: 900. cellSpecificKoffset; slot; LEO: 14 GEO: 258. kmac; slot; Not configured. ta-Common; 0. ta-CommonDrift; 0. ta-CommonDriftVariant; 0. ntn-PolarizationDL; linear. ntn-PolarizationUL; linear. ephemerisInfo; Detailed ephemeris information is provided in TS 38.508-1 [38]. ta-Report; Not configured.

For RF and demodulation requirements, we think 10.24s satellite ephemeris information is enough.

Use satellite ephemeris information 2.56s for RRM requirements.

Use satellite ephemeris information 10.24s for RF and demodulation requirements.

Assumption of Multiple satellite scenario

- RAN4 focuses TE emulated channel modeling for one satellite - FFS on the TE emulated channel modelling for multiple satellite scenario, considering the serving satellite changes.

For RF and demodulation requirements, it is not necessary to consider multiple satellite scenario. For RRM requirements, In Rel-17 and Rel-18, multiple satellite scenario is considered in some cases. However, those cases are out of scope since only uplink timing requirements should be considered based on WID. So, we propose to not consider multiple satellite scenario for all NGSO testing.

Do not consider multiple satellite scenario for all NGSO testing.

RAN4 requirements and tests

UE assumption on propagator model for satellite motion for RAN4 simulation purpose

- Option 1: Consider Keplerian model for UE assumption on the propagator model - Option 2: Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

In our view, UE side propagator model is up to UE implementation, it is not necessary to be aligned, so Option 2 can be adopted.

For UE assumption on propagator model for satellite motion for RAN4 simulation purpose, Option 2 can be adopted.

Inclusion in Specification

Specification of Channel Model

- RAN4 further discuss how to incorporate the channel model into specification pending on channel model discussion progress.

In HST related WI, the Doppler and delay trajectory is captured in the annex of TS 38.101-4. We think similar method can be reused, i.e. channel model with time varying Doppler and delay shift is captured into the annex of TS 38.101-5.

Channel model with time varying Doppler and delay shift is captured into the annex of TS 38.101-5.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Consider above Doppler shift and propagation delay derivation procedure in Table 2.1.1-1.

Do not consider LEO-1200 orbit for all NGSO testing.

Do not assume UE in motion during the RRM requirement testing.

Only consider elevation angle around 30 degrees for all NGSO testing.

Use satellite ephemeris information 2.56s for RRM requirements.

Use satellite ephemeris information 10.24s for RF and demodulation requirements.

Do not consider multiple satellite scenario for all NGSO testing.

For UE assumption on propagator model for satellite motion for RAN4 simulation purpose, Option 2 can be adopted.

Channel model with time varying Doppler and delay shift is captured into the annex of TS 38.101-5.

Reference

R4-2416556, Way Forward for [112bis][321] NTN_testing_NGSO_channel_model, RAN4#112bis, Samsung

RP-241985, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, RAN#105, Samsung