3GPP TSG-RAN WG4 Meeting #113	R4-2418944

# Orlando, US, 18 - 22 Nov, 2024

Title: 	Discussion on aperiodic CSI reporting requirements

Source: 	Huawei,HiSilicon

Agenda item:	4.6

Document for:	Discussion

Discussion

FR1 FDD aperiodic PMI reporting requirements less or equal to 4Tx

For FR1 FDD aperiodic PMI reporting requirements less or equal to 4Tx, there is conflict between the Note 2 and the CQI/RI/PMI delay. Based on CQI/RI/PMI delay in current TS 38.101-4 shown as below, the reported PMI is applied in slot#7. However, based on Note 2, the reported PMI should be applied as early as slot#8.

| Aperiodic Report Slot Offset |  | 4 |
| --- | --- | --- |
| CQI/RI/PMI delay | ms | 6 |

Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the gNB downlink before slot#(n+3).

Change CQI/RI/PMI delay from 6 ms to 7 ms for FR1 FDD aperiodic PMI reporting requirements less or equal to 4Tx.

FR1 FDD aperiodic PMI reporting requirements larger than 4Tx

For FR1 FDD aperiodic PMI reporting requirements larger than 4Tx, there is conflict between the Note 2 and the CQI/RI/PMI delay. Based on CQI/RI/PMI delay in current TS 38.101-4 shown as below, the reported PMI is applied in slot#9. However, based on Note 2, the reported PMI should be applied as early as slot#10.

| Aperiodic Report Slot Offset |  | 5 |
| --- | --- | --- |
| CQI/RI/PMI delay | ms | 8 |

Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4).

Change CQI/RI/PMI delay from 8 ms to 9 ms for FR1 FDD aperiodic PMI reporting requirements larger than 4Tx.

FR1 TDD aperiodic PMI reporting requirements less or equal to 4Tx

For FR1 TDD aperiodic PMI reporting requirements less or equal to 4Tx, there is conflict between the Note 2 and the CQI/RI/PMI delay. Based on CQI/RI/PMI delay in current TS 38.101-4 shown as below, the reported PMI is applied in slot#12. However, based on Note 2, the reported PMI should be applied as early as slot#13.

| Aperiodic Report Slot Offset |  | 8 |
| --- | --- | --- |
| CQI/RI/PMI delay | ms | 5.5 |

Note 2: If the UE reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4).

Change CQI/RI/PMI delay from 5.5 ms to 6 ms for FR1 TDD aperiodic PMI reporting requirements less or equal to 4Tx.

FR1 TDD aperiodic PMI reporting requirements larger than 4Tx

For FR1 TDD aperiodic PMI reporting requirements larger than 4Tx, there is conflict between the Note 2 and the CQI/RI/PMI delay. Based on CQI/RI/PMI delay in current TS 38.101-4 shown as below, the reported PMI is applied in slot#14. However, based on Note 2, the reported PMI should be applied as early as slot#15.

| Aperiodic Report Slot Offset |  | 8 |
| --- | --- | --- |
| CQI/RI/PMI delay | ms | 6.5 |

Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6).

Change CQI/RI/PMI delay from 6.5 ms to 7 ms for FR1 TDD aperiodic PMI reporting requirements larger than 4Tx.

FR2 TDD aperiodic CSI reporting requirements with DDSU TDD pattern

For FR2 TDD aperiodic CSI reporting requirements with DDSU TDD pattern, there is conflict between the TS 38.214 and the CQI/RI/PMI delay. Based on CQI/RI/PMI delay in current TS 38.101-4 shown as below, the CSI is reported in slot#7. However, based on the TS 38.214, the CSI should be reported as early as slot#9.

| Aperiodic Report Slot Offset |  | 6 |
| --- | --- | --- |
| CQI/RI/PMI delay | ms | 1.375 |

- (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI' Table 5.4-2: CSI computation delay requirement 2 Z1 [symbols]; Z2 [symbols]; Z3 [symbols]. Z1; Z'1; Z2; Z'2; Z3; Z'3. 0; 22; 16; 40; 37; 22; X0. 1; 33; 30; 72; 69; 33; X1. 2; 44; 42; 141; 140; min(44, X3X3+ KB1); X2. 3; 97; 85; 152; 140; min(97, X3+ KB2); X3. 5; 388; 340; 608; 560; min(388, X5+ KB3); X5. 6; 776; 680; 1216; 1120; min(776, X6+ KB4); X6.

Change Aperiodic Report Slot Offset from 6 slots to 10 slots for FR2 TDD aperiodic CSI reporting requirements with DDSU TDD pattern.

Change CQI/RI/PMI delay from 1.375 ms to 2.375 ms for FR2 TDD aperiodic CSI reporting requirements with DDSU TDD pattern.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Change CQI/RI/PMI delay from 6 ms to 7 ms for FR1 FDD aperiodic PMI reporting requirements less or equal to 4Tx.

Change CQI/RI/PMI delay from 8 ms to 9 ms for FR1 FDD aperiodic PMI reporting requirements larger than 4Tx.

Change CQI/RI/PMI delay from 5.5 ms to 6 ms for FR1 TDD aperiodic PMI reporting requirements less or equal to 4Tx.

Change CQI/RI/PMI delay from 6.5 ms to 7 ms for FR1 TDD aperiodic PMI reporting requirements larger than 4Tx.

Change Aperiodic Report Slot Offset from 6 slots to 10 slots for FR2 TDD aperiodic CSI reporting requirements with DDSU TDD pattern.

Change CQI/RI/PMI delay from 1.375 ms to 2.375 ms for FR2 TDD aperiodic CSI reporting requirements with DDSU TDD pattern.