3GPP TSG-RAN WG4 Meeting #113	  R4-2418968

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.17.1

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects on AIML for NR air interface

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #112 [1], we discuss the remaining issues related to the general aspects on AIML for NR air interface.

# AI testing goal

RAN4 #112 WF [1] The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

We observe that there is no sufficient progress on the definition of the functionality/feature granularity from other WGs. If the functionality is specified by a set of configurations/parameters, RAN4 can choose the typical value of the associated configurations/parameters to define the test cases. The selection of the typical value needs to be discussed per use case.

For cases where the set of configurations/parameters related to AI is a subset of or the same as the configurations/parameters related to non-AI, the test configurations/parameters for AI can reuse legacy.

Proposal 1: If the AI related configurations/parameters is a subset of (or the same as) the legacy test configurations/parameters, take the existing test configurations/parameters as a starting point.

RAN4 #112bis WF [2] For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy FFS on target for AI/ML based beam management

For cases where there exists legacy requirement, for example the CSI prediction, AI enabled DUT should not be worse than the legacy requirements since the intention of introducing AI is to bring benefits.

For cases where the legacy test environment is not applicable, potential new or enhanced test environment/conditions may be needed.  For example, in AI/ML BM, larger number than two of beams is under discussion.

Observation 1: For some cases, to verify AI performance, new test set up is under discussion.

To define AI requirements in cases where new test set up is introduced, two options are identified.

- Option 1: Define non-AI requirements under new test set up subject to AI. AI requirement has to be higher than that of non-AI.

- Option 2: Define absolute requirement of AI without considering the comparison with non-AI.

Both solutions have open issues. For Option 1, the feasibility of aligning non-AI performance baseline under new test environment subject to AI needs to be investigated. Though it may be feasible, heavy workload is expected. For Option 2, without considering the comparison to non-AI, how to verify the AI benefits needs to be resolved.

Proposal 2: If new test set up is introduced for AI test, two options are identified for defining AI performance requirement.

- Option 1: Define non-AI requirements under new test set up subject to AI. AI requirement has to be higher than that of non-AI.

  - FFS: Feasibility of aligning non-AI performance baseline under new test set up

- Option 2: Define absolute requirement of AI without considering the comparison with non-AI.

  - FFS: How to verify the AI benefits.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: If the AI related configurations/parameters is a subset of (or the same as) the legacy test configurations/parameters, take the existing test configurations/parameters as a starting point.

Observation 1: For some cases, to verify AI performance, new test set up is under discussion.

Proposal 2: If new test set up is introduced for AI test, two options are identified for defining AI performance requirement.

- Option 1: Define non-AI requirements under new test set up subject to AI. AI requirement has to be higher than that of non-AI.

  - FFS: Feasibility of aligning non-AI performance baseline under new test set up

- Option 2: Define absolute requirement of AI without considering the comparison with non-AI.

  - FFS: How to verify the AI benefits.

# References

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #111, May 20th – 24th, 2024.

- R4-2417212, “WF on AI/ML”, 3GPP RAN4 #112bis, Oct 14th – 18th, 2024.

- RP-233133, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, 3GPP TR 38.843, V2.0.0, December 11-15, 2023.