3GPP TSG-RAN WG4 Meeting #113	R4-2418976

Orlando, United States, 18th - 22nd November, 2024

Title:	Discussion on circular polarization for NTN OTA test

Source:	OPPO

Agenda item:	7.12.2.2

Document for:	Approval

- Introduction

The test method related to the circular polarization for NTN OTA was discussed during the past two RAN4 meetings. The consensus was not reached so far, and the WF was captured as below [1].

This contribution presents our considerations and views on whether the circular polarization is needed for NTN OTA test system.

- Discussion

The discussion on the need for circular polarization for NTN OTA is divided into two parts, i.e. Downlink (EIS) and Uplink (EIRP).

For Downlink measurement of OTA test system, the DL signal is emitted by the measurement antenna/probe of OTA chamber, and received by DUT. The discussion is that, for NTN OTA, the measurement antenna/probe should be right-hand or left-hand circular polarization antenna or orthogonal linear polarization antenna. Refer to the experience of GNSS test systems, which are already used worldwide for many years, the two orthogonal linear polarization antennas are used for transmitting DL GNSS signals. This GNSS test method is widely used for device measurement and GNSS certification. From this observation, orthogonal linear polarization antennas can be used for DL performance measurement, i.e. EIS, for NTN OTA test.

Observation 1: GNSS OTA test system has used the two orthogonal linear polarization antennas for DL measurement for many years.

For Uplink measurement, DUT radiates the output signal towards the measurement antenna/probe of OTA chamber, the measurement antenna/probe receives the signal and transmits the power to signal analyzer or call box for power measurement. The table below shows the measured power using orthogonal linear polarization antennas when the UE transmits with different types of polarizations. It can be found that the radiated power by UE can be measured correctly with orthogonal linear polarization measurement antennas.

| UE polarization | Receive with Vertical Polarization | Receive with Horizontal Polarization | Sum of Vertical and Horizontal Polarization |
| --- | --- | --- | --- |
| Vertical Polarization | 0dB | -∞ | 0dB |
| Horizontal Polarization | -∞ | 0dB | 0dB |
| Left-Hand Circular Polarization | -3dB | -3dB | 0dB |
| Right-Hand Circular Polarization | -3dB | -3dB | 0dB |

Observation 2: The radiated power by UE can be measured correctly with orthogonal linear polarization measurement antennas.

From the above observations, circular polarization is not necessary for both DL and UL measurement. and it is proposed to apply two orthogonal linear polarization antennas/probes for NTN OTA test.

Proposal: Apply two orthogonal linear polarization antennas/probes for NTN OTA test.

- Conclusions

This contribution presents our considerations and views on whether the circular polarization is needed for NTN OTA test system. And provide the following proposals.

Observation 1: GNSS OTA test system has used the two orthogonal linear polarization antennas for DL measurement for many years.

Observation 2: The radiated power by UE can be measured correctly with orthogonal linear polarization measurement antennas.

Proposal: Apply two orthogonal linear polarization antennas/probes for NTN OTA test.

- References

[1] R4-2416590 Way Forward for [112bis][323] TRP_TRS_Ph3, vivo