3GPP TSG-RAN WG4 Meeting #113	R4-2418977

Orlando, United States, 18th - 22nd November, 2024

Title:	Discussion on target value of dynamic path gain for UMi route

Source:	OPPO

Agenda item:	7.12.2.3

Document for:	Approval

- Introduction

The TP [1] for TR 38.762 defining dynamic channel model and validation target values was approved in RAN4 #112bis meeting. And it is found that the target values of dynamic path gain for UMi route are not normalized.

This contribution proposes to update the normalized target values of dynamic path gain for Umi route accordingly by CE vendors.

- Discussion

The current endorsed target values of dynamic path gain for UMi route are not apparently normalized as shown above. If we simply normalize the values with the target value of Segment 2 which is the largest one equals to 0, we get the following table.

| Segment # | Target [dB] |
| --- | --- |
| 1 | [18][-3.7] |
| 2 | [21.7][0] |
| 3 | [8.6][-13.1] |
| 4 | [8.1][-13.6] |
| 5 | [8.5][-13.2] |
| 6 | [21.1][-0.6] |
| 7 | [20.7][-1] |

These target values are not well consistent with the figure provided in the TP as shown below, where the values of the third and fifth segments are lower than -15 dB.

Given this situation, it is proposed that the CE vendors should update the normalized target values of dynamic path gain for UMi route in Table 8.2.2.4-2 and potentially together with Figure 8.2.2.4-2.

Proposal: It is proposed that the CE vendors should update the normalized target values of dynamic path gain for UMi route in Table 8.2.2.4-2 and potentially together with Figure 8.2.2.4-2.

- Conclusions

This contribution presents the non-normalization and misalignment on the target values of dynamic path gain for UMi route. And provide the following proposals.

Proposal: It is proposed that the CE vendors should update the normalized target values of dynamic path gain for UMi route in Table 8.2.2.4-2 and potentially together with Figure 8.2.2.4-2.

- References

[1] R4-2416578 TP on Dyn MIMO, Keysight Technologies UK Ltd, Spirent Communications