3GPP TSG-RAN WG4 Meeting # 113	R4-2418978

# Orlando, US, 18th – 22th November, 2024

Source:	LG Electronics

Title:	Simultaneous operation of Ambient IoT and Uu in intermediate UE

Agenda item:	7.21.3.3

Document for:	Discussion

Introduction

RF requirements discussion started in RAN4 #112. We are also currently focusing on D1T1 and D2T2 scenarios, where the intermediate node comes from the D2T2 scenario. The intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT as specified in TR38.848. So Intermediate User Equipment (UE) for Ambient IoT can operate with Uu interfaces such as NR and LTE. In this perspective, we want to share view on Ambient IoT Intermediate node. This contribution is the resubmission [2] since it was not discussed in RAN4 #112-bis due to the shortage of time.

- Discussion

If the concurrent operation of the A-IoT and Uu interface in intermediate UE is considered, the Uu UL and the A-IoT signals (CW2D and R2D) can be considered in the following two scenarios.

- Scenario 1: TDM for Uu UL and A-IoT signal (CW and R2D)

- Scenario 2: Simultaneous Uu UL and A-IoT(CW and R2D) signal configuration

For Scenario 1, since the Uu UL signal and A-IoT signal are configured separately at different times, there is no impact on DL performance. However, in the case of scenario 2, it is similar to CA/DC operation, so it may affect DL performance. Therefore, if scenario 2 is considered in concurrent operation, relevant requirements should be defined by considering cases such as Example 1 and Example 2 below. The relevant requirements can reuse the CA/DC MSD requirement approach.

Example 1

Aggressor: A-IoT(CW or R2D), legacy service UL (ex.NR) Victim: legacy service DL

Example 2

Aggressor:Legacy service UL(ex. NR) Victim: A-IoT D2R

Observation 1: If the concurrent operation of the A-IoT and Uu interface in intermediate UE is considered, the Uu UL and the A-IoT signals (CW2D and R2D) can be considered in the following two scenarios.

-	Scenario 1: TDM for Uu UL and A-IoT signal (CW and R2D)

-	Scenario 2: Simultaneous Uu UL and A-IoT(CW and R2D) signal configuration

Observation 2: For Scenario 1, since the Uu UL signal and A-IoT signal are configured separately at different times, there is no impact on DL performance. However, in the case of Scenario 2, it is similar to CA/DC operation, so it may affect DL performance.

Proposal: It is necessary to study which scenario can be considered for concurrent operation of the A-IoT and Uu interface in intermediate UE.

- Option 1: Scenario 1 & Scenario 2

- Option 2: Scenario 1

- Option 3: Scenario 2

- Option 4: Do not consider concurrent operation

- Option 5: TBD

- Conclusion

In this contribution, we want to share view on Ambient IoT Intermediate node and proposal is made as following:

Observation 1: If the concurrent operation of the A-IoT and Uu interface in intermediate UE is considered, the Uu UL and the A-IoT signals (CW2D and R2D) can be considered in the following two scenarios.

-	Scenario 1: TDM for Uu UL and A-IoT signal (CW and R2D)

-	Scenario 2: Simultaneous Uu UL and A-IoT(CW and R2D) signal configuration

Observation 2: For Scenario 1, since the Uu UL signal and A-IoT signal are configured separately at different times, there is no impact on DL performance. However, in the case of Scenario 2, it is similar to CA/DC operation, so it may affect DL performance.

Proposal: It is necessary to study which scenario can be considered for concurrent operation of the A-IoT and Uu interface in intermediate UE.

- Option 1: Scenario 1 & Scenario 2

- Option 2: Scenario 1

- Option 3: Scenario 2

- Option 4: Do not consider concurrent operation

- Option 5: TBD

- 3. Reference

[1] TR38.848, “Study on Ambient IoT (Internet of Things) in RAN”

[2] R4-2416352, “Simultaneous operation of Ambient IoT and Uu in intermediate UE”