3GPP TSG RAN WG4 Meeting #113		R4-2418979

Orlando, Nov.18th – 22nd, 2024

Source: 	Nokia

Title: 	IoT-NTN HPUE RF requirements for Tx

Agenda item:       7.8.2.3.1

Document for:     Discussion

# 1	Introduction

Based on the WF on IoT-NTN HPUE RF requirements from RAN4 #112bis meeting [1]. In this paper, we’d like to share our views :

RAN4 WF from #112bis meeting: ### SAR related Issue 1-3-1: Duty cycle based approach - Proposals - Option 1: Support the study of this direction. - Other solutions than P-MPR need to be specified for HPUE in NTN to ensure a sufficient uplink budget. (Sony) - RAN4 shall check if the duty cycle-based approach for SAR can be applied in NTN system. (Sony) - Further investigate the duty cycle method for regulation of NTN hand-held UE (oppo) - Consider applying the current duty cycle-based SAR solution to Iot based NTN. (vivo) - Option 2: Not support this direction. - For SAR requirements for handheld UEs, only P-MPR mechanism shall be used, as it is the case for band n14 in LTE TN. (Ericsson) - In addition, regarding the SAR requirement for FDD NTN HPUE, we could follow the legacy approach to leave it up to p-MPR design instead of defining any uplink duty cycle to guarantee the SAR limits. (ZTE) - Option 3: Others - The duty cycle based SAR solution may not be applicable to IoT-NTN cases, as the IoT-NTN uplink duty cycle for each UE (or for each cell/beam) can be up to network implementation. (Samsung) Agreement: Continue the effort to study other potential solutions in addition to P-MRP solution Issue 1-3-2: Network related - Proposals - Proposal 1: If the SAN can send its configured uplink ratio for a particular UE or cell or beam, the NTN HPUE should be allowed to transmit with its max Tx power as much as possible, considering the uplink transmission in a NTN scenario can be critical but short in time compared to 6mins SAR regulatory evaluation period. (Samsung) - Proposal 2: Send RAN1 LS to check whether the network scheduling can be enhanced to guarantee NTN UE comply with SAR requirements without NTN UE output power reduction. (Huawei) - Recommended WF - FFS

# 2	Discussion

2.1	Discussion for SAR related issues

In RAN4 #112-is meeting, the duty cycle, P-MPR, network signaling has been discussed to reduce SAR for the handheld IoT NTN HP UE.

First of all, to satisfy the SAR requirement, each UE may apply different methods, i.e., P-MPR, duty cycles. How much the P-MPR or the ratio of duty cycles should apply to satisfy SAR requirement are different for different UEs. Only UE itself known the details and this can only be determined based on SAR test. The network has no prior knowledge of those. Therefore, it is not reasonable for network to send the control message to control SAR of each UE.

Observation 1: Only UE itself known the details of how to applying duty cycle or P-MPR to satisfy SAR requirement, which can only be determined based on SAR test. The network has no prior knowledge of those.

Proposal 1: It is not reasonable for network to send the control message to control SAR of each UE.

Regarding control duty cycle or applying P-MPR of the Tx chain are both effective methods of reduce the SAR. However, we can see both have drawback.

- Controlling duty cycle to a lower radio will reduce the overall throughput even the UE is far away from human hand or head. If the duty cycle cannot be dynamically controlled, then the throughput/data-rate during the communication will be impact significantly.

- Applying P-MPR, will reduce the power of HP UE. If adding too much P-MPR, the advantage of using HP UE will be invalidated. There is no difference between HP UE and normal power UE.

For NTN communication, the signal strength is a big issue, therefore, using a relatively higher Tx power to maintain the connection between NTN and UE can be prioritized. So, in principle, applying P-MPR cannot invalidate the purpose of letting IoT-NTN UEs have high Tx power as PC1/PC1.5/PC2 devices do. This limits P-MPR to be a small number. If the SAR specification still cannot be satisfied after applying P-MPR, then duty cycle control can be used to further reduce SAR.

Therefore, in order to both maintain high Tx power and reduce the SAR, the combination of using duty cycle and small P-MPR can be considered in practice.

Observation 2: There are drawbacks of applying duty cycle or P-MPR to control SAR.

Observation 3: If the duty cycle cannot be dynamically controlled, then the throughput will be impact. Adding too much P-MPR, the advantage of using HP UE will be invalidated.

Proposal 2: In principle, applying P-MPR cannot invalidate the purpose of letting IoT-NTN UEs have high Tx power as PC1/PC1.5/PC2 devices do.

Proposal 3: The combination of using duty cycle and small P-MPR can be considered for high power UE to meet SAR requirements.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: Only UE itself known the details of how to applying duty cycle or P-MPR to satisfy SAR requirement, which can only be determined based on SAR test. The network has no prior knowledge of those.

Proposal 1: It is not reasonable for network to send the control message to control SAR of each UE.

Observation 2: There are drawbacks of applying duty cycle or P-MPR to control SAR.

Observation 3: If the duty cycle cannot be dynamically controlled, then the throughput will be impact. Adding too much P-MPR, the advantage of using HP UE will be invalidated.

Proposal 2: In principle, applying P-MPR cannot invalidate the purpose of letting IoT-NTN UEs have high Tx power as PC1/PC1.5/PC2 devices do.

Proposal 3: The combination of using duty cycle and small P-MPR can be considered for high power UE to meet SAR requirements.

.

# References

- R4-2417205, WF on HPUE for IoT NTN