3GPP TSG-RAN WG4 Meeting #113	R4-2418986

Orlando, FL, 18 November – 12 November 2024

Agenda Item:	7.8.4

Source:	Ericsson

Title:	TE-emulated channel model for NTN UE tests

Document for:	Discussion

# 1	Introduction

RAN4#112bis agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. In the WF, RAN4 first focus on the varying Doppler and delay shift models.

WF: - RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay model matching a satellite motion with following option: - Satellite motion emulated using the two-body Keplerian propagator model, [i.e., spherical gravity model] - RAN4 further discuss how to derive the Doppler shift and propagation delay variation based on Keplerian model with the necessary parameters and details procedure - Companies are encouraged to provide the two-body Keplerian propagator model to be implemented in the TE, to derive the Doppler shift and propagation delay.

This contribution presents on the satellite propagator model and how to derive the time varying Doppler shift and propagation delay.

# 2	NGSO NTN test environment

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 illustrates the overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite ephemeris information and UE’s location information at a given time. The UE location information is pre-configured or provided via AT command, according to TS 38.508 (See A.3). The estimated Doppler shift and propagation delay are used to perform the pre-compensation before the baseband processing in the NTN UE.

As agreed in RAN4#112bis, the TE decides the satellite position and velocity based on the Keplerian propagator model. The TE derives the Doppler shift and propagation delay based on the satellite position/velocity and UE location at a given time and applies them to send signals before feeding to the fading simulator. The TE should also know the configured UE location information, given by longitude, latitude, and altitude. The determined satellite position/velocity information is signaled to UE periodically with SIB19 ephemerisInfo-r17 for NR or SIB31 ephemerisInfo-r17 for IoT-NTN periodically together with epochTime.

# 3	Doppler shift and propagation delay model

This section discusses the procedure to generate the varying Doppler shift and propagation delay at time t. The procedure consists of 1) decide the satellite position/velocity, 2) decide UE the position, and 3) derive the Doppler shift and propagation delay.

## 3.1	Determine Satellite position and velocity

This sub-procedure decides the satellite position/velocity using the Keplerian propagator model (e.g., [3]). The satellite position/velocity is given in either position/velocity state vectors or orbital parameters. Since the Kepler’s equation is expressed by the orbital parameters, if the satellite position/velocity information is given in the state vector format, we first need to convert the position/velocity state vectors to the orbital elements to derive the eccentric anomaly. After calculating the eccentric anomaly at time t by solving the Keplerian equation, we convert back to the position/velocity state vector format.

The following tables list the input/output values and required parameter in this procedure.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| r0=v0,xv0,yv0,z | km | Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| v0=v0,xv0,yv0,z | km/s | Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| rt=rt,xrt,yrt,z | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vt=vt,xvt,yvt,z | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004 x 105 | km2/s2 |

### Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

#### Step1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0=r0,x2+r0,y2+r0,z2

v=v0=v0,x2+v0,y2+v0,z2

vr=v0⋅r0r=r0,xv0,x+r0,yv0,y+r0,zv0,zr

h=hxhyhz=r×v=r0,yv0,z-r0,zv0,yr0,zv0,x-r0,xv0,zr0,xv0,y-r0,yv0,x

h=h=hx2+hy2+hz2

#### Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

#### Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17.

#### Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0-r0⋅v0v0=1μv2-μrr0,xr0,yr0,zT-r⋅vrv0,xv0,yv0,zT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

#### Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

#### Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0e⋅r=cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr≥02π-cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

### Step 2	Determine the satellite position and velocity at time t (sec)

#### Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

#### Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-3-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-3-5: If Et(n+1)-Et(n)<ϵ, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Note ϵ=10-6, for example.

Note n=0, 1, 2, …

#### Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

#### Step 2-4	Convert the orbital elements to the state vector

rt,xwrt,ywrt,zw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0

vt,xwvt,ywvt,zw=μh-sinνt 1+cosνt0

rt,xrt,yrt,z=cos-ω-sin-ω0sin-ωcos-ω00011000cos-i-sin-i0sin(-i)cos(-i)cos-Ω-sin-Ω0sin-Ωcos(-Ω)0001rt,xwrt,ywrt,zw

vt,xvt,yvt,z=cos-ω-sin-ω0sin-ωcos-ω00011000cos-i-sin-i0sin(-i)cos(-i)cos-Ω-sin-Ω0sin-Ωcos(-Ω)0001vt,xwvt,ywvt,zw

## 3.2	Determine UE position

This step calculates the UE position according to the earth angular speed. Since the UE location is configured with a format of latitude, longitude, and altitude, we need to convert the values to ECEF format to align with the satellite position/velocity state vectors.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xUEt,yUEt,z | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.694385 x 10-3 |  |
| ωE | Earth angular speed | 7.292115 x 10-5 | rad/s |

ϕ=π180UElatitude

λ0=π180UElongitude

N=aE1-eE2sin2ϕ

Derive the longitude of UE at time t considering the earth angular.

λt=mod(λ0+ωE, 2π)

UEt,x=N+UEaltitudecosϕcosλt UEt,y=N+UEaltitudecosϕsinλtUEt,z=N1-eE2+UEaltitudesinϕ

Note the range of λt is between 0 and 2π (radian).

## 3.3	Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xUEt,yUEt,z | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| rt=rxtrytrzt | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vt=vxtvytvzt | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| Delay | sec | Propagation delay at time t |

Constant parameters:

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 300000 | km/s |

ρt=ρt,xρt,yρt,z=rt,x-UEt,xrt,y-UEt,yrt,z-UEt,z

Range(t)=ρt=ρt,x2+ρt,y2+ρt,z2

RangeRate(t)=rt⋅vtRnage(t)=rt,x⋅vt,x+rt,y⋅vt,y+rt,z⋅vt,zRange(t)

Delay(t)=Range(t)c

Doppler(t)=-RangeRate(t)×fcc

## 3.4	Elevation angle

RAN4#112bis discussed the range of elevation angles of the satellite from the UE viewpoint as follows:

WF - Option 1 - In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE. - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator. - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test. - Option 2: To align with Rel-17 NR NTN channel model discussion, the elevation angle with 30 degrees and 90 degrees can be considered for testing. Large testing period is required for IoT NTN testing - Other options are not precluded.

The elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

et=-sinλtcosλt0, nt=-sinϕcos λt-sinϕsin λtcosϕ, ut=cosϕcosλtcosϕsinλtsinϕ

Note et, nt, and ut are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρt⋅et=ρt,xet,x+ρt,yet,y+ρt,zet,z

lt,n=ρt⋅nt=ρt,xnt,x+ρt,ynt,y+ρt,znt,z

lt,u=ρt⋅ut=ρt,xut,x+ρt,yut,y+ρt,zut,z

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

## 3.5	Examples

This section shows three examples of the Doppler shift and propagation delays derived from the procedure presented above.

### 3.5.1	Example 1: NGSO (LEO-600) satellites (zero Doppler conditions)

This example refers to the NGSO (LEO-600) satellites (zero Doppler conditions) specified in TS 38.508 Table 5.6.2.1-2. Table 1 shows the configured satellite position/velocity state vectors and UE location at time 0.

Table 1	Initial Satellite/UE location for NGSO (LEO-600) satellites (zero Doppler conditions).

| Parameters | Values |
| --- | --- |
| r0 | [-3315.1053, 5396.4261, 2941.9442] (km) |
| v0 | [1.3182, -2.88984, 6.84654] (km/s) |
| UE Latitude | 25.08439333 (deg) |
| UE Longitude | 121.56076999 (deg) |
| UE Altitude | 0 (m) |
| fc | 2 (GHz) |

Figure 2 shows the elevation angle of the satellite position from the UE in this scenario. As mentioned in TS 38.508 Table 5.6.2.1-3 Note 1, the elevation angle of the satellite position is 89.54 degrees at time 0.

Figure 2	Elevation angle for NGSO (LEO-600) satellites (zero Doppler conditions).

Figure 3 shows (a) propagation delay and (b) Doppler shift. As mentioned in TS 38.508 Table 5.6.2.1-3 Note 1, the one-way delay equal to 2.03 ms and Doppler shift is almost 0 Hz (0.013 ppm = -26Hz@2GHz) at time 0.

| - Propagation delay | - Doppler shift |
| --- | --- |

Figure 3	Propagation delay and Doppler shift for NGSO (LEO-600) satellites (zero Doppler conditions).

### 3.5.2	Example 2: NGSO (LEO-600) satellites (maximum positive Doppler)

This example refers to the NGSO (LEO-600) satellites (maximum positive Doppler) specified in TS 38.521-5 Table 6.4.1_2.4.3-1. Table 2 shows the configured satellite position/velocity state vectors and UE location at time 0.

Table 2	Initial Satellite/UE location for NGSO (LEO-600) satellites (maximum positive Doppler)

| Parameters | Values |
| --- | --- |
| r0 | [-3532.9021, 5915.5447, 1108.6387] (km) |
| v0 | [0.36984, -1.16544, 7.45686] (km/s) |
| UE Latitude | 25.08439333 (deg) |
| UE Longitude | 121.56076999 (deg) |
| UE Altitude | 0 (m) |
| fc | 2 (GHz) |

Figure 4	Elevation angle for NGSO (LEO-600) satellites (maximum positive Doppler).

| - Propagation delay | - Doppler shift |
| --- | --- |

Figure 5	Propagation delay and Doppler shift for NGSO (LEO-600) satellites (maximum positive Doppler).

Figure 4 is the elevation angle of the satellite position from the UE in this scenario, and Figure 5 show the propagation delay and Doppler shift. As mentioned in TS 38.521-5 Table 6.4.1_2.4.3-1 Note 1, the elevation angle of the satellite position is 10 degrees, the one-way delay equal to 6.44 ms and Doppler shift equal to 22.65 ppm (= +45.3kHz@2GHz) at time 0.

### 3.5.3	Example 3: NGSO (LEO-600) satellites (maximum negative Doppler)

This example refers to the NGSO (LEO-600) satellites (maximum positive Doppler) specified in NGSO (LEO-600) satellites (maximum negative Doppler). Table 3 shows the configured satellite position/velocity state vectors and UE location at time 0.

Table 3	Initial Satellite/UE location for NGSO (LEO-600) satellites (maximum negative Doppler).

| Parameters | Values |
| --- | --- |
| r0 | [-2859.0536, 4425.4977, 4596.5322] (km) |
| v0 | [2.12364, -4.46484, 5.68092] (km/s) |
| UE Latitude | 25.08439333 (deg) |
| UE Longitude | 121.56076999 (deg) |
| UE Altitude | 0 (m) |
| fc | 2 (GHz) |

Figure 6	Elevation angle for NGSO (LEO-600) satellites (maximum negative Doppler).

| - Propagation delay | - Doppler shift |
| --- | --- |

Figure 7	Propagation delay and Doppler shift for NGSO (LEO-600) satellites (maximum negative Doppler).

Figure 6 is the elevation angle of the satellite position from the UE in this scenario, and Figure 7 show the propagation delay and Doppler shift. As mentioned in TS 38.521-5 Table 6.4.1_2.4.3-2 Note 1, the elevation angle of the satellite position is 10.03 degrees (169.97 degrees), the one-way delay equal to 6.60 ms and Doppler shift equal to -22.62 ppm (=-45.24kHz@2GHz) at time 0.

## 3.6		Discussion

We have shown the varying Doppler shift and propagation delay based on the Keplerian satellite propagator model and UE location. It is important to note that we have considered the earth rotation when the UE position state vector at time t is calculate in step 3.2, although it is assumed the UE is stationary during the test.

Observation 1: Although it is assumed that UE is stationary during the tests, it is necessary to consider the earth rotation when we derive the Doppler shift and propagation delay between the satellite and UE.

Proposal 1:	The TE-emulated varying Doppler shift and propagation delay should be derived with the assumption the DUT (UE) considers the earth rotation when it estimates the Doppler shift and propagation delay.

Proposal 2:	RAN4 should consider the procedure presented in this contribution to specify the varying Doppler shift and propagation delay model used for NTN UE tests.

# 4	Other open issues

## 4.1	Parameters for TE-emulated channel models

WF - Option 1 - In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE. - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator. - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test. - Option 2: To align with Rel-17 NR NTN channel model discussion, the elevation angle with 30 degrees and 90 degrees can be considered for testing. Large testing period is required for IoT NTN testing - Other options are not precluded.

WF - RAN4 further discuss on general test principle and how to generate channel model to match with satellite motion trajectory with the following options - Option 1: The entire N orbits within a test time of X seconds are generated as follow - The initial ephemeris for each orbit is generated by either GMAT or any numerical method. - The initial ephemeris for each orbit should be chosen such that the elevation angle from the UE perspective is not smaller than Z degrees. The value of Z may vary for different test cases. And if multiple satellites (i.e. N > 1) are needed in RRM mobility/measurement test cases, the value of Z may be different for different satellites. - The ephemeris data after the initial data is generated based on the two-body Keplerian model at every Y sec. The value of Y may differ for NR NTN and IoT NTN. - The propagation delay and Doppler shift at every Y sec are derived based on the projected ephemeris data and given UE location - In case Y is larger than the slot length of interest in the test case due to, e.g., test equipment/channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying, e.g., linear interpolation between the propagation delays and Doppler shifts at Y sec intervals. - Other options are not precluded

Once RAN4 decide the TE-emulated varying Doppler shift and propagation delay model, the next step is the parameters (scenarios) specified in the performance requirements. According to our understanding, the Doppler shift and propagation delay are affected by the following parameters:

- Configured UE location (Longitude, Latitude, Altitude) at time 0

- Configured satellite position and velocity state vectors at time 0

- Test time (range of elevation angles)

Regarding the configured UE location, RAN5 has already specified the parameters as shown in A.3. We propose to reuse it. Regarding the satellite location/velocity state vector at time 0, RAN5 has also specified the parameter sets as shown in A.1 and A.2. We think the parameters with the NGSO (LEO-600) satellites (maximum positive Doppler) specified in TS 38.521-5 Table 6.4.1_2.4.3-1 is a good candidate to use to configure at time 0. The reasons are because 1) it covers the elevation angles range from 10 to 80 degrees, and can realize the Doppler shift changes from +22ppm to -22ppm (See Figure 4 and Figure 5).

In the last meeting, it was proposed to consider N satellites within a test time of X seconds and choose the serving satellite so that the elevation angle is within Z degrees. We understand this is more generic, however it makes the TE implementation more complex, and the test results might be different depending on the TE implementation. Moreover, setting the satellite position/velocity vectors at time 0 is not simple task. We therefore prefer the RAN4 specification sets the satellite position/velocity at time 0, like the high-speed train frequency offset modelling.

Observation 2: With LEO600 satellite (maximum positive Doppler) parameters, it takes about 500 seconds to cover more than 10 degrees in elevation angle,

Observation 3: With LEO600 satellite (maximum positive Doppler) parameters, it takes about 250 seconds to cover more than 30 degrees in elevation angle.

Proposal 3: RAN4 specification should set one (or more) satellite position/velocity at time zero.

Proposal 4: Set the configured UE location to Longitude: 121.56076999, Latitude: 25.08439333, and Altitude: 0.0 as same as TS 38.508-1.

Proposal 5: Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 as a starting point to specify the TE-emulated Doppler shift and propagation delay model.

Proposal 6: RAN4 discuss further the elevation angle (range) of the satellite position from the UE considering the test time, e.g., more than 30 degrees.

## 4.2	Impact to RAN4 performance requirements

WF - Option 1: Consider Keplerian model for UE assumption on the propagator model - Option 2: Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

As we discussed in RAN4#112bis, the TE-emulated varying Doppler shift and propagation delay models will verify the pre-compensation functionality performed before the UE baseband processing. Like the time/frequency synchronization functionality, we think it is up to UE implementation what method is used to estimate the Doppler shift and propagation delay based on the signaled ephemeris information. Since the existing UE demodulation requirements configure the SSB/TRS scheduling so that UE can use them for the time/frequency synchronization, we think RAN4 only need to specify the scheduling of the satellite ephemeris information. Based on the period of ephemeris information signaling, RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors under the TE-emulated varying Doppler shift and propagation delay models.

Proposal 7: It is up to UE implementation what method is used to estimate the Doppler shift and propagation delay based on the signaled ephemeris information.

Proposal 8: RAN4 NTN UE demodulation requirements specification need to configure the period of ephemeris information update.

Proposal 9: RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models.

# 5	Summary

Observation 1: Although it is assumed that UE is stationary during the tests, it is necessary to consider the earth rotation when we derive the Doppler shift and propagation delay between the satellite and UE.

Observation 2: With LEO600 satellite (maximum positive Doppler) parameters, it takes about 500 seconds to cover more than 10 degrees in elevation angle,

Observation 3: With LEO600 satellite (maximum positive Doppler) parameters, it takes about 250 seconds to cover more than 30 degrees in elevation angle.

Proposal 1:	The TE-emulated varying Doppler shift and propagation delay should be derived with the assumption the DUT (UE) considers the earth rotation when it estimates the Doppler shift and propagation delay.

Proposal 2:	RAN4 should consider the procedure presented in this contribution to specify the varying Doppler shift and propagation delay model used for NTN UE tests.

Proposal 3: RAN4 specification should set one (or more) satellite position/velocity at time zero.

Proposal 4: Set the configured UE location to Longitude: 121.56076999, Latitude: 25.08439333, and Altitude: 0.0 as same as TS 38.508-1.

Proposal 5: Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 as a starting point to specify the TE-emulated Doppler shift and propagation delay model.

Proposal 6: RAN4 discuss further the elevation angle (range) of the satellite position from the UE considering the test time, e.g., more than 30 degrees.

Proposal 7: It is up to UE implementation what method is used to estimate the Doppler shift and propagation delay based on the signaled ephemeris information.

Proposal 8: RAN4 NTN UE demodulation requirements specification need to configure the period of ephemeris information update.

Proposal 9: RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models.

# References

- R4-2416556, “Way Forward for [112bis][321] NTN_testing_NGSO_channel_model”, Samsung.

- R5-237216, “NGSO satellite Ephemeris file generation methodology for NTN NR UE testing”, THALES.

- Roger R. Bate, Donald D. Mueller, Jerry E. White, William W. Saylor, “Fundamentals of Astrodynamics: Second Edition”, Dover Publication, 2020.

# Appendix

## A.1	SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-1: SIB19- NR NTN Ephemeris information for GSO satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 | -22027.8799 (km) |  |
| positionY-r17 | 27362209 | 35570.8717 (km) |  |
| positionZ-r17 | 4019393 | 5225.2109 (km) |  |
| velocityVX-r17 | -336 | -0.02016 (km/s) |  |
| velocityVY-r17 | -208 | -0.01248 (km/s) |  |
| velocityVZ-r17 | 1 | 0.00006 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 | -3315.1053 (km) |  |
| positionY-r17 | 4151097 | 5396.4261 (km) |  |
| positionZ-r17 | 2263034 | 2941.9442 (km) |  |
| velocityVX-r17 | 21970 | 1.3182 (km/s) |  |
| velocityVY-r17 | -48164 | -2.88984 (km/s) |  |
| velocityVZ-r17 | 114109 | 6.84654 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 | -3603.0046 (km) |  |
| positionY-r17 | 4509243 | 5862.0159 (km) |  |
| positionZ-r17 | 2455572 | 3192.2436 (km) |  |
| velocityVX-r17 | 20837 | 1.25022 (km/s) |  |
| velocityVY-r17 | -46176 | -2.77056 (km/s) |  |
| velocityVZ-r17 | 109528 | 6.57168 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

## A.2	SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 | -3532.9021 (km) |  |
| positionY-r17 | 4550419 | 5915.5447 (km) |  |
| positionZ-r17 | 852799 | 1108.6387 (km) |  |
| velocityVX-r17 | 6164 | 0.36984 (km/s) |  |
| velocityVY-r17 | -19424 | -1.16544 (km/s) |  |
| velocityVZ-r17 | 124281 | 7.45686 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 | -2859.0536 (km) |  |
| positionY-r17 | 3404229 | 4425.4977 (km) |  |
| positionZ-r17 | 3535794 | 4596.5322 (km) |  |
| velocityVX-r17 | 35394 | 2.12364 (km/s) |  |
| velocityVY-r17 | -74414 | -4.46484 (km/s) |  |
| velocityVZ-r17 | 94682 | 5.68092 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Table 6.4.1_2.4.3-3: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2592823 | -3370.6699 (km) |  |
| positionY-r17 | 4245650 | 5519.345 (km) |  |
| positionZ-r17 | 2024520 | 2631.876 (km) |  |
| velocityVX-r17 | 19359 | 1.16154 (km/s) |  |
| velocityVY-r17 | -43278 | -2.59668 (km/s) |  |
| velocityVZ-r17 | 116553 | 6.99318 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 60.25 degrees, one-way delay equal to 2.30 ms and Doppler equal to 11.29 ppm. |  |  |  |

## A.3	UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0