3GPP TSG-RAN WG4 Meeting #113	R4-2418991

Orlando, Florida, USA, 18th to 22nd November 2024

Agenda Item:	7.2.4.2

Source:	Ericsson

Title:	On 15 GHz BS-UE radio and antenna parameters

Document for:	Approval

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz.

In the previous 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] [4] and TR 38.922 v0.3.0 [4] captured the agreements on co-existence simulation assumptions for this frequency range.

This contribution provides further discussion points on radio and antenna parameters relevant for 14800 to 15350 MHz frequency range.

# 2	Discussion

## 2.1	BS antenna configuration

In the previous RAN4 meeting, it was agreed to present results with 3072 antenna elements as priority. If any coverage issues are identified, it was proposed to further discuss possibility of 4096 antenna elements.

In our companion contribution [7], we have presented results assuming both 3072 and 4096 antenna elements.

Both the array architectures are feasible from UL coverage perspective. As a result, we propose to consider the array with 3072 antenna elements (AE) for inclusion in the reply to the LS.

However, it is important to note on technology feasibility for output power per branch, ACLR and efficiency for power amplifiers based on reference study in TR 38.820 [9], presented in Clause 2.3.1.2 of this contribution.

RAN4 to consider an array architecture with 3072 AEs for Urban Macro in the reply to the LS. The array configuration is of 16x24 and subarrays of dimension 4x1.

The additional information in BS antenna parameter clause should be updated with a note in the TR – “The choice of BS antenna configuration is an example to derive reasonable requirements. This does not limit or preclude from implementing other possible array architectures.”

Other antenna parameters are included in the following Table 2.1-1, extracted from our other companion contribution [8].

Table 2.1-1: Beamforming antenna characteristics for IMT in 14800 to 15350 MHz

|  |  | Macro suburban | Macro urban | Dense Urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Table 3 [8] |  |  | N/A |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | 5 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | 90º for H 90º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 16x24 | 16x24 | 16x24 | 4x4 |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.7a | Number of element rows in sub-array | 4 | 4 | 4 | N/A |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V | 0.7 of wavelength for V | 0.7 of wavelength for V | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | 3 | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | ## 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 17.1 | 17.1 | 8.1 | 8.9 |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 | ±60 | ±60 | ±90 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 | 0-180 |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | 6 | N/A |
| 1.13 | Base station output power/sector (e.i.r.p.) (dBm) (Note 7) | 81.26 | 81.26 | 71.26 | 32.81 |

RAN4 to consider antenna parameters from Table 2.1-1 for the LS response, relevant to 14800 – 15350 MHz frequency range.

## 2.2	General parameters

#### 2.2.1	Duplex mode

In the previous RAN4#111 and RAN4#112 meetings, it has been agreed on TDD as baseline duplexing for the 14800 – 15350 MHz frequency range for the LS response to WP5D, but also capture additional information related to SBFD in the TR.

Following additional information has been suggested to be captured in the TR:

“There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6]. To provide a timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range.”

To remain consistent with the agreements for other frequency ranges in the TR, it is most unlikely that FDD will be used for this frequency range. However, additional text should be added stating FDD is not precluded.

The additional information in Duplex Mode should be updated with the following text in the TR – “There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range even though FDD is not precluded. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].”

#### 2.2.2	Typical Channel Bandwidth

In our companion contribution [7], we have presented results assuming both 100 MHz and 200 MHz in Uplink. It can be observed that the ACIR requirements are more or less in similar ballpark. At the same time, the maximum BS and UE transmission powers (46 dBm and 23 dBm, respectively) have been defined independently of the channel bandwidth, and the simulation assumption considers that the total power is transmitted over the entire bandwidth. In a real deployment it can be considered however that multiple users can be scheduled over the wide bandwidth and the bandwidth allocated to a UE can be adapted to improve its coverage.

ACIR requirements are in similar ballpark when considering 100 or 200 MHz, both in Uplink and Downlink.

UL coverage could be challenging and multiple UEs can be scheduled to address coverage issues.

It is important to communicate to ITU-R some combinations of power/bandwidth which are feasible in terms of coverage and propose them as typical values in this band. Otherwise, if power and bandwidth remain untied, this leads to unclear Power Spectral Density (PSD), which plays a significant role in coexistence simulations, especially when there are coverage issues.

The additional information in Channel Bandwidth should be updated with the following text in the TR – “Higher channel bandwidths are not precluded. The typical channel bandwidth is associated with the maximum transmission power as it impacts the Power Spectral Density (PSD), when full channel bandwidth is transmitted.”

Following a de-facto approach, to enable wider channel bandwidth with increasing frequency, it is proposed to adopt 200 MHz as typical channel bandwidth with higher channel bandwidths not precluded.

RAN4 to adopt 200 MHz as typical channel bandwidth, and also state in TR that higher channel bandwidths are not precluded, and that in case of coverage issues, more UEs can be considered to be scheduled on the wide bandwidth.

#### 2.2.3	Signal Bandwidth

To maintain consistency with the previous LS responses for 4400 – 4800 MHz and 7125 – 8400 MHz ranges, we propose to refer to TS 38.104 sub-clause 5.3.2 and to mention the dependency of signal bandwidth on the SCS and number of RBs.

Signal bandwidth = NRB  x SCS x 12

RAN4 to not specify a fixed signal bandwidth but rather mention the dependency of signal bandwidth on the SCS and number of RBs.

## 2.3	BS parameters

### 2.3.1	Transmitter characteristics

#### 2.3.1.1	Spectral mask

In the previous IMT SI for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz band, it was agreed to apply the same basic limits as for bands above 1 GHz (Clause 6.6.4.2 and 9.7.4.2 of TS 38.104) but the frequency offset step size was updated to 50 MHz.

But in the last RAN4 meeting, some concerns were raised whether Category B limits should be included in the TR and LS response. It should be noted that the limits are not yet defined by CEPT.

AAS BS OBUE limits for Category B are not yet defined by CEPT.

At this point, it is reasonable to not define any limits for this frequency range but update the TR later once ECC SE21 has concluded the limits.

RAN4 to not state AAS OBUE Category B limits for this frequency range. However, it is important to point out in LS response and TR, that Category B limits are not yet defined by CEPT.

RAN4 to follow previous agreements for Table 1B AAS OBUE Category A limits for 7125 to 8400 MHz in LS [10].

#### 2.3.1.2	ACLR

In TR 38.921 [6], the agreed values for ACLR/ ACS for 10 GHz has been specified in Table 2.3-1 below. The values were based on ACIR requirements of 30 dB in DL and 23.9 dB in UL, as per the co-existence simulation results.

Table 2.3-1: ACLR and ACS values for 10-10.5 GHz [6]

| (dB) | ACLR | ACS |
| --- | --- | --- |
| BS | 37 | 40 |
| UE | 24 | 31 |

Based on the simulation results presented in our companion contribution [7], the ACIR requirements in DL and UL are 24 dB and 14 dB respectively, when considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users.

Based on co-existence simulation results of 24 dB ACIR in DL and assuming 25 dB UE ACS into the formula below:

This would correspond to BS ACLR of 31 dB.

Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, BS ACLR corresponds to 31 dB.

RAN4 in Release-16 studied the RF technology feasibility for the 7-24 GHz range documented in the report 3GPP TR 38.820 [5], provides in-depth analysis of power amplifiers based on different semi-conductor technologies. The study is essential to consider as it depicts the relation between ACLR and output power (dBm), for the relevant 14800 to 15350 MHz frequency range.

In the clause 5.4.3 stated in TR 38.820 [9], the study describes the current technological capability for different semi-conductor technologies to achieve a certain output power level at a prescribed ACLR level for different power amplifiers (PA). Figure 1 taken from clause 5.4.3 [9], shows the results for relation between output power and ACLR for 15 GHz.

Figure 1: ACLR versus output power at 10, 15, and 20 GHz example frequencies, and 30 GHz based on 30 GHz PA model for reference [9]

Based on our co-existence simulation results and assuming 31 dB ACLR is required for 15 GHz, as per the latest findings from Figure 1 this accounts for:

- 22 dBm output power per PA branch for CMOS PA at 15 GHz

- 33 dBm output power per PA branch for GaN PA at 15 GHz

At 31 dB ACLR, the maximum output power per PA branch stated in TR 38.820 is 22 dBm for CMOS PA and 33 dBm GaN PA.

Calculation maximum out power per PA branch in dBm for 3072 AEs: 16x24 array and 4x1 sub-array,

Keeping that in mind, if for example we assume to aim for EIRP of 81.2 dBm per 200 MHz bandwidth and a conservative 2 dB transmission loss in RDN (probably higher transmission losses should be considered due to bigger sub-arrays assumed).

Antenna gain G (dBi) = 6.4 (Element gain in dBi) + 10*log10(16*24) + 10*log10(4*1) = 38.2gThis will lead to, TRP (dBm) = EIRP – 38.2 = 81.2 – 38.2 = 43 dBm per polarization

Maximum output power per PA branch in dBm = TRP (both polarizations) – 10*log10(16*24*2) – 2 (transmission loss) = 46 – 28.85 – 2 = 17.15 dBm.

Based on the calculations for 3072 AEs, the output power per PA branch is closer to CMOS semiconductor technology at 31 dB ACLR for 15 GHz.

#### 2.3.1.3	Spurious emissions

In the last RAN4 meeting, some concerns were raised whether Category B limits should be included in the TR and LS response. It should be noted that there is a work item which has just started in ECC WG SE21 and the limits are not yet defined by CEPT.

AAS BS spurious emission limits for Category B are not yet defined by CEPT.

If ITU-R considers the worst case, Category A limits are always higher than Category B.

RAN4 to not state AAS spurious emission Category B limits for this frequency range. However, it is important to point out in LS response and TR, that Category B limits are not yet defined by CEPT.

RAN4 to follow previous agreements for AAS spurious emission Category A limits with the upper frequency limit defined as 2nd harmonic.

#### 2.3.1.4	Maximum output power

The maximum base station output power/ sector (e.i.r.p.) should be aligned with the antenna characteristics and are provided in Table 2.3.1.4-1 below.

Table 2.3.1.4-1: Maximum BS output power

|  | Indoor | Macro suburban | Macro urban | Dense Urban |
| --- | --- | --- | --- | --- |
| Maximum base station output power/sector (e.i.r.p.) (dBm) | 32.81 | 81.26 | 81.26 | 71.26 |

The Total Radiated Power (TRP) for two polarizations are shown in Table 2.3.1.4-2 below.

Table 2.3.1.4-2: Total Radiated Power

| Parameter | Indoor | Macro Sub-urban | Macro Urban | Dense Urban |
| --- | --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 23 | 46 | 46 | 37 |

RAN4 to adopt 37 dBm as BS max output power for Dense Urban.

### 2.3.2	Receiver characteristics

#### 2.3.2.1	Blocking response

In the previous RAN4 meeting, it was agreed to provide simulations to evaluate the blocking power level. Figure 2.3.2.1-1 shows the wideband power received at the BS receiver from the adjacent channel UL and without including the beamforming gain (but including the subarray gain). At the 99.99%tile, we observe the potential blocking requirement, for a BS array with 3072 AE and 4x1 subarrays, and 200 MHz channel bandwidth, would be in the range of -62 dBm.

Figure 2.3.2.1-1 Received blocking ACI

This is an indicative value, and more analysis needs to be performed to reach an agreement. The requirements can be derived when 3GPP defines the band.

Based on our pre-liminary analysis, we observe blocking requirement of -62 dBm. This is an indicative value, and requirements can be derived when 3GPP defines the band.

#### 2.3.2.2	ACS

The agreed values for ACLR/ ACS for 10 GHz has been specified in Table 2.3-1.

Based on the simulation results presented in our companion contribution [7], the ACIR requirements in DL and UL are 24 dB and 14 dB respectively, when considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users.

Based on co-existence simulation results of 14 dB ACIR in UL and assuming 15 dB UE ACLR into the formula below:

This would correspond to BS ACS of 21 dB.

Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, BS ACS corresponds to 21 dB.

## 2.4	UE parameters

### 2.4.1	Transmitter characteristics

#### 2.4.1.1	Power dynamic range

RAN4 to agree on 56 dB as power dynamic range.

#### 2.4.1.2	ACLR

This would correspond to UE ACLR of 15 dB.

Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, UE ACLR corresponds to 15 dB.

#### 2.4.1.3	Spurious emissions

From TS 38.101-1 [11], the general spurious can be defined as per clause 6.5.3.1 for this frequency range -

-	30 MHz ≤ f ≤ 1 GHz: - 36 dBm / 100 kHz

-	1 GHz ≤ f ≤ 26 GHz: - 30 dBm / 1 MHz

RAN4 to consider UE spurious emission limits from TS 38.101-1, - 36 dBm/ 100 MHz and - 30 dBm / 1 MHz as baseline for discussions.

### 2.4.2	Receiver characteristics

#### 2.4.2.1	Noise figure

RAN4 to agree on the UE NF as 13 dB, for both co-existence assumptions and LS response.

#### 2.4.2.2	ACS

This would correspond to UE ACLR of 25 dB.

Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, UE ACS corresponds to 25 dB.

#### 2.4.2.3	Blocking response

The blocking characteristic specified in clause 7.6 of TS 38.101-1 [11] for frequency larger than 3300 MHz could be applied for this frequency range.

RAN4 to consider Clause 7.6 of TS 38.101-1 with with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, applicable to this range.

# Conclusion

In the previous sections we made the following observations:

Observation 1	ACIR requirements are in similar ballpark when considering 100 or 200 MHz, both in Uplink and Downlink.

Observation 2	UL coverage could be challenging and multiple UEs can be scheduled to address coverage issues.

Observation 3	AAS BS OBUE limits for Category B are not yet defined by CEPT.

Observation 4	At 31 dB ACLR, the maximum output power per PA branch stated in TR 38.820 is 22 dBm for CMOS PA and 33 dBm GaN PA.

Observation 5	Based on the calculations for 3072 AEs, the output power per PA branch is closer to CMOS semiconductor technology at 31 dB ACLR for 15 GHz.

Observation 6	AAS BS spurious emission limits for Category B are not yet defined by CEPT.

Observation 7	Based on our pre-liminary analysis, we observe blocking requirement of -62 dBm. This is an indicative value, and requirements can be derived when 3GPP defines the band.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to consider an array architecture with 3072 AEs for Urban Macro in the reply to the LS. The array configuration is of 16x24 and subarrays of dimension 4x1.

Proposal 2	The additional information in BS antenna parameter clause should be updated with a note in the TR – “The choice of BS antenna configuration is an example to derive reasonable requirements. This does not limit or preclude from implementing other possible array architectures.”

Proposal 3	RAN4 to consider antenna parameters from Table 2.1-1 for the LS response, relevant to 14800 – 15350 MHz frequency range.

Proposal 4	The additional information in Duplex Mode should be updated with the following text in the TR – “There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range even though FDD is not precluded. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].”

Proposal 5	The additional information in Channel Bandwidth should be updated with the following text in the TR – “Higher channel bandwidths are not precluded. The typical channel bandwidth is associated with the maximum transmission power as it impacts the Power Spectral Density (PSD), when full channel bandwidth is transmitted.”

Proposal 6	RAN4 to adopt 200 MHz as typical channel bandwidth, and also state in TR that higher channel bandwidths are not precluded, and that in case of coverage issues, more UEs can be considered to be scheduled on the wide bandwidth.

Proposal 7	RAN4 to not specify a fixed signal bandwidth but rather mention the dependency of signal bandwidth on the SCS and number of RBs.

Proposal 8	RAN4 to not state AAS OBUE Category B limits for this frequency range. However, it is important to point out in LS response and TR, that Category B limits are not yet defined by CEPT.

RAN4 to follow previous agreements for Table 1B AAS OBUE Category A limits for 7125 to 8400 MHz in LS [10].

Proposal 9

Proposal 10	Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, BS ACLR corresponds to 31 dB.

Proposal 11	RAN4 to not state AAS spurious emission Category B limits for this frequency range. However, it is important to point out in LS response and TR, that Category B limits are not yet defined by CEPT.

Proposal 12	RAN4 to follow previous agreements for AAS spurious emission Category A limits with the upper frequency limit defined as 2nd harmonic.

Proposal 13	RAN4 to adopt 37 dBm as BS max output power for Dense Urban.

Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, BS ACS corresponds to 21 dB.

Proposal 14

Proposal 15	RAN4 to agree on 56 dB as power dynamic range.

Proposal 16	Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, UE ACLR corresponds to 15 dB.

Proposal 17	RAN4 to consider UE spurious emission limits from TS 38.101-1, - 36 dBm/ 100 MHz and - 30 dBm / 1 MHz as baseline for discussions.

Proposal 18	RAN4 to agree on the UE NF as 13 dB, for both co-existence assumptions and LS response.

Proposal 19	Based on co-existence results considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users, UE ACS corresponds to 25 dB.

RAN4 to consider Clause 7.6 of TS 38.101-1 with with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, applicable to this range.

Proposal 20

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2417107, “WF on 15 GHz BS parameters”, 3GPP RAN4#112bis, ZTE Corporation

- R4-2417108, “WF on 15 GHz UE parameters”, 3GPP RAN4#112bis, Qualcomm

- R4-2417200, “3GPP TR 38.922 v0.3.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#112bis

- 3GPP TR 38.921 v18.0.1: Study on IMT parameters for 6425 – 7025, 7025 – 7125 and 10000 - 10500 MHz”, 3GPP SA#103

- R4-2418993, “On 15 GHz co-existence simulation results”, 3GPP RAN4#113, Ericsson

- R4-2418997, “Draft LS Reply to ITU-R on IMT parameters for 14800 - 15350 MHz”, 3GPP RAN4#113, Ericsson

- 3GPP TR 38.820 v16.1.0 (2021-03), “Study on 7-24 GHz frequency range (Release 16)”.

- R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, 3GPP RAN4#112, Ericsson

- 3GPP TS 38.101-1 v.18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”